3GPP TSG-RAN WG4 Meeting #114bis					R4-2504073

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On NR MIMO Phase 5 UEIBM requirements

Agenda item:	7.22.3.1

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the UE-initiated beam management feature.

Discussion on reporting delay requirements

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)?

Agreement:

The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

- update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

  - TReport = 0 or be removed from the measurement period

  - N: Reuse the legacy definition

  - M: FFS pending RAN1 further agreement (timeRestrictionForChannelMeasurement, M=1 or M=3 in legacy)

  - TSSB (taking SSB as example)

    - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new

    - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1

      - Option A: Max{TSSB_current, TSSB_new}

      - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

      - Option C: others are not precluded

In the last RAN4 meeting, there was a general agreement on how to define the measurement period with some open points.

In RAN4#113, it was agreed to take the SSB and CSI-RS based L1 measurement delay in 9.5.4.1, 9.5.4.2 and 9.13.4 as the baseline for TL1-RSRP_event triggered_measurement_Period_SSB and TL1-RSRP_event triggered_measurement_Period_CSI-RS, and update the individual components of the equation where necessary.

The value for M was left as FFS pending RAN1 discussion. However, the M value that is being discussed in RAN1 is the number of Event instances that should occur within a time window for the UE to determine that an UEI Event-2 has occurred. This M has no relation to the M that depends on timeRestrictionForChannelMeasurement, which used in the computation of RAN4 L1 RSRP measurement period, and is not being discussed in RAN1. Therefore,  it is not expected that any ongoing RAN1 discussion is going to impact this parameter.

M value that is being discussed in RAN1 is the number of Event instances that should occur within a time window for the UE to determine that an UEI Event-2 has occurred.

M that RAN1 is discussing has no relation to timeRestrictionForChannelMeasurement (M) used in the computation of RAN4 L1 RSRP measurement period, which is not being discussed in RAN1.

It can of course be questioned whether supporting M=1 through timeRestrictionForChannelMeasurement makes sense for event-triggered reporting. When the reporting is based on a single shot, triggering a report based on one sample may lead to too frequent reporting. However, whether this parameter is configured or not is up to network, and hence at the existence of this parameter, UE should follow it. If RAN1/RAN2 would agree not to support timeRestrictionForChannelMeasurement configuration when the UE is configured with UEIBM, then M would always get value 3. However, we are not aware of such discussion or agreement.

Issue 1-3: Same or different periodicities of current beam and new beam(s)?

<Way forward >

- Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo)

  - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities.

  - According to RAN1 LS (R1-2410914)

- Option 2: (Huawei)

  - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1

- Option 3: (vivo)

  - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

If timeRestrictionForChannelMeasurement configuration is supported when UE is configured with UEIBM event-triggered reporting, RAN4 to reuse M=1 or M=3 for computation of L1 RSRP event triggered measurement period depending on whether timeRestrictionForChannelMeasurement is configured, similar to legacy.

RAN1 has only agreed on having same periodicities of current beam and new beam while different periodicities of current beam and new beam is still under discussion. Hence, we propose to only define requirements considering same periodicity of current beam and new beam and defer defining requirements considering different periodicity of current beam and new beam till RAN1 agreements on the same.

Issue 1-4: Components of Event triggered measurement reporting delay?

<Way forward >

- Option 1: (Apple, vivo, Samsung, E///, MTK)

  - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel

    - FFS on whether other components are needed.

    - FFS the definition of Tfirst UL channel

- Option 2: (CMCC, Xiaomi)

  - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering.

- Option 3: (Nokia)

  - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

Only define requirements considering same periodicity of current beam and new beam unless there are further RAN1 agreements on the support of different periodicities for current beam and new beam.

RAN4 has agreed that the reporting delay shall in totality be the time from when an event that triggers the measurement report occurs to the time instance when the UE sends the first PUCCH at the first available PUCCH occasion after the event that triggered the measurement report has occurred. The UE may hence send the PUCCH at any occasion from the moment the reporting condition becomes fulfilled, depending on when the measurement occasion based on which the UE determines the condition has become fulfilled occurs in relation to the moment the condition becomes fulfilled. In the context of Option 1 this means that the Tfirst UL channel  is the first PUCCH after the event evaluation delay component, which may be defined based on the existing L1 measurement delay. As we commented in the last meeting, whether the component Tfirst UL channel is introduced or not is not as important and making it clear that the latest PUCCH occasion the UE shall use is clearly defined. Following the L3 event triggered reporting principle, we have a slight preference to not include this uncertainty parameter and instead just directly define the delay as proposed below:

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M?

<Way forward >

- Option 1: (Qualcomm, Apple, Ericsson)

  - Only define requirements for the number of events is met at once

- Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable M

- Option 2a: (ZTE)

  - Event1&Event 2: Case 2: multiple samples M and timer X should be considered

  - Event1&Event 7: FFS

- Option 2b (Samsung)

  - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

- Option 2c (MediaTek)

  - For multiple events is met by counter:

    - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

    - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS

    - C: event instance counter is configured by NW.

The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period).

RAN1 has defined two types of triggering event determination for Event-2:

- Basic: UE initiates a beam report when L1-RSRP of the new beam becomes a threshold value better than the current beam.

- Optional (subject to UE capability): UE initiates a beam report if within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M.

With respect to the optional UE capability, discussions are ongoing in RAN1 on the kind of approach to be followed with respect to the time window. i.e. should a fixed window approach be followed or a sliding window one. The latest agreement in RAN1 is given below:

Agreement (RAN1 #120 Athens)

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

- The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time tt, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

- Note: Other options are not precluded.

Considering that the design of the window-based reporting is not finalized, we propose that RAN4 should initially define requirements for the basic case where the UE initiates a beam report when L1-RSRP of the new beam becomes a threshold value better than the current beam. RAN4 shall revisit the need for defining requirements for the time window based UEI based on RAN1 agreement on the kind of window to be used.

RAN1 is discussing the approach (fixed window or sliding window) to be followed for the window-based UE event triggered report.

RAN4 to defer discussion on defining requirements for window based approach till RAN1 concludes the details of this approach.

To describe the proposals in this section better, we have prepared a text proposal in our draft CR in R4-2504074 to show how we think the reporting requirements can be captured.

Discussion on TCI switching requirements

Issue 1-11: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Xiaomi, Nokia)

  - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Option 3: (LGE)

  - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

- Option 4: (ZTE)

  - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc

- Option 5: (Samsung)

  - Not consider unknown case.

In the last RAN4 meeting, the following was left as open issue for discussion [1]:

In RAN1#119 meeting various alternatives about beam application latency reduction after a UEI beam report is sent were discussed and agreed to be studied. No conclusion was reached about whether or which alternative to use.

Agreement (RAN1 #119 Orlando)

Study the following to reduce beam application latency after a UEI/ED beam report is sent

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

- after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

- update TCI state(s) with the reported new beam(s)

- activate new beam(s) without additional SSB reception

- Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

- Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

- FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

- Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

- In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

- Other alternatives are not precluded

If some of these alternatives are agreed in RAN1, these could have a potential impact on the RAN4 TCI state switching delay requirements. Hence, we propose to wait for RAN1 progress on this issue before defining the TCI state switching delay requirements in RAN4.

RAN1 is discussing various alternatives to reduce the beam application latency, some of which may have an impact on the TCI state switch delay.

RAN4 to wait for RAN1 progress on discussion of beam application latency reduction after a UEI beam report is sent before defining the related TCI state switching delay requirements.

Discussion on other measurement requirements

Issue 1-7: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time?

<Way forward >

- Option 1: (Xiaomi)

  - For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Option 2: (LGE, Nokia)

  - Need to wait RAN1 progress

- Option 3: (vivo)

  - Follow the assumption: the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting. FFS on other cases if time allows

In the last RAN4 meeting, the following was left as open issue for discussion regarding L1 report [1]:

The issue about UE behaviour while waiting to send the UEI report after sending the 1st PUCCH when the conditions that originally caused the UEI event changes, could be a likely scenario in the network. However, we believe that this needs to be discussed and decided in RAN1 instead of RAN4.

UE behaviour in the interim between sending the 1st PUCCH and waiting for UL resources to send the UEI report is up to RAN1.

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM

There are currently no baseline RAN4 requirements defined for inter-cell CSI-RS based L1-RSRP measurements for beam management. Hence, unless a clear use case is identified we propose not to introduce CSI-RS based L1-RSRP requirements for inter-cell beam management.

No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case.

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1: (Apple, Ericsson)

  - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 2: (China Telecom)

  - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI.

- Option 3: (CMCC, MediaTek)

  - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

Issue 1-15: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Apple, Ericsson)

  - Discuss it later.

- Option 2: (CMCC, Huawei, Nokia, Samsung)

  - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately.

- Option 4: (MediaTek)

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

RAN1 is currently discussing the UE capability for the number of events to be supported for UEIBM. RAN4 should wait for RAN1 agreements before deciding whether and how to define related requirements. Considering coordination between MIMO UEIBM and LTM mobility L1 events, we think it is better to keep the discussion and requirements independent, as these are two separate parallel work items.

RAN1 is discussing and defining UE capability for UEIBM. If any RAN4 specific UE capability is required, then this can be discussed once discussions in RAN1 are completed.

Do not combine capabilities across different L1 measurements.

Conclusion

In this paper, the following observations and proposals were made:

Observation 1: M value that is being discussed in RAN1 is the number of Event instances that should occur within a time window for the UE to determine that an UEI Event-2 has occurred.

Observation 2: M that RAN1 is discussing has no relation to timeRestrictionForChannelMeasurement (M) used in the computation of RAN4 L1 RSRP measurement period, which is not being discussed in RAN1.

Proposal 1: If timeRestrictionForChannelMeasurement configuration is supported when UE is configured with UEIBM event-triggered reporting, RAN4 to reuse M=1 or M=3 for computation of L1 RSRP event triggered measurement period depending on whether timeRestrictionForChannelMeasurement is configured, similar to legacy.

Proposal 2: Only define requirements considering same periodicity of current beam and new beam unless there are further RAN1 agreements on the support of different periodicities for current beam and new beam.

Proposal 3: The UE shall be able to send the PUCCH latest at the first PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. TL1-RSRP_event triggered_measurement_Period).

Observation 3: RAN1 is discussing the approach (fixed window or sliding window) to be followed for the window-based UE event triggered report.

Proposal 4: RAN4 to defer discussion on defining requirements for window based approach till RAN1 concludes the details of this approach.

Observation 4: RAN1 is discussing various alternatives to reduce the beam application latency, some of which may have an impact on the TCI state switch delay.

Proposal 5: RAN4 to wait for RAN1 progress on discussion of beam application latency reduction after a UEI beam report is sent before defining the related TCI state switching delay requirements.

Proposal 6: UE behaviour in the interim between sending the 1st PUCCH and waiting for UL resources to send the UEI report is up to RAN1.

Observation 5: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

Proposal 7: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case.

Observation 6: RAN1 is discussing and defining UE capability for UEIBM. If any RAN4 specific UE capability is required, then this can be discussed once discussions in RAN1 are completed.

Proposal 8: Do not combine capabilities across different L1 measurements.

- References

- R4-2502595    WF on RRM requirements for NR_MIMO_Ph5_Part1

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management