3GPP TSG-RAN WG4 Meeting #114bis	R4-2504074

Wuhan, China, April 7th – April 11th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for UEIBM reporting requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The draftCR captures reporting requirements for UE initiated beam management related to the Rel-19 MIMO Ph5 WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce L1 UEIBM reporting requirements for: - Intra-cell measurement reporting in section 9.5.3 - Inter-cell measurements in section 9.13.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEIBM requirements are not included in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses 9.5.3.x and 9.13.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

#### 9.5.3.x	Event Triggered Reporting

Reported L1-RSRP measurements contained in event-triggered L1 measurement reports shall meet the measurement accuracy requirements in clauses 10.1.19 (FR1) and 10.1.20 (FR2).

The UE shall not send any event-triggered L1-RSRP reports as long as no reporting condition is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger the measurement report and the time UE starts to transmit the first PUCCH to request for reporting resources (Mode-A), or to inform the network about uplink reporting in the next reporting occasion (Mode-B), over the air interface. The delay between the first PUCCH and the transmission of the actual event-triggered L1-RSRP report is not taken into account in the reporting delay.

In both Mode-A and Mode-B, the UE shall send the PUCCH latest at the first PUCCH occasion that occurs after TL1-event_evaluation_period from the time the reporting event occurs, where TL1-event_evaluation_period is the measurement evaluation delay defined as:

- When the report is triggered by SSB-based intra-cell measurement:

  - TL1-event_evaluation_period = TL1-RSRP_Measurement_Period_SSB defined in Table 9.5.4.1-1 for FR1 and Table 9.5.4.1-2 for FR2, where

    - TReport = 0

    - TSSB is the periodicity of the SSB that triggers the report.

- When the report is triggered by CSI-RS based intra-cell measurement:

  - TL1-event_evaluation_period = TL1-RSRP_Measurement_Period_CSI-RS defined in Table 9.5.4.2-1 for FR1 and Table 9.5.4.2-2 for FR2, where

    - TReport = 0

    - TCSI-RS is the periodicity of the CSI-RS that triggers the report.

Change 2

#### 9.13.3.x	Event Triggered Reporting

Reported L1-RSRP measurements contained in event-triggered L1 measurement reports shall meet the measurement accuracy requirements in clauses 10.1.19 (FR1) and 10.1.20 (FR2).

The UE shall not send any event-triggered L1-RSRP reports as long as no reporting condition is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger the measurement report and the time UE starts to transmit the first PUCCH to request for reporting resources (Mode-A), or to inform the network about uplink reporting in the next reporting occasion (Mode-B), over the air interface. The delay between the first PUCCH and the transmission of the actual event-triggered L1-RSRP report is not taken into account in the reporting delay.

In both Mode-A and Mode-B, the UE shall send the PUCCH latest at the first PUCCH occasion that occurs after TL1-event_evaluation_period from the time the reporting event occurs, where TL1-event_evaluation_period is the measurement evaluation delay defined as:

- When the report is triggered by SSB-based serving cell measurement:

  - TL1-event_evaluation_period = TL1-RSRP_Measurement_Period_SSB defined in Table 9.5.4.1-1 for FR1 and Table 9.5.4.1-2 for FR2, where

    - TReport = 0

    - TSSB is the periodicity of the SSB that triggers the report.

- When the report is triggered by SSB based inter-cell measurement:

  - TL1-event_evaluation_period = TL1-RSRP_Measurement_Period_SSB_CDP defined in Table 9.13.4.1-1 for FR1 and Table 9.13.4.1-2 for FR2, where

    - TReport = 0

    - TSSB_CDP is the periodicity of the SSB that triggers the report.

End of changes