3GPP TSG-WG RAN4 Meeting #114bis	R4- 2504093

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Astrum Mobile

Title: 	Discussion on SAN RF requirements

Agenda item:	7.9.4

Document for:	Approval

1. Introduction

In RAN#107, the new WID for new LTE band for 5G Broadcast for region 3 Utilizing Geosynchronous satellite was approved in [1] with the following objectives:

- Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

In this contribution, we provide our views on SAN RF requirements.

2. BS RF requirements

For SAN RF requirements, as stated in [1], existing SAN RF requirements specified in TS 38.108 for 10MHz should be considered as the baseline. As only 1.4MHz SAN channel bandwidth is defined in TS 36.108, the SAN RF requirements for LTE-based 5G Broadcast with 10MHz should be added in TS 36.108 according to SAN RF requirements specified in TS 38.108 for 10MHz channel bandwidth.

Proposal 1: It is proposed to specify the total power dynamic range for LTE based 5G broadcast over satellite for 10MHz channel bandwidth as following. FFS on other SCS options including 2.5kHz, 1.25kHz, 0.37kHz.

Table 6.3.3.2-1: Total power dynamic range

| SAN channel | Total power dynamic range (dB) |
| --- | --- |
| bandwidth (MHz) | 15 kHz SCS |
| 1.4 | 7.7 |
| 10 | 17.0 |
| NOTE: 10MHz channel bandwidth only applies for LTE based 5G broadcast. |  |

For EVM requirements, as agreed in [1], only consider QPSK and 16QAM modulation for LTE based 5G broadcast over satellite scenario. Therefore, it is proposed to add one note in the EVM requirements of TS 36.108 to clarify only QPSK and 16QAM apply for LTE based 5G broadcast.

Proposal 2: It is proposed to specify the EVM requirements for LTE based 5G broadcast over satellite for as following.

Table 6.5.2-1: EVM requirements for E-UTRA carrier

| Modulation scheme for PDSCH or sPDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM (NOTE) | 8 % |
| NOTE 1: EVM requirement for 64QAM is optional. NOTE 2: Only QPSK and 16QAM apply for LTE based 5G broadcast. |  |

For ACLR, the 10MHz channel bandwidth for GEO class should be added for LTE based 5G broadcast.

Proposal 3: It is proposed to specify the SAN ACLR limit for 10MHz channel bandwidth for LTE based 5G broadcast as following.

Table 6.6.3.2-1: SAN ACLR limit for GEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 1.4,10 | BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 14 |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 14 |
| NOTE 1: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: 10MHz channel bandwidth only applies for LTE based 5G broadcast. And ACLR requirements for LTE based 5G broadcast shall also comply with regional requirements. |  |  |  |  |

3. Conclusion

In this paper, the following proposals are made:

Proposal 1: It is proposed to specify the total power dynamic range for LTE based 5G broadcast over satellite for 10MHz channel bandwidth as following. FFS on other SCS options including 2.5kHz, 1.25kHz, 0.37kHz.

Table 6.3.3.2-1: Total power dynamic range

| SAN channel | Total power dynamic range (dB) |
| --- | --- |
| bandwidth (MHz) | 15 kHz SCS |
| 1.4 | 7.7 |
| 10 | 17.0 |
| NOTE: 10MHz channel bandwidth only applies for LTE based 5G broadcast. |  |

Proposal 2: It is proposed to specify the EVM requirements for LTE based 5G broadcast over satellite for as following.

Table 6.5.2-1: EVM requirements for E-UTRA carrier

| Modulation scheme for PDSCH or sPDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM (NOTE) | 8 % |
| NOTE 1: EVM requirement for 64QAM is optional. NOTE 2: Only QPSK and 16QAM apply for LTE based 5G broadcast. |  |

Proposal 3: It is proposed to specify the SAN ACLR limit for 10MHz channel bandwidth for LTE based 5G broadcast as following.

Table 6.6.3.2-1: SAN ACLR limit for GEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 1.4,10 | BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 14 |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) (NOTE) | 14 |
| NOTE 1: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: 10MHz channel bandwidth only applies for LTE based 5G broadcast. And ACLR requirements for LTE based 5G broadcast shall also comply with regional requirements. |  |  |  |  |

4. References

- RP-250788, “New WID: New band for 5G Broadcast Utilizing Geosynchronous Satellite”, Astrum Mobile,    Qualcomm, RAN#107, March, 2025