3GPP TSG-RAN WG4#114-bis	R4-2504116

Wuhan, China, April 7th – 11th, 2025

Agenda item:	6.3.3

Source: 	Qualcomm France

Title: 	CA_n5A-n8A capabilities

Document for:	Approval

# Introduction

UE capabilities and other issues for CA_n5A-n8A are discussed in this contribution.

- Discussion

RAN2 sent a reply LS [1] to RAN4 LS.

CA_n5A-n8A is indeed a unique combination, with frequency overlap between n8 UL and n5 DL.

As pointed out by RAN2, the NW can take care of con-concurrent scheduling of n8 UL and n5 DL. RAN4 has agreed that the case with partial RF filters are not considered in Rel-19, and if they are considered later than Rel-19 then new bands are considered. Hence, it is not possible to support simultaneous Rx/Tx (in particular, concurrent n8 Tx and n5 Rx) without any scheduling restriction for CA_n5A-n8A with UL CA_n5A-n8A.

Proposal 1: Answer to RAN2 Question 1 as follows.

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

- Support of simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction would require partial band RF filter(s) and explicit frequency range definitions in RAN4 specification. RAN4 has agreed that this approach is not pursued in Rel-19, and if pursued in later releases then new bands are considered. Hence it is not possible to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, and it’s not possible to support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands.

In DL CA_n5A-n8A with UL CA_n5A-n8A, the UL/DL transmissions in time domain include both“ n5DL-n8DL-n5UL”, and “n8 DL-n5UL-n8UL”, and their lower order combinations. UE must do switching between transmissions including n8 UL and transmissions including n5 DL as same static RFFE configuration cannot be used for both due to frequency overlap. One example RFFE is illustrated below. There are obviously different RFFE implementation alternatives, but common theme to them is that one static RFFE configuration cannot be used for all transmissions. While the frequency overlap has been discussed a lot during the past years, the need to do dynamic switching when DL CA_n5A-n8A with UL CA_n5A-n8A is configured was not discussed. Despite that the need to dynamically configure RFFE depending on the actual transmissions exists.

Figure 1Example RFFE configuration with dynamic switching

Based on this, we propose to define an interruption time or equivalent to address switching. Because NW can take care of con-concurrent scheduling of n8 UL and n5DL, it could also take care of arranging {35,140us} gap between these two which would serve as interruption time. This interruption time could be explicitly written in CA configuration table note for UL CA_n5A-n8A along with NW scheduling restriction, e.g. as follows. Please note that to avoid ambiguity, we propose to have this gap right before the n5DL/n8UL for which the switching must be done. Alternatively, a UE capability with different interruption times can be discussed. Whether this interruption can be handled purely in RAN4 specifications or if RAN1 impact (switching periods) is needed to be discussed.

| CA_n5A-n8A15 | CA_n5A-8Axx | n5 | 5, 10 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n8 | 5, 10 |  |

NOTE 15: Uplink is only n5 for CA_n5-n8 when UL CA is not supported.

NOTE XX: When UL CA n5A-8A is supported, the network does not schedule n5 DL and n8 UL concurrently. Network ensures {35,140}us gap right before the start of the first n5DL when switching from n8UL, during which the network does not schedule any of n5 DL/UL or n8 DL/UL. Network ensures {35,140}us gap right before the start of the first n8UL when switching from n5DL, during which the network does not schedule any of n5 DL/UL or n8 DL/UL.

Figure 2 Illustration of [35,140us] scheduling gap

Proposal 2: Specify interruption time for DL CA_n5A-n8A with UL CA_n5A-n8A between n8 UL and n5 DL transmissions. This applies both between n8 UL and n5 DL, and between n5DL and n8 UL

Proposal 3: Interruption time for DL CA_n5A-n8A with UL CA_n5A-n8A between n8 UL and n5 DL is {35,140us} and between n5 DL and n8 UL is {35,140us}.

Proposal 4: RAN4 to conclude if interruption time can be embedded in NW scheduling restriction note in 38.101, or if it is specified as a UE capability. RAN4 shall include this information in LS reply to RAN2.

Proposal 5: Answer to RAN2 Question 2 as follows. Note that conclusion in Proposal 4 must be added before sending.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

- CA_n5-n8 cannot support simultaneous Rx/Tx because of the frequency overlap (in particular, concurrent n8 Tx and n5 Rx). As explained in Answer 1, frequency gap/guard band is not considered as a solution to enable DL_CA_n5A-n8A with UL CA_n5A-n8A. Hence additional UE capabilities to enable simultaneous Rx/Tx in this case would not help to enable simultaneous Rx/Tx.

- Conclusion

Considerations on CA_n5A-n8A with UL CA_n5A-n8A capabilities were considered with the following proposals.

Proposal 1: Answer to RAN2 Question 1 as follows.

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

- Support of simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction would require partial band RF filter(s) and explicit frequency range definitions in RAN4 specification. RAN4 has agreed that this approach is not pursued in Rel-19, and if pursued in later releases then new bands are considered. Hence it is not possible to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, and it’s not possible to support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands.

Proposal 2: Specify interruption time for DL CA_n5A-n8A with UL CA_n5A-n8A between n8 UL and n5 DL transmissions. This applies both between n8 UL and n5 DL, and between n5DL and n8 UL

Proposal 3: Interruption time for DL CA_n5A-n8A with UL CA_n5A-n8A between n8 UL and n5 DL is {35,140us} and between n5 DL and n8 UL is {35,140us}.

Proposal 4: RAN4 to conclude if interruption time can be embedded in NW scheduling restriction note in 38.101, or if it is specified as a UE capability. RAN4 shall include this information in LS reply to RAN2.

Proposal 5: Answer to RAN2 Question 2 as follows. Note that conclusion in Proposal 4 must be added before sending.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

- CA_n5-n8 cannot support simultaneous Rx/Tx because of the frequency overlap (in particular, concurrent n8 Tx and n5 Rx). As explained in Answer 1, frequency gap/guard band is not considered as a solution to enable DL_CA_n5A-n8A with UL CA_n5A-n8A. Hence additional UE capabilities to enable simultaneous Rx/Tx in this case would not help to enable simultaneous Rx/Tx.

- Reference

[1] R2-2501572, “Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx”, CATT