3GPP TSG-RAN WG4#114-bis	R4-2504118

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.1.1.1.1

Source: 	Qualcomm France

Title: 	MSD beyond PC3

Document for:	Approval

# Introduction

Considerations and proposals for HPUE MSD are provided in this contribution.

- Background

There was quite a bit of discussion on HPUE MSD in previous meeting. Draft WF was prepared, but at the end it was obvious that companies need more time to digest the topic even there were a few concrete proposals submitted for RAN4#114 [1][2][3]. This is completely understandable as the set of decisions RAN4 hopefully agrees during Rel-19 will substantially impact the HPUE work.

Our contribution is mostly a resubmission of [1], with some further considerations and clarifications.

- Discussion

General aspects:

We begin by discussing general aspects related to this rather big evolutionary step. As discussed earlier, the MSD requirements specified before concluding all the agreements related to HPUE MSD rules should remain intact in the specifications. This is to respect all the work done during the past releases, as well as not to change any of the existing MSD requirements.

Proposal 1: All PC2 MSD and PC1.5 MSD tables specified before concluding all new HPUE MSD rules will remain in untouched TS 38.101-1 and TS38.101-3 after new HPUE MSD rules apply for new HPUE combinations.

Most recent WF [4] is worded in a way that either Equations or Look-Up tables could be included in TS for HPUE MSD. Equations which by their nature are very complex are used to derive the values for the Look-Up Tables. Table presentation is way simpler in TS. Our original proposal to use Equations was proposed for the case when the 3GPP procedure of explicitly specifying MSD for PC2 and PC1.5 would have continued. Now that the group has agreed not to specify HPUE MSD explicitly for each band but using equations/LUT’s instead, LUT is much better approach for that purpose.

Proposal 2: Agree to use Look-Up Tables in TS38.101-1 and TS 38.101-3 to define HPUE MSD for the new HPUE combinations (For which HPUE MSD is not explicitly specified).

1UL cases:

We assumed 6dB PRX/DRX imbalance between aggressor power, and 0.5dB additional noise for 2Tx cases. For single Tx we assumed interferer power to increase 3dB when power increases 3dB. Specifically, for the PRX/DRX assumption we did look at both practical RFFE implementations as well as the resulting 2Tx MSD. These two should be reasonably well balanced, meaning the PC2 2Tx MSD should be reasonable compared to PC2 1Tx MSD. We note that all the parameters used are coarse simplifications, and in real devices they may be different. For instance, cross-band noise may increase more or less than 3dB depending on the case and UL harmonic can increase a bit more than 3dB, or a bit less depending on the case. Even there are some variations in the increase of the noise level with higher power, the real difference to resulting MSD is very small, negligible, accounting the overall amount of simplification achieved. We illustrate this by an example with small, intermediate and large cross-band MSD. It can be noted that even if there is deviation from 3dB assumption, the real difference in MSD is small.

Table 1 Impact of interference increase difference with 1dB/5dB/15dB PC3 MSD

We propose the following LUT for 1UL cases for consideration. Please note that it would be possible to further reduce the number of lines in the table by applying some kind or merging; we didn’t propose any before seeing everyone’s proposals.

Proposal 3: Use the following LUT for 1 UL cases (UL harmonics, Harmonic Mixing, Cross-band) for discussions.

Table 2 LUT for 1UL aggressor cases

Cases where PC3 MSD does not exist are potentially very tricky, as there is no non-zero reference value. We have had some frequency relation-based proposals earlier on how and for which combinations to capture the HPUE MSD, however they would be rather complex to be used in specifications using LUT format. Instead of that, we propose to capture such HPUE MSD cases still explicitly in existing HPUE MSD tables.

Proposal 4: PC2/PC1.5 MSD is specified for each applicable combination in existing PC2/PC1.5 MSD tables for the HPUE combinations for which PC3 MSD is not specified.

There is one important aspect which needs to be well understood, that is how to procedurally handle HPUE combinations which don’t have PC3 MSD but could have PC2 and/or PC1.5 MSD. There have been discussions on making HPUE to automatically apply, in which case the process how to manage cases with no PC3 MSD but PC2 and/or PC1.5 MSD. We can envision a few alternatives. One is to browse through all PC3 combinations which don’t have MSD and identify those who would need small PC2 and/or PC1.5 MSD. At least for 1UL aggressor cases these combinations are very likely those which are within same or adjacent band frequency group. Another one would be to agree small PC2 and/or PC1.5 MSD all combinations with no PC3 MSD. Third would be to agree no PC2 and/or PC1.5 MSD for these combinations.

We note that basically the approach how to handle these cases should be agreed in this meeting to have draft proposal on impacted combinations in May meeting, and official CR in August meeting. Our preference would be to browse through all combinations with no PC3 MSD and propose combinations which would require PC2 and/or PC1.5 MSD. Note that any MSD analysis for the identified combinations is not needed as we can decide the MSD allowance beforehand. For instance, we could agree that the following MSD’s are applied for identified combinations.

Table 3 Example MSD to be specified for identified combinations with no PC3 MSD

Proposal 5: Browse through all combinations with no PC3 MSD and propose combinations which would require PC2 and/or PC1.5 MSD

2UL cases:

There are multiple different TX configurations and respective power combinations for 2UL cases. The number of Tx per band can be 1 or 2, TX power per band can be 20/23/26/29dBm, and UE power class can be PC3/PC2/PC1.5. All this combined with IMD order, IMD indexes per band, and power increase per band could make things extremely complicated in worst case.

To tackle all of this, we did derive the proposed LUT based on simple approach, where the interferer power increase is calculated by multiplying the UE power class increase by the IMD order using scaling factor 1. For instance, PC2 IMD4 interferer increase over PC3 is 3dB*4*1=12dB. This approach is universal and can be used for 1+1, 2+1, 1+2, 2+2 Tx per band i.e. all cases discussed by now. It is important to agree that 2UL LUT applies for any combination of number of TX’es per band, to avoid extra hassle for 3Tx and 4Tx. We assumed same 6dB PRX/DRX interference imbalance as for 1UL case when deriving the reference PC3 interferers.

Like in 1UL case, it would be possible to further reduce the number of lines in the table by applying some kind or merging; we didn’t propose any before seeing everyone’s proposals.

An alternative for this could be using one step more rigorous approach in which the interferer power increase is calculated by multiplying the power increase per each band by the IMD indexes per each band. This method would give more accurate interferer increase for combinations where the power per band in is inequal, e.g. UE PC1.5 with PC3 FDD+PC1.5 TDD where the power config in PC1.5 MSD is 20+27.8dBm, but the big drawback is that then referencing the right LUT table would be extremely difficult and the evolutionary step of defining HPUE MSD easily would be lost.

The IMD interferer may behave differently depending on the scenario and source of the interference. The interference may not scale up proportionately to the multiplication of power and IMD order i-e scaling factor could be <1. Consequently, with scaling factor 1, HPUE IMD interferers especially for cases with high IMD order (IMD4, IMD5, IMD>5) become large. To address this, we propose to discuss if this is something which should be resolved somehow. Among possibilities could be to use scaling factor <1 for IMD4 and above, or to limit the HPUE ΔMSD for IMD4 and above to a certain value.

Proposal 6: LUT for 2UL cases (Intermodulations) applies for 1Tx+1Tx, 1Tx+2Tx, 2Tx+1Tx, and 2Tx+2Tx cases.

Proposal 7: Use the following LUT’s for 2UL cases (Intermodulations) for initial discussions. Final LUT format(s) should be further simplified.

Proposal 8: RAN4 should discuss if some kind of limitations to HPUE MSD’s for ≥[IMD4] should be considered.

- Conclusion

Considerations on PC2 and PC1.5 MSD were provided with the following proposals.

Proposal 1: All PC2 MSD and PC1.5 MSD tables specified before concluding all new HPUE MSD rules will remain in untouched TS 38.101-1 and TS38.101-3 after new HPUE MSD rules apply for new HPUE combinations.

Proposal 2: Agree to use Look-Up Tables in TS38.101-1 and TS 38.101-3 to define HPUE MSD for the new HPUE combinations (For which HPUE MSD is not explicitly specified).

Proposal 3: Use the following LUT for 1 UL cases (UL harmonics, Harmonic Mixing, Cross-band) for discussions.

Proposal 4: PC2/PC1.5 MSD is specified for each applicable combination in existing PC2/PC1.5 MSD tables for the HPUE combinations for which PC3 MSD is not specified.

Proposal 5: Browse through all combinations with no PC3 MSD and propose combinations which would require PC2 and/or PC1.5 MSD

Proposal 6: LUT for 2UL cases (Intermodulations) applies for 1Tx+1Tx, 1Tx+2Tx, 2Tx+1Tx, and 2Tx+2Tx cases.

Proposal 7: Use the following LUT’s for 2UL cases (Intermodulations) for initial discussions. Final LUT format(s) should be further simplified.

Proposal 8: RAN4 should discuss if some kind of limitations to HPUE MSD’s for ≥[IMD4] should be considered

- Reference

[1] R4-2501579, “MSD beyond PC3”, Qualcomm France

[2] R4-2501955, “1UL MSD simplifications rules for Rel-19”, Skyworks Inc, Nokia

[3] R4-2500218, “Simplification of HPUE 2UL DeltaMSD”, Skyworks Inc.

[4] R4-2420380, “WF on MSD rule”, Skyworks, Murata, Nokia, Qualcomm