3GPP TSG-RAN WG4#114-bis	R4-2504120

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.6.1

Source: 	Qualcomm France

Title: 	RedCap PC2 FDD REFSENS

Document for:	Approval

# Introduction

RedCap PC2 1Rx FD-FDD REFSENS for n1 and n3 is analyzed in this contribution.

- Discussion

# WF for REFSENS is as follows [2]

Issue 1-2-1: REFSENS for HD-FDD

Online agreement:

- The existing reference sensitivity for RedCap PC3 for HD-FDD in sub-clause 7.3I of TS 38.101-1 can be applicable for RedCap PC2 for HD-FDD.

Issue 1-2-2: REFSENS for 2Rx in FD-FDD

Online agreement:

- 2Rx RSD for 1Tx PC2 is provided for non-Redcap UE in Table 7.3.2-1c in TS38.101-1, and this requirement could apply to RedCap UE for channel BWs up to 20MHz.

Issue 1-2-3: REFSENS for 1Rx in FD-FDD

Agreement:

- Further discuss the following formats for further evaluations

  - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - Evaluate the values of ΔR band by band

  - Note: Other options are not precluded

Way forward: It is encouraged for companies to provide analysis for the above options and give the preference in next meeting.

It was agreed to reuse FDD PC2 1Tx/2Rx Non-RedCap REFSENS for RedCap FDD PC2 1Tx/2Rx FD-FDD. Hence we use those REFSENS numbers as a reference when analyzing respective 1Rx REFSENS. We note that, as usual, the PC2 1Tx/2Rx REFSENS numbers, especially the RSD, is a compromise from several companies proposals and may not exactly reflect what would be the reality of RSD behaviour for different BW’s in real UE’s. Despite that, we think that 1Rx and 2Rx REFSENS should be reasonably well aligned. Figure 1 below lists PC2 1Tx/2Rx REFSENS for n1 and n3 in dBm format, i.e. accounting RSD for each BW.

Figure 1 FD-FDD PC2 2RX REFSENS

We assumed 9dB NF for n1 and 12dB NF for n3. Then we calculated the combined TX noise/TX leakage which resulted in PC2 2RX REFSENS assuming 6dB difference between PRX/DRX interference and calculated respective PRX REFSENS.

n1:

n3:

REFSENS for 1Rx FD-FDD is as follows. The numbers are verified by actual measurements using typical 3GPP assumptions for RFFE.

Figure 2 FD-FDD PC2 1Rx REFSENS

We note that in this case, the delta between PC2 2Rx and PC2 1Rx is 3dB for all n1 and n3 BW’s. If the PRX/DRX imbalance were larger than 6dB/smaller than 6dB, the delta would be larger than 3dB/smaller than 3dB, respectively.

According to WF, the following options to define FD-FDD REFSENS were listed.

- Further discuss the following formats for further evaluations

  - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - Evaluate the values of ΔR band by band

  - Note: Other options are not precluded

REFSENS according to option 1 calls to evaluate RSD for each BW. The difference for 5MHz RSD is because the 5MHz PC3 RSD was 0.5dB tighter than RSD for 10/15/20MHz.

Figure 3 RSD for FD-FDD 1RX REFSENS (vrt PC3 1RX REFSENS)

REFSENS according to option 2 calls to evaluate ΔR. Please note, that here we define ΔR for each band/bandwidth. In case ΔR would be, after RAN4 discussions, one number for all BW’s for all bands then a simplified way to capture ΔR should be used e.g. like in PC3 1Rx.

Figure 4 ΔR for FD-FDD PC2 REFSENS (vrt PC2 2RX REFSENS)

In essence, the dBm values are the ones that matter, i.e. there is no fundamental reason why either Option 1 or Option 2 would need to be used. However, in case e.g. ΔR would turn out to be same number for all bands, then Option 2 would be more desirable.

Observation 1: There is no fundamental reason driving to choose Option 1 or Option 2 as vehicle to define FD-FDD RedCap PC2 1Rx REFSENS. RAN4 should discuss based on the proposed numbers which one is more suitable.

- Conclusion

PC2 1RX FD-FDD REFSENS for n1 and n3 is analyzed in this contribution with the following proposals.

Proposal 1: Use the following numbers for FD-FDD PC2 1Rx RedCap REFSENS.

Figure 5 RSD for FD-FDD 1RX REFSENS (vrt PC3 1RX REFSENS)

Figure 6 ΔR for FD-FDD PC2 REFSENS (vrt PC2 2RX REFSENS)

Observation 1: There is no fundamental reason driving to choose Option 1 or Option 2 as vehicle to define FD-FDD RedCap PC2 1Rx REFSENS. RAN4 should discuss based on the proposed numbers which one is more suitable.

- Reference

[1] R4-2502874, “WF on NR PC2 RedCap UE in FDD bands”, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, Nokia