# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504155
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.10.2.2.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Tx requirements Issue 2-1: NR-NTN PC2 Agreement: - Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible - 1Tx handheld - 2Tx handheld - 1Tx non-handheld - 2Tx non-handheld - If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19 Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed Issue 2-3: NR-NTN PC1 Agreement: - RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19. Issue 2-4-1: PC2 ACLR Agreement: - PC2 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-2: PC1.5 ACLR Agreement: - PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-3: PC1 ACLR Agreement: - PC1 ACLR is [31 or 37] dBc for NR-NTN UE - Further study averaging value, i.e. 34. Issue 2-5: SEM Agreement: - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202 Issue 2-6: MPR/A-MPR Agreement: - For PC 2 and PC 1.5 MPR: - Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc. - Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc. - For PC 1 MPR: - Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc: - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec. - Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc - Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

This contribution further discussed the UE Tx RF requirements for NR NTN HPUE.

- Discussion

In last meeting, it was agreed that defining the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible

  - 1Tx handheld

  - 2Tx handheld

  - 1Tx non-handheld

  - 2Tx non-handheld

And in last meeting, how much ACLR for PC 2 to choose was discussed based on the co-existence study that observed the required ACLR for NR-NTN PC2 is less than 20 dB with 1.5km isolation distance assumed, finally, the discussion focused on whether to reuse 30dB ACLR for PC3 or reuse 31dB ACLR for TN PC2.

To research the impact of different ACLR to the UE Tx requirements, we did some MPR simulations due to the ACLR values only impact the MPR requirements.

We did the MPR simulation by configured different parameters given as shown in table 2-1 based on the agreed simulation assumption:

- For PA model calibration:

  - DFT-s-OFDM QPSK 20 MHz

  - 100RB0

  - 4dB post PA loss

  - 1dB MPR

  - Carrier leakage 28dBc

  - IQ image:28 dBc

  - CIM3: 60 dBc

  - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB

  - SEM: existing PC3 SEM

  - EVM: 17.5%

Table 2-1 MPR simulation cases for PC2 1Tx

|  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| UE CBW | 10MHz | 10MHz | 30MHz | 10MHz | 10MHz | 10MHz |
| SCS | 15kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| OFDM type | CP-OFDM | CP-OFDM | CP-OFDM | DFT-s-OFDM | CP-OFDM | CP-OFDM |
| modulation | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
| ACLR | 30dB 31dB | 30dB 31dB | 30dB 31dB | 30dB 31dB | 30dB 31dB | 30dB 31dB |

In the simulation, the MPR values for 30dB ACLR and 31 dB ACLR are compared in each simulation case. In additional,

- Case 1 and case 2 are to verify the effects of different SCS on the MPR improvements;

- Case 2 and case 3 are to verify the effects of different UE CBW on the MPR improvements;

- Case 2 and case 4 are to verify the effects of different OFDM types on the MPR improvements;

- Case 2 and case 5 and case 6 are to verify the effects of different modulations on the MPR improvement;

We didn’t simulate 256QAM since the modulation of 256QAM for NR NTN has been removed from the spec in last meeting.

Case 1:

Figure 2-1 MPR results for 10 MHz with 15kHz SCS, CP-OFDM, QPSK

Case 2

Figure 2-2 MPR results for 10MHz with 30kHz SCS, CP-OFDM, QPSK

Case 3

Figure 2-3 MPR results for 30MHz with 30kHz SCS, CP-OFDM, QPSK

Case 4

Figure 2-4 MPR results for 10MHz with 30kHz SCS, DFT-s-OFDM, QPSK

Case 5

Figure 2-5 MPR results for 10MHz with 30kHz SCS, CP-OFDM, 16QAM

Case 6

Figure 2-6 MPR results for 10MHz with 30kHz SCS, CP-OFDM, 64QAM

Figure 2-1 to Figure 2-6 gave the MPR simulation results of 30dB ACLR and 31dB ACLR for six cases, and also compared the MPR value of 30dB ACLR with the MPR value of 31dB ACLR for each cases. Based on the simulation results, in can be observed:

- There was no significant difference in MPR improvement due to different SCSs compared to case 1 and Case 2. To save time, 30kHz SCS was used in other simulation cases;

- Although different UE CBWs will cause some difference in the MPR improvement compared to case 2 and case 3, it also can be seen that the MPR improvements resulting from relaxing ACLR values from 31dB to 30dB are only for some outer RB allocations not for all outer RB allocations in both of case 2 and case 3 as shown in figure 2-2 and figure 2-3. To save time, 10MHz UE CBW with 30kHz SCS was used in simulation case 4, case 5 and case 6;

- Case 2 as shown in Figure 2-2 is for CP-OFDM QPSK, the MPR improvements resulting from relaxing ACLR values from 31dB to 30dB are up to 0.7 dB only on several outer RB allocation points, and most outer RB allocations and all inner RB allocation have no MPR improvement.

- Case 4 as shown in Figure 2-4 is for DFT-s-OFDM QPSK, it can be observed that the MPR improvements resulting from relaxing ACLR values from 31dB to 30dB is tiny and only 0.1dB for all RB allocation.

- Case 5 and case 6 in Figure 2-5 and Figure 2-6 are for CP-OFDM 16QAM and 64 QAM, the MPR improvements resulting from relaxing ACLR values from 31dB to 30dB are similar with case 2 for CP-OFDM QPSK, only on several outer RB allocation points with 0.7 dB MPR improvement.

- For case 2, case 5 and case 6, the maximum MPR value for ACLR from 31dB relaxed to 30dB doesn’t change.

From above observations, it can get regardless of OFDM types and modulation orders, the MPR improvements resulting from relaxing ACLR values from 31dB to 30dB for the maximum MPR value is 0dB. Since the MPR for outer RB allocations are defined based on the maximum MPR value, there is no any MPR improvements for ACLR from 31dB relaxed to 30dB for PC2 NR-NTN UE.

Therefore, PC2 NR-NTN UE can reuse the MPR values of PC2 NR-TN UE whether using 30dB ACLR or 31 dB ACLR.

Proposal 1: Existing MPR requirements for NR-TN PC2 UE in TS 38.101-1 can apply to NR-NTN PC2 UE whether using 30dB ACLR or 31 dB ACLR.

From current spec, the ACLR will only impact the MPR requirements, and we can see from above simulations, it’s no obvious improvement for MPR requirements resulting from relaxing ACLR values from 31dB to 30dB. So it’s unnecessary to relax 1dB ACLR for NR-NTN PC2 UE.

Similar reason applies to for NR-NTN PC1.5 UE. And in last meeting, it was agreed that PC1.5 only for non-handheld NR-NTN UE with 2Tx in Rel-19.

Proposal 2: Existing MPR requirements for NR-TN PC1.5 UE with 2Tx in TS 38.101-1 can apply to NR-NTN PC1.5 UE.

Proposal 3: Adopt 31dB ACLR for NR-NTN PC2 and PC 1.5 UE.

In current spec TS 38.101-1, the MPR requirements of NR TN PC1 UE with 1Tx for non-handheld UE is the same with the MPR requirements of NR TN PC3 UE. Since, for PC3 UE, the 30dB ACLR means the Tx leakage is -7dBm in the adjacent channel for the maximum output power 23dBm, and for PC1 UE, the 37dB ACLR means the Tx leakage is -6dBm in the adjacent channel for the maximum output power 31dBm.

In last meeting, the ACLR for NR-NTN PC1 UE was proposed 31dB, 34dB and 37dB (37dB is to keep the same value with NR-TN PC1 UE).

If 31dB ACLR applies to NR-NTN PC1 UE, it means the Tx leakage is 0 dBm in the adjacent channel for the maximum output power 31dBm. It equals to the ACLR relaxed from 31dB to 26 dB for PC2 UE or the ACLR relaxed from 30 dB to 23 dB for PC2 UE, so we did the MPR simulation for PC2 UE with 26dB ACLR and PC3 UE with 23dB ACLR based on the parameters 10MHz with 30kHz SCS, CP-OFDM, QPSK as shown in Figure 2-7:

Figure 2-7 MPR results for PC2 compared to 31dB ACLR and 26dB ACLR

Figure 2-8 MPR results for PC3 compared to 30dB ACLR and 23dB ACLR

From above simulation results for PC2 and PC3 using different ACLRs as shown in Figure 2-7 and Figure 2-8, it observed that

- For PC2, the MPR improvements resulting from relaxing ACLR values from 31dB to 26dB are up to 0.8 dB only on several outer RB allocation points, and most outer RB allocations and all inner RB allocation have no MPR improvement. The maximum MPR value for ACLR from 31dB relaxed to 26dB doesn’t change.

- For PC3, the MPR improvements resulting from relaxing ACLR values from 30dB to 23dB are up to 1.4 dB only on several outer RB allocation points, all inner RB allocations have no MPR improvement. the maximum MPR value for ACLR from 30dB relaxed to 23dB doesn’t change.

- Compared to the MPR of PC3 UE with 23 dB ACLR and the MPR of PC2 UE with 26 dB ACLR, the Tx leakage is 0 dBm in the adjacent channel for both of PC3 and PC2 UE. Although the outer MPR values of PC2 become larger than the outer MPR values of PC3, the maximum MPR value of PC2 are the same with the maximum MPR value of PC3.

Since the MPR for outer RB allocations are defined based on the maximum MPR value, there is no MPR increasing when the maximum output power increasing from 23dBm to 26 dBm. That is, the limit factor for the maximum MPR values for QPSK, 16QAM and 16QAM is SEM not ACLR and for 64QAM is EVM. SEM are the same for all power class, it can get the MPR values won’t obviously change when relaxed the ACLR for NR-NTN PC1 UE.

Proposal 4: Existing MPR requirements for NR-TN PC1 UE with 1Tx in TS 38.101-1 can apply to NR-NTN PC1 UE.

Proposal 5: Only consider non-handheld UE with 1Tx in Rel-19.

Like NR-NTN PC2 and PC1.5 UE, it’s unnecessary to relax the ACLR, since no MPR improvements by relaxing ACLR.

Proposal 6: Adopt 37dB ACLR for NR-NTN PC2 and PC 1 UE.

In last meeting, it was agreed that using 31dB ACLR as the PA model calibration condition for PC 1UE. Actually, it means the NR-NTN PC1 non-handheld UE is allowed to use one PA with worse performance than NR-TN PC1 non-handheld UE. It may cause NR-NTN PC1 non-handheld UE needs larger MPR values than NR-TN PC1 non-handheld UE.

Proposal 7: Adopt 37dB ACLR as the PA model calibration condition for PC 1UE whether using 30dB ACLR or 31 dB ACLR as the requirements.

- Conclusion

In this paper we give further discussion on the TX requirement for HPUE NTN. The proposals are as following:

Proposal 1: Existing MPR requirements for NR-TN PC2 UE in TS 38.101-1 can apply to NR-NTN PC2 UE whether using 30dB ACLR or 31 dB ACLR.

Proposal 2: Existing MPR requirements for NR-TN PC1.5 UE with 2Tx in TS 38.101-1 can apply to NR-NTN PC1.5 UE.

Proposal 3: Adopt 31dB ACLR for NR-NTN PC2 and PC 1.5 UE.

Proposal 4: Existing MPR requirements for NR-TN PC1 UE with 1Tx in TS 38.101-1 can apply to NR-NTN PC1 UE.

Proposal 5: Only consider non-handheld UE with 1Tx in Rel-19.

Proposal 6: Adopt 37dB ACLR for NR-NTN PC2 and PC 1 UE.

Proposal 7: Adopt 37dB ACLR as the PA model calibration condition for PC 1UE whether using 30dB ACLR or 31 dB ACLR as the requirements.

- Reference

[1] R4-2502594, WF on HPUE requirements for NR-NTN, Samsung, RAN4#114.