3GPP TSG-RAN WG4 Meeting #114bis		R4-2504168
Wuhan, China, 7– 11 April, 2025

Title:	R19 MPR for 3Tx

Source:	OPPO

Agenda item:	7.22.2

Document for:	Approval

- Introduction

This paper discusses the MPR for 3Tx PC2 and PC1.5 per WF [1].

This is based on paper [2].

- Discussion

  - MRP for PC2 with 3Tx

2.1.1 Simulation assumptions

The assumptions are:

- 3 PC3 PAs with equal PSD and equal back off

- CP-OFDM / DFT-S-OFDM

- CIM3 = -60dBc

- CC leakage = -28dBc for pi/2 bpsk, qpsk, 16qam, 64qam, and -34dBc for 256qam

- IQ image = -28dBc for pi/2 bpsk, qpsk, 16qam, 64qam, and -34dBc for 256qam

- RFFE IL = -4dB

- PC3 PA calibrate point: 20MHz 100RB@0, QPSK, DFT-S-OFDM, 15khz SCS, meet 30dB ACLR

- Forward IMD and Reverse IMD considered

The MPR results can be found in annex A.

2.1.2 MRP requirements

Based on above MPR region definition, the MPR requirements are summarized in table 1 for DFT-S-OFDM and CP-OFDM with around 1dB implementation margin considered.

Table 1 MPR for 3T PC2

|  | DFT-S-OFDM |  |  | CP-OFDM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Edge RB | Outer RB | Inner RB | Edge RB | Outer RB | Inner RB |
| Pi/2 BPSK | 1 | 1 | 0.5 | N/A | N/A | N/A |
| QPSK | 2 | 2 | 0.5 | 3.5 | 3.5 | 1.5 |
| 16QAM | 2.5 | 2.5 | 0.5 | 3.5 | 3.5 | 1.5 |
| 64QAM | 3 | 3 | 1 | 3.5 | 3.5 | 2.5 |
| 256QAM | 3.5 | 3.5 | 3.5 | 5.5 | 5.5 | 5.5 |

From table 1 it can be seen that with 3PA transmit concurrently, the MPR required is slightly smaller than the current 2Tx PC2 UL MIMO MPR requirements. Therefore, the 2Tx UL MIMO MPR table can be reused for 3Tx.

Observation 1:   For PC2, with 3 PC3 PA transmit concurrently, the MPR required is smaller than the current 2Tx PC2 MPR requirements.

Proposal 1:         Reuse current PC2 2Tx UL MIMO MPR table for 3Tx with PC2 power class.

  - MRP for PC1.5 with 3Tx

2.1.1 Simulation assumptions

The assumptions are:

- 3 PC2 PAs with equal PSD and equal back off

- CP-OFDM / DFT-S-OFDM

- CIM3 = -60dBc

- CC leakage = -28dBc for pi/2 bpsk, qpsk, 16qam, 64qam, and -34dBc for 256qam

- IQ image = -28dBc for pi/2 bpsk, qpsk, 16qam, 64qam, and -34dBc for 256qam

- RFFE IL = -4dB

- PC2 PA calibrate point: 20MHz 100RB@0, QPSK, DFT-S-OFDM, 15khz SCS, meet 31dB ACLR

- Forward IMD and Reverse IMD considered

The MPR results can be found in annex B.

2.1.2 MRP requirements

Based on above MPR region definition, the MPR requirements are summarized in table 2 for DFT-S-OFDM and CP-OFDM with around 1dB implementation margin considered.

Table 2 MPR for 3T PC1.5

|  | DFT-S-OFDM |  |  | CP-OFDM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Edge RB | Outer RB | Inner RB | Edge RB | Outer RB | Inner RB |
| Pi/2 BPSK | 1.5 | 1.5 | 1 | N/A | N/A | N/A |
| QPSK | 1.5 | 1.5 | 1 | 3.5 | 3.5 | 1.5 |
| 16QAM | 2.5 | 2.5 | 1 | 4 | 4 | 2.5 |
| 64QAM | 2.5 | 2.5 | 1 | 4 | 4 | 3 |
| 256QAM | 2.5 | 2.5 | 2.5 | 4.5 | 4.5 | 4.5 |

From table 2 it can be seen that with 3PA transmit concurrently, the MPR required is smaller than the current 2Tx PC1.5 UL MIMO MPR requirements. Therefore, the 2Tx UL MIMO MPR table can be reused for 3Tx.

Observation 2:   For PC1.5, with 3 PC2 PA transmit concurrently, the MPR required is smaller than the current 2Tx PC1.5 MPR requirements.

Proposal 2:         Reuse current PC1.5 2Tx UL MIMO MPR table for 3Tx with PC1.5 power class.

- Conclusion

This paper shared the MPR simulation results for 3Tx with PC2/1.5 and got below observations and proposals.

Observation 1:   For PC2, with 3 PC3 PA transmit concurrently, the MPR required is smaller than the current 2Tx PC2 MPR requirements.

Proposal 1:         Reuse current PC2 2Tx UL MIMO MPR table for 3Tx with PC2 power class.

Observation 2:   For PC1.5, with 3 PC2 PA transmit concurrently, the MPR required is smaller than the current 2Tx PC1.5 MPR requirements.

Proposal 2:         Reuse current PC1.5 2Tx UL MIMO MPR table for 3Tx with PC1.5 power class.

- Reference

[1] R4-2503037 WF on UE RF requirements for MIMO enhancement, Samsung, #114 2025-02

[2] R4-2417912 R19 MPR for 3Tx, OPPO, #113 2024-11

Annex A: 3T PC2

Annex A.1 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B: 3T PC1.5

Annex B.1 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]