3GPP TSG RAN WG4 Meeting #114bis	R4-2504174

Wuhan, China, 7th – 11th April 2025

Source:		Rohde & Schwarz

Title:	Views on propagation delay correction

Agenda Item:	7.10.4.1

Document for:	Approval

# Introduction

During the last RAN4 meeting good progress was made on the definition of the NGSO channel model and agreements were captured in [1]. A new aspect, which was initially brought up in [2], is to correct the propagation delay by considering the earth rotation during the signal flight time. In the following we share our views on the issue

# Discussion

The way forward in [1] captures the problem of the correction of the propagation delay as follows:

Issue 1-1-2 Correction of Propagation Delay Calculation - WF - RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay - Considering the earth rotation, the propagation delay, DelayERtR can be derived as DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR> Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b - The proponent is encouraged to provide the detail information, e.g., reference, if possible, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting - Participating companies are encouraged to examine this issue from their perspective.

From this way forward, we have some identified some issues that require clarification on if or how the correction should be included in the calculation of the propagation delay.

Firstly, from the description it is not clear whether the correction needs to be applied to DL, UL or both DL and UL. For example for UE testing, one could imagine that while the propagation delay of the DL signal could be adjusted accordingly, it would need to be clarified whether on UE side the same would apply for the UL as well.

Proposal 1: Clarify how or if the propagation delay correction applies to UL, DL or both.

Furthermore, in our understanding the formula in the WF leads to an iterative process, which can lead to large increase in computational effort, especially when multiple satellites need to be considered like in RRM scenarios.

Typically rtRECEFand vtRECEF at tR are not known beforehand because the ECEF is rotating, so while it is the same ECEF system the coordinates of a point not connected to the ECEF (a.k.a. the satellite) change. In orbit calculation this iteration leads to re-calculation of satellite positions multiple times, i.e. all Kepler algorithms (or whatever is used) must be fully re-calculated multiple times.

Observation 1: Applying the formula from the WF leads to an iterative process, increasing the computational effort on TE side.

This iterative process could however be avoided in case rtRECEFand vtRECEF at tR are known before starting the calculation. Another option, would be to restrict the process to a single iteration, this may be also be feasible, but also depends on the required accuracy.

Proposal 2:  Discuss further in RAN4 how to limit the iterative process when calculating the propagation delay correction.

Therefor we think that if and how to apply the propagation delay correction as described in the WF requires further clarifications, before it can be included in the general procedure for the NGSO doppler/delay profile generation.

# Summary

In this contribution we shared our views on open issues for the doppler and delay profiles for NGSO satellites. We have the following observation and proposals:

Proposal 1: Clarify how or if the propagation delay correction applies to UL, DL or both.

Observation 1: Applying the formula from the WF leads to an iterative process, increasing the computational effort on TE side.

Proposal 2:  Discuss further in RAN4 how to limit the iterative process when calculating the propagation delay correction.

# References

[1]	R4-2502314; “Way Forward for [114][325] NTN_testing_NGSO_channel_model”; Samsung; RAN4#114 February 2025

[2]	R4-2500116; ”Discussion on NTN testing for NGSO”; Qualcomm; RAN4#114 February 2025