3GPP TSG-RAN4 Meeting #114bis	R4-2504183

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title: 	On Ku-band general requirements

Agenda Item:	7.11.3

Document for:	Discussion and Approval

# Introduction

RAN4 is discussing general requirements for Ku-band NTN VSAT. Way forward [1] was agreed in RAN WG4 Meeting #114 in Athens in February 2025.

This document further discusses and makes proposals on the following sub-topics captured to the Way Forward.

Sub-topic 3-6: FR1-NTN ARFCN definitions

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Sub-topic 3-10: Default Tx Rx frequency separation

Agreement: Variable TX RX spacing is assumed, how it is specified is FFS.

Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.

# Discussion

## FR1-NTN ARFCN definitions

RAN4 agreed the following in the meeting #114 based on 30kHz SCS assumption.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

However, RAN plenary meeting further discussed the supported SCS for FR1 numerology and 15kHz was added to the WID [2] and therefore RAN4 should discuss if this change should also be reflected to the NR-ARFCN of FR1 numerology bands or if 30kHz raster is still valid assumption also for the SCS= 15kHz ?

Observation 1: RAN plenary meeting further discussed the supported SCS for FR1 numerology and 15kHz was added to the WID [2].

Proposal 1: RAN4 to discuss if this change in supported SCS for FR1 numerology bands should also be reflected to the NR-ARFCN of FR1 numerology bands or if 30kHz raster is still valid assumption also for the SCS= 15kHz ?

## General RF requirements impact on numerologies dependency

RAN4 agreed the following in the meeting #114:

Sub-topic 3-10: Default Tx Rx frequency separation

Agreement: Variable TX RX spacing is assumed, how it is specified is FFS.

Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.

Proposal 2: The variable TX/RX spacing can be defined as shown in Figure 1 and tables below for Ku-bands with channel raster agreed in the WF.

Figure 1. Minimum and maximum TX/RX spacing for FR1 and FR2 numerologies

|  | Lowest DL 5MHz | Highest DL 5MHz | Lowest UL 5MHz | Highest UL 5MHz | Min spacing | Max spacing |
| --- | --- | --- | --- | --- | --- | --- |
| Band nY | 10 702.500 | 12 747.510 | 14 002.500 | 14 497.500 | 1 254.990 | 3 795.000 |
| Band nB | 10 702.500 | 12 747.510 | 13 752.510 | 13 997.490 | 1 005.000 | 3 294.990 |
| Band nD | 10 702.500 | 12 747.510 | 12 752.490 | 13 247.490 | 4.980 | 2 544.990 |
|  | Lowest DL 50MHz | Highest DL 50MHz | Lowest UL 50MHz | Highest UL 50MHz | Min spacing | Max spacing |
| Band nY | 10 725.120 | 12 724.920 | 14 025.120 | 14 474.880 | 1 300.200 | 3 749.760 |
| Band nB | 10 725.120 | 12 724.920 | 13 775.160 | 13 974.960 | 1 050.240 | 3 249.840 |
| Band nD | 10 725.120 | 12 724.920 | 12 775.080 | 13 224.840 | 50.160 | 2 499.720 |

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n248 | -1254.99 to -3795.00 MHz |
| n247 | -1005.00 to -3294.99 MHz |
| nD FR1 | -4.98 to -2544.99 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -1300.20 to -3749.76 MHz |
| n508 | -1050.24 to -3249.84 MHz |
| nD FR2 | -50.16 to -2499.72 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

The TX – RX carrier centre frequency separation is very small in case of band nD (priority 2 band), but small delta is not a problem for the NTN VSAT as long as RX and TX are not operated concurrently (Half duplex operation)

# Conclusions

Observation 1: RAN plenary meeting further discussed the supported SCS for FR1 numerology and 15kHz was added to the WID [2].

Proposal 1: RAN4 to discuss if this change in supported SCS for FR1 numerology bands should also be reflected to the NR-ARFCN of FR1 numerology bands or if 30kHz raster is still valid assumption also for the SCS= 15kHz ?

Proposal 2: The variable TX/RX spacing can be defined as shown in Figure 1 and tables below the figure for Ku-bands with channel raster agreed in the WF.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n248 | -1254.99 to -3795.00 MHz |
| n247 | -1005.00 to -3294.99 MHz |
| nD FR1 | -4.98 to -2544.99 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -1300.20 to -3749.76 MHz |
| n508 | -1050.24 to -3249.84 MHz |
| nD FR2 | -50.16 to -2499.72 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

# References

[1] R4-2502290	Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat), TSG-RAN WG4 Meeting #114, Feb 2025

[2] RP-250799

[3] 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)