# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504238

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.3

Source: 	OPPO

Title: 	RRM core requirement and testing framework for beam management

Document for:	Approval

- Introduction

In 3GPP RAN #107[1], the “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” has been agreed, including following aspects:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Core requirements for the above three use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - For Beam Management, Positioning Accuracy enhancement and CSI prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring. Note: the following aspects may be considered: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g. CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In this contribution, we will continue to discuss the RRM core requirement and testing framework for beam management.

- Core requirement

KPI:

In RAN1#116 meeting, 4 options regarding the content in the BM report of inference results have been agreed, as listed below,

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Based on the agreements, metrics for BM model tests may depend on RAN1’s final decision, e.g.,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, then Beam prediction accuracy could be a test KPI accordingly.

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, then both Beam prediction accuracy and RSRP accuracy should be considered when defining RAN4 test KPIs

In RAN4 R19 tests, for both BM-Case1 and BM-Case2, three options have been agreed, i.e.,

Option 1: RSRP accuracy

Option 2: Beam prediction accuracy

Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among 		top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

For option1, in RAN4#112bis meeting, RAN4 agreed “If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]”.

However, from our understanding, the definition of absolute RSRP accuracy may not truly reflect the effectiveness of beam prediction in some cases. For example, if beam i is not the best choice, yet an AI/ML model designates it as the optimal beam. Despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

Observation 1: Absolute RSRP accuracy could be utilized as metric for L1-RSRP reporting.

Observation 2: At least for beam ID + L1-RSRP reporting, absolute RSRP accuracy may not truly reflect the effectiveness of beam prediction in some cases.

- For example, if beam i is not the best choice, yet an AI/ML model designates it as the optimal beam. Despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

Combining Option 1 and Option 2 could be a solution to address the above issue. This approach requires that the DUT fulfills both the requirement for beam prediction accuracy and the requirement for RSRP accuracy for each predicted beam.

Option 3 may handle the aforementioned issues as well. For instance, as long as the quality of the predicted top-K beams is not significantly worse than that of the strongest beam, it can ensure that the selected beam functions properly. In the textual description, to avoid ambiguity(as we see during the e-mail discussion after RAN4 #114 meeting), we add some updates to Option 3, as stated below:

Option 3(revised):  The successful rate for the correct prediction which is considered as maximum RSRP ground truth L1-RSRP among top-K predicted beams is larger than the RSRP ground truth L1-RSRP of the strongest beam – x dB,

Besides, from our understanding, introducing a certain margin for Option 2, could also be considered to avoid cases that predicting a suboptimal beam (where the suboptimal beam’s quality is close to the best beam) is incorrectly judged as a prediction failure. For example, as long as the actual measured value of the predicted beam is not significantly worse than that of the true best beam, the predicted beam could still be deemed correct. In fact, for Top-K/1 (%) Beam Prediction Accuracy, if a similar margin (e.g., −x dB) is added, it would essentially become the current Option 3. However, for Top-1/K (%), this scenario is not covered by the current Option 3, so incorporating such a margin could also be considered.

Given these observations, the overall Option 2 could be updated as follows:

Option 2(revised): Beam prediction accuracy with margin

    - Top-1 (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP of top-1 predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB,

    - Top-K/1 (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB,

    - Top-1/K (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP of  top-1 predicted beams is larger than the ground truth L1-RSRP of the K-th strongest beam – x dB.

Accordingly, we suggest updating the agreement achieved in RAN4 #114 meeting as: 1) If RSRP and beam ID are reported: combining Option 1 and Option 2(revised) will be used as the KPIs for beam prediction; 2) If only beam ID is reported, Option 2(revised) will be used as the KPIs for beam prediction.

Proposal 1: Revised option3 could be utilized as BM performance metric for beam ID + L1-RSRP reporting.

    - Option 3(revised):  The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB. FFS the value of x.

Proposal 2: Revise option2 to avoid cases that predicting a suboptimal beam (where the suboptimal beam’s quality is close to the best beam) is incorrectly judged as a prediction failure.

Option 2(revised): Beam prediction accuracy with margin

    - Top-1 (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP of top-1 predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB,

    - Top-K/1 (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB,

    - Top-1/K (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP of  top-1 predicted beams is larger than the ground truth L1-RSRP of the K-th strongest beam – x dB.

Proposal 3: Regarding Metrics/KPIs for beam prediction,

    - If RSRP and beam ID are reported, combining Option 1 and Option 2(revised) will be used as KPIs for beam prediction.

    - If only beam ID is reported, Option 2(revised) s will be used as KPIs for beam prediction.

Performance monitoring:

UE can monitor and estimate the performance of AI/ML based BM model/functionality through identified KPIs according to the discussion and conclusions in RAN1. Besides, stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes, includes: (1) obtaining a consistent monitoring result by considering multiple evaluating samples within an evaluation window, (2) assessing whether model monitoring should be handled at the UE level or the cell level.

Proposal 4: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

Impact of measurement error on BM performance evaluation:

A simulation assumption for beam prediction was agreed as a starting point for the evaluation in RAN4#112bis meeting. Currently, the assumption is to create a measurement error modeling through link-level simulations. And then hope to apply this link-level measurement error model to system-level simulation assumptions, ultimately completing the evaluation of the impact of measurement error on BM performance requirements.

The current discussion in RAN4 and preliminary simulation assumptions that made in precious meetings are beneficial to the progress of research, but still some issues need further clarification and discussion. For example, 1) considering that the testing environment assumes CDL channel modeling, it needs to be clarified whether the impact of measurement error obtained from the SLS UMA channel (measuring error impact added from a CDL channel evaluation) can be directly applied to the final OTA testing environment. Besides, 2) differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results. How to align the evaluation needs to be considered.

Observation 3: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered

- Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment

- Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results

- Testing framework

Regarding the testing framework on BM, from our understanding, following aspects need to be considered.

- The number of FR2 beams that RAN4 tests can support remains unclear. It is necessary to clarify the limitations regarding the FR2 beams or probes that TE vendors could support. Based on this limitation(if any), RAN4 can further consider how to reach a consensus on designing/conducting tests for BM use cases and how to set/form beam set A and B of different sizes for different BM cases.

- A potential test setup could be (as shown in Figure 1)

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

As agreed in RAN4 #114 meeting, we can start with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Figure 1 A potential test setup for BM tests

- Besides, it's also crucial to further consider that, as a data/scenario-driven solution, the chamber and beam patterns provided by different TEs may impact the test results of BM (e.g., due to variations in beam settings, different considerations on multipath, and other implementations). Besides, even for a same TE, different DUT hardware implementations may also lead to variations in receiving behavior and beam measurement results. Attention must be paid to ensure that the BM model constructed on the DUT side(e.g. by corresponding training dataset) can match(or approximate match) and be utilized in the chamber environment on the TE side. Otherwise, it becomes challenging for a DUT to pass the test under unknown chamber conditions/scenarios. This may require a close collaboration between DUT and TE to ensure that the model under test is compatible with the testing environment.

In RAN4 #114 meeting, RAN4 agreed to use CDL-based channel model as starting point. Following issues should be taken into account when identifying the BM test settings,

- to model correlation between the measurements of different beams to ensure that the beams remain predictable

- the predictability of the beams must not be too strong, as an extreme case where there is no distinction between different beams would undermine the necessity for BM testing

- the test should not be overly complex and should optimally utilize the existing test environment.

Regarding the TCI state issue, in RAN4 #113 meeting, we have got agreements as below:

- Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS

- RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not.

- Other way to know UE Rx beams is not precluded

- UE capability is one of the solutions

Based on our understanding, for the BM test, the TCI state associated with a predicted Tx beam is better to be known to DUT. Otherwise, it is difficult to evaluate the performance uncertainty due to different RX beam assumptions and implementation differences among different UEs. To address how DUT can determine a optimal receiving beam, a beam pair (TX-RX pair) matching process should be introduced into the test procedure to ensure that the DUT has opportunities to obtain the TCI state associated with a predicted Tx beam.

Proposal 4: For BM testability, RAN4 need to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 5: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

Proposal 6: The TCI state associated with a predicted Tx beam should be known to DUT.

- Conclusions

In this contribution, we have discussed the RRM core requirement and testing framework for beam management, and got the following proposals:

Observation 1: Absolute RSRP accuracy could be utilized as metric for L1-RSRP reporting.

Observation 2: At least for beam ID + L1-RSRP reporting, absolute RSRP accuracy may not truly reflect the effectiveness of beam prediction in some cases.

- For example, if beam i is not the best choice, yet an AI/ML model designates it as the optimal beam. Despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

Proposal 1: Revised option3 could be utilized as BM performance metric for beam ID + L1-RSRP reporting.

- Option 3(revised):  The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB. FFS the value of x.

Proposal 2: Revise option2 to avoid cases that predicting a suboptimal beam (where the suboptimal beam’s quality is close to the best beam) is incorrectly judged as a prediction failure.

Option 2(revised): Beam prediction accuracy with margin

- Top-1 (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP of top-1 predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB,

- Top-K/1 (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB,

- Top-1/K (%) with margin: The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP of  top-1 predicted beams is larger than the ground truth L1-RSRP of the K-th strongest beam – x dB.

Proposal 3: Regarding Metrics/KPIs for beam prediction,

- If RSRP and beam ID are reported, combining Option 1 and Option 2(revised) will be used as KPIs for beam prediction.

- If only beam ID is reported, Option 2(revised) s will be used as KPIs for beam prediction.

Observation 3: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered

- Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment

- Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results

Proposal 4: For BM testability, RAN4 need to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 5: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

Proposal 6: The TCI state associated with a predicted Tx beam should be known to DUT.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface