3GPP TSG-RAN WG4 Meeting#114bis		R4-2504239

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.5.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion of impacts on UE RF requirements and DL performance

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was approved. In this contribution, we continue to provide some initial views on methods for reducing the number of UE RX chains.

#### Issue 3-2: Discussion of impacts on DL performance for UE enabling one Rx RF chain mode - Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report. - DL performance degradation can be considered based on configurations - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI - Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies evaluation TPs

Discussion

Discussion of impacts on DL performance for UE enabling one Rx RF chain mode

In the previous meeting, the table format agreed to calculate relaxation for REFSENS. In this paper[2], imbalanced PRx/DRx noise level was provided. Based on this assumption and the table format , we also have calculated ΔRIBNC , the results as follow.

According to the current CA receiver sensitivity requirements, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. ΔRIBNC is caused by Tx leakage.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |

Table 1: Link budge of self-interference for CA_3(2A)(5MHz+5MHz,Wgap=65MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -97 | -93.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -101 | -97.3 | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - |
| LPF output (dBm) | -36.4 | -102 | -98.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -94 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.6 | - |
| Noise floor (including ADC noise) | - | - | - | -83.4 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -88 | - | - |
| SNR after ADC (dB) | - | - | - | - | -10(PCC) |
|  |  |  |  |  | -10.7(SCC) |

Table 2: Link budge of self-interference for CA_25(2A)(5MHz+5MHz,Wgap=55MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -110.5 | -95.5 | - | - |
| LPF attenuation | 6 | 1 | 1 | - | - |
| LPF output (dBm) | -29 | -111.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -93.5 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.6 | - |
| Noise floor (including ADC noise) | - | - | - | -82.9 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -85 | - | - |
| SNR after ADC (dB) | - | - | - | - | -10.9 (PCC) |
|  |  |  |  |  | -11.5(SCC) |

Table 3: Link budge of self-interference for CA_2(2A)(5MHz+5MHz,Wgap=50MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -98 | -93 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -112 | -97 | - | - |
| LPF attenuation | 8 | 1 | 1 | - | - |
| LPF output (dBm) | -31 | -113 | -98 | - | - |
| Noise floor for basic REFSENS | - | - | - | -95 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.6 | - |
| Noise floor (including ADC noise) | - | - | - | -84.4 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -87 | - | - |
| SNR after ADC (dB) | - | - | - | - | -10.6 (PCC) |
|  |  |  |  |  | -11.1(SCC) |

Table 4: Link budge of self-interference for CA_7(2A)(10MHz+5MHz,Wgap=55MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -98 | -93 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -112 | -97 | - | - |
| LPF attenuation | 8 | 1 | 1 | - | - |
| LPF output (dBm) | -31 | -113 | -98 | - | - |
| Noise floor for basic REFSENS | - | - | - | -95 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 8.6 | - |
| Noise floor (including ADC noise) | - | - | - | -86.4 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -85.8 | - | - |
| SNR after ADC (dB) | - | - | - | - | -7.6 (PCC) |
|  |  |  |  |  | -7.7(SCC) |

Table 5: Link budge of self-interference for CA_26(2A)(15MHz+10MHz,Wgap=10MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -92.7 | -69.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -106.7 | -73.3 | - | - |
| LPF attenuation | 11.5 | 1 | 1 | - | - |
| LPF output (dBm) | -33.5 | -107.7 | -74.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -89.7 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 8.6 | - |
| Noise floor (including ADC noise) | - | - | - | -81.1 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -59.5 | - | - |
| SNR after ADC (dB) | - | - | - | - | -11.1 (PCC) |
|  |  |  |  |  | -25.7(SCC) |

Observation1: For CA_n3 (2A (5MHz+5MHz, Wgap=55MHz), ΔRIBNC maybe 10dB for PCC and 10.7dB for SCC.

Observation2: For CA_n25 (2A (5MHz+5MHz, Wgap=65MHz), ΔRIBNC maybe 10.9dB for PCC and 11.5dB for SCC.

Observation3: For CA_n2 (2A (5MHz+5MHz, Wgap=50MHz), ΔRIBNC maybe 10.6 dB for PCC and 11.1dB for SCC.

Observation4: For CA_n7 (2A (10MHz+5MHz, Wgap=55MHz), ΔRIBNC maybe 7.6 dB for PCC and 7.7 dB for SCC.

Observation5: For CA_n26 (2A (15MHz+10MHz, Wgap=10MHz), ΔRIBNC maybe 11.1 dB for PCC and 25.7dB for SCC.

Conclusions

In this paper, we provide our views of impacts on DL performance for UE enabling one Rx RF chain mode, we have made the following observations.

Observation1: For CA_n3 (2A (5MHz+5MHz, Wgap=55MHz), ΔRIBNC maybe 10dB for PCC and 10.7dB for SCC.

Observation2: For CA_n25 (2A (5MHz+5MHz, Wgap=65MHz), ΔRIBNC maybe 10.9dB for PCC and 11.5dB for SCC.

Observation3: For CA_n2 (2A (5MHz+5MHz, Wgap=50MHz), ΔRIBNC maybe 10.6 dB for PCC and 11.1dB for SCC.

Observation4: For CA_n7 (2A (10MHz+5MHz, Wgap=55MHz), ΔRIBNC maybe 7.6 dB for PCC and 7.7 dB for SCC.

Observation5: For CA_n26 (2A (15MHz+10MHz, Wgap=10MHz), ΔRIBNC maybe 11.1 dB for PCC and 25.7dB for SCC.

- Reference

- R4-2502873,WF on fragmented DL carriers study, Mediatek Inc. TSG-RAN WG4 114 meeting