3GPP TSG-RAN WG4 Meeting #114bis	R4-2504242

# Wuhan, China, 7th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on CSSF optimization of R19 RRM enhancements

Agenda Item:	7.17.2.2

Document for:	Discussion

- Introduction

During last RAN4 meeting, good progress were achieved around the CSSF enhancement and the following progress were summarized in [1] as below:

Issue 1-1: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF - Option 1 (Apple, CMCC, HW, Ericsson, vivo): The enhanced CSSF apply to all the 3 aspects and do not define the priority. - Option 2 (Nokia, ZTE, CATT, CTC, vivo): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion. Agreement: - Option 1 is agreed Issue 1-3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19. Issue 1-3-4: Others Proposals (Nokia): - RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline. - The SCell activation delay requirement shall be improved considering the optimized measurement for CSSF reduction. Agreement: - This issue is closed

In this paper, we provide our further views on the maintenance of CSSF enhancement.

- Discussion

Regarding this sub topic, we share our views from the following aspects:

- Applicability requirement of L3 measurement delay reduction by optimizing CSSF

- Exact solutions

- 3 searchers for CSSF

- Applicability requirement of L3 measurement delay reduction by optimizing CSSF

- Around the applicable scenario, it is depicted as below in the approved WID[2]:

- For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

- Therefore, all CA, NR-DC and NE-DC, EN-DC are applicable scenarios for the CSSF optimization. No need to restrict such enhancement only to the SA scenarios.

- Proposal 1: The CSSF optimization is applicable to all CA, NR-DC, EN-DC and NE-DC scenarios.

- Exact solutions

- During last meeting, the discussion was held around three tentative solutions:

- Tentative solution 1: measure one serving CC per band

- Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

- Tentative solution 3: 3 searchers for CSSF

- Tentative solution 1 has been concluded in last meeting, no need further discussion. For Tentative solution 2, since the concern on the potential impact on PCell measurement, companies are a kind of conservative on it, so RAN4 decieded not to support Teneative solution 2. In such situation, only the Tentatvie solution 3 can be used to implement the demand of prioritizing some specific frequency layer by NW. So companies are still diverse on Tentative solution 3.

- From our perspective, since solution 2 is decided not to develop, if NW wants to prioritize any SCC frequency layer for inter-band case, which can only reply on Solution 3. So we hope Solution 3 can leave room for such NW demand.

- Proposal 2: For solution 3, the 3rd searcher resouce sharing is same as the 2nd searcher sharing by default. If one SCC is prioritized by NW indication, the 3rd searcher is exclusively used by the prioritized SCC.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

- Proposal 1: The CSSF optimization is applicable to all CA, NR-DC, EN-DC and NE-DC scenarios.

- Proposal 2: For solution 3, the 3rd searcher resouce sharing is same as the 2nd searcher sharing by default. If one SCC is prioritized by NW indication, the 3rd searcher is exclusively used by the prioritized SCC.

- Reference

- R4-2502592 “WF for NR_RRM_Ph5_Part1”, Apple

- RP-242380 “Revised WID: NR Radio Resource Management (RRM) Phase 5”, Apple, CATT