3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504259

Wuhan, China 7th – 11th April 2025

Source: 	Nokia

Title: 					NTN n254 3 MHz A-MPR proposal + other CBWs

Agenda Item: 			7.10.3.2

Document for:	Approval

# 1	Introduction

We provide our A-MPR proposal for 3 MHz channel bandwidth in n254 considering the additional emission requirements signaled through NS_03N, NS_04N and NS_05N. In particular, our proposal takes into account the averaging time limitation imposed on spectral analysis.

In addition, we present simulation results of channel bandwidths greater than 3 MHz. The simulation applied the 2 ms averaging time limit in spectral analysis as specified for the additional spurious emission limits. The results indicate that the currently specified A-MPR values are insufficient.

# 2	Simulation assumptions and scenario

### 2.1	Simulation parameters

- Operating band n254 (UL 1610 MHz – 1626.5 MHz)

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional emission requirements from NS_03N, NS_04N and NS_05N are simulated. Other gating factors (ACLR, general SEM, etc.) are excluded because they are handled by MPR.

- 2 ms averaging time limit applied in spectral analysis where specified (see [1][2])

# 3	A-MPR proposal for 3 MHz CBW

Here, we present our A-MPR proposal for 3 MHz CBW in n254 (NS_03N, NS_04N, NS_05N).

NS_03N and NS_04N

The proposed A-MPR is given in Table # and Table #. A-MPR is only needed when channel is up to 2 MHz away from lower edge of n254. The A-MPR values are somewhat greater than those listed in WF [3].

Table 1: A-MPR regions for NS_03N and NS_04N

| Channel BW | Carrier Center Frequency (MHz) | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) |
| --- | --- | --- | --- |
| 3 MHz | 1611.5 <= fc <= 1613.5 |  | >= 2.16 |

Table 2: A-MPR for NS_03N and NS_04N

|  | Modulation | A-MPR |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 |
|  | QPSK | 4.0 |
|  | 16QAM | 4.5 |
|  | 64QAM | 4.5 |
|  | 256QAM |  |
| CP-OFDM | QPSK | 5.0 |
|  | 16QAM | 5.0 |
|  | 64QAM | 5.0 |
|  | 256QAM |  |

Proposal 1: For a 3 MHz channel with NS_03N or NS_04N, specify the A-MPR according to Table 1 and Table 2.

NS_05N

Proposal 2: For a 3 MHz channel confined within 1618.25-1626.5 MHz, with NS_05N, specify no A-MPR.

# 4	Simulation results for 5, 10, 15 MHz CBWs

We also present results for higher CBWs considering the time limitation for the additional spurious emission masks.

### 4.1	NS_03N

At 5 MHz CBW A-MPR is not needed for Fc ≥ 1618.8 MHz. In the Appendix we show A-MPR triangles at positions relevant to the current specification. In some cases, the A-MPR exceeds significantly the values in the current specification. In Fig. 4 we present examples of the A-MPR triangles at the lower edge of the operating band.

Fig. 4: Examples of the A-MPR triangles at the lower edge of the operating band (Fc = 1612.5 MHz and Fc=1617.5 MHz, for 5 MHz and 15 MHz CBWs, respectively).

### 4.2	NS_04N

For NS_04N the A-MPR triangles are the same as for NS_03N 5 MHz CBW.

### 4.3	NS_05N

At 5 MHz CBW the relevant channel positions are so far from the time-limited mask segments that they don’t cause A-MPR. The A-MPR resulting from other mask segments is below the current A-MPR specification. In the Appendix we show the A-MPR triangles for 10 MHz and 15 MHz CBWs at positions relevant to the current specification. In some cases, the A-MPR exceeds significantly the values in the current specification. In Fig. 5 we present examples of the A-MPR triangles at the lower edge of the operating band.

Fig. 5: Examples of the A-MPR triangles at the lower edge of the operating band (Fc = 1615 MHz 
and Fc=1617.5 MHz, for 10 MHz and 15 MHz CBWs, respectively).

# 5	Conclusions

Observation 1: The 2 ms averaging time limit clearly affects the required A-MPR.

Proposal 1: For a 3 MHz channel with NS_03N or NS_04N, specify the A-MPR according to Table 1 and Table 2.

Proposal 2: For a 3 MHz channel confined within 1618.25-1626.5 MHz, with NS_05N, specify no A-MPR.

Proposal 3: The wider channel bandwidths should be resimulated applying the 2 ms averaging time limit.

# 6	References

[1]	R4-2419282, NTN 3 MHz A-MPR simulations, Nokia, RAN4#113

[2] R4-2501308, NTN n254 3 MHz A-MPR simulations, Nokia, RAN4#114

[3] R4-2502282, WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF, RAN4#114

# 7	Appendix: A-MPR triangles for 5, 10, 15 MHz CBW

### 7.1	NS_03N

### 7.2	NS_05N