3GPP TSG-RAN WG4 Meeting #114bis								R4-2504267

Wuhan, China, 7th – 11th April 2025

Source:			Ericsson

Title:	On Co-existence parameters for BS Tx spurious emission co-existence requirement limits

Agenda item:		7.13.4

Document for:		Approval

# 1	Introduction

LS from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence Tx spurious emission requirement levels for BS. It was acknowledged by RAN4 in the response LS [2].

The co-existence spurious emission requirement was originally derived in 1999 for UTRA. The technical background at that time was very much focused on the introduction of UTRA FDD/TDD operating at 2 GHz. The assumptions used as references to the original requirement definition can be found in a contribution [3] presented at RAN4#113 in Orlando. Over the years RAN4 have defined new operating bands below and above 2 GHz using the same requirement level. In current specification, bands are defined within the range 410 MHz up to 7125 MHz with the same requirement level, except few occasions (e.g. protection of GSM). Therefore, it is now time to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

The agreement is captured in a moderator summary as an outcome of the discussion related to WID update for Rel-19 BS RF evolution WI in RP-243288 [4].

The objective can be summarized as:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

An overview with background information and challenges was provided in our companion contribution [5] presented in RAN4#114.

In this contribution we present some discussion points in relation to transmitter co-existence spurious emission requirements and share some initial simulated coupling loss analysis.

# 2	Discussion

## 2.1	Requirement Background

In current BS RF specifications additional spurious emissions requirements have been defined to protect other networks operating in other bands in the same geographical area. These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the current RAN4 specifications whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.).

The extract from TS 38.104 Clause 6.6.5.2.3 states the following -

‘The spurious emission basic limits are provided in table 6.6.5.2.3 -1 for a BS where requirements for co-existence with the system listed in the first column apply.’

It is a protection requirement. The spurious emission limits in TS 38.104 define the maximum emissions that an aggressor BS is allowed to emit towards the concerned band stated in each row of Table 6.6.5.2.3-1 of TS 38.104.

The different requirement levels can be traced to different protection situations, as visualized in Figure 2.1-1.

Figure 2.1-1: Interference situations captured by the requirement

For both cases the requirement level is calculated based on a fixed assumption of Minimum Coupling Loss (MCL) for the following scenarios:

- BS-to-UE: Technical background information is traced back to a RAN4 contribution [6].

- BS-to-BS: Technical background information is traced back to a RAN4 contribution [7].

The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive antennas with 17 dBi antenna gain. With the introduction of AAS BS in frequencies up to 7125 MHz, the impact of beams pointing towards the UE, considerably larger antenna gain, the sub-array antenna structures, in addition to more relevant propagation models that are available today, should be also considered in the coupling loss evaluation. Also, at higher frequencies it can be seen that the AAS BS antenna gain is higher while the propagation condition is different compared to what originally was assumed at 2 GHz.

The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive antennas with 17 dBi antenna gain

With the introduction of AAS BS in frequencies up to 7125 MHz, the impact of beams pointing towards the UE and considerably larger antenna gain, sub-array antenna structures and relevant propagation models should be considered in the coupling loss evaluation.

The first preference should be to validate the existing spurious emission requirements with relevant AAS models for the different bands under considerations, and relevant propagation models

It can be noticed that different requirement levels apply to FDD and TDD based on different technical backgrounds [6] [7]. For bands in the upper region of FR1, the requirement allocation for FDD and TDD is listed in Table 2.1-1.

Table 2.1-1: Requirement allocation

| Duplex method | BS-to-BS (dBm/MHz) | BS-to-UE (dBm/MHz) |
| --- | --- | --- |
| FDD | -49 | -52 |
| TDD | -52 | -52 |

The requirement levels may differ, depending on the duplex method for BS-to-BS and BS-to-UE coupling.

### 2.1.1	Protection of UE receiver (BS-to-UE)

For this case the transmitter co-existence spurious emission requirement for BS is defined to protect the downlink of another network in the same geographical area. According to TR 25.942, subclause 5.1.4.1, the coupling loss between BS and UE was set to:

- MCL equal to 70 dB for the Macro-cellular environment,

- MCL equal to 53 dB for the Microcell environment.

However, the MCL for Microcell environment was adopted for requirement level derivation. The Microcell MCL was selected since it was seen as the worst-case MCL [6].

The assumed values are listed in Table 2.1.1-1.

Table 2.1.1-1: Original assumptions

| Parameter | Variable | Value |
| --- | --- | --- |
| UE receiver noise figure | F | 9 dB |
| BS-to-UE coupling loss | L | 53 dB |

The current requirement level is derived based on a link budget calculation in logarithmical scale:

Pem=kT+B+F+L

where kT is equal to -174 dBm/Hz and B is equal to 1 MHz. Assuming the following values for the UE noise figure, F equal to 9 dB and L equal to 53 dB, the maximum allowed spurious emission level, Pem at the BS transmitter can be calculated to -52 dBm/MHz.

It can be observed that MCL equal to 53 dB relevant for Microcell environment is used for requirement derivation of requirements for Wide Area BS in TR 25.942.

It can be noticed that the UE noise figure assumed for 2 GHz in TR 25.942 is not relevant for frequencies in the upper region of FR1.

### 2.1.2	Protection of BS receiver (BS-to-BS)

Traditionally the requirement level was determined by a static link budget calculation based on assumptions of BS noise figure and BS-to-BS coupling loss. For the BS-to-BS case the requirement levels were derived based on similar background as for co-location requirement level [7]. The assumed values are listed in Table 2.1.2-1.

Table 2.1.2-1: Original assumptions

| Parameter | Variable | Value | Note |
| --- | --- | --- | --- |
| BS receiver noise figure | F | 7 dB |  |
| BS-to-BS coupling loss | L | 67 dB | The current requirement level is derived based on an assumption of MCL between two base stations operating in different networks in the same geographical area of 67 dB (see TR 25.942, subclause 10.2.1). |
| Protection criteria | M | 9 dB | Assumed value corresponds to 0.5 dB degradation of victim receiver sensitivity. |

The current requirement level is derived based in logarithmical scale as:

Pem=kT+B+F+L-M

where kT is equal to -174 dBm/Hz, B is equal to 1 MHz and parameter values in Table 2.1.2-1. With the assumption the maximum allowed spurious emission level in another band is calculated to -49 dBm/MHz.

It can be observed that the requirement level for BS-to-BS co-existence spurious emission is based on the same link budget as co-location spurious emission but with a different assumption on coupling.

### 2.1.3	Reference case following the legacy approach from R99

To calibrate our simulator and validate the MCL methodology, we attempted to reproduce the same results following the legacy approach from R99 to derive the BS-to-UE emission requirement levels stated in Table 2.1-1.

Figure 2.1.3-1 Coupling Loss CDF from regression test to reproduce TR 25.942

The reference parameter values are highlighted in Table 2.1.3-1, extracted from TR 25.942.

Table 2.1.3-1 Reference parameter values from BS-to-UE regression test

| Parameter | BS-to-UE |
| --- | --- |
| Antenna loss | N/A |
| Aggressor antenna gain | BS antenna with 11 dBi gain (TR 25.942, subclause 5.1.4.1) |
| Propagation Loss | Macro cell propagation in TR 25.942, subclause 5.1.4.2 |
| Victim antenna gain | UE antenna with 0 dBi gain |
| Protection margin | 0 dB |
| Noise Figure | 9 dB |
| Bandwidth | 1 MHz |
| Thermal Noise | -74 dBm/ Hz |

## 2.2	Timeline to re-evaluate the spurious requirements and scope of co-existence work

In our companion contributions [8] [9], we shared a work-plan highlighting that the re-evaluation would include new aspects such as bands at higher frequencies and assumption of AAS BS models in the co-existence work. Moreover, the timeline of the work is limited to Release-19 schedule which leaves us with a couple of meetings to define re-evaluation methodology and agree on simulation parameters.

It is essential to define re-evaluation methodology and agree on simulation parameters at an early stage of the work, since the Release-19 time schedule is limited.

As highlighted in the previous section, during the work in TR 25.942, the concept of BS classes was not established. The current BS specification has different requirements established for Wide Area BS intended for Macro environments, Medium Range BS intended for Micro cell environments and Local Area BS intended for Indoor scenarios.

- RAN4 to differentiate the spurious emission requirements based on BS classes as per the current BS RF specification.

The bands in lower region of FR1 have duplexing mode as both TDD and FDD as summarized in the following section. Additionally, the simulation assumptions of the antenna models need to consider both AAS and Non-AAS models depending on the simulation frequency. With the variables stated before, it is important to balance the scope of co-existence work with the available time to conclude on this work item.

With the variables – High number of bands, Deployment scenarios, Duplexing mode, AAS/ Non-AAS antenna model assumptions, it is important to balance the scope of existence work with the available time to conclude on this work item.

- RAN4 to prioritize Macro deployment scenarios for the co-existence evaluation.

## 2.3	Frequency ranges for co-existence work

The WID emphasizes that from 410 MHz to 7125 MHz frequency ranges, TS 38.104 considers the same requirement levels, which marks the need to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

### 2.3.1	Aggressor

A survey for the bands between 410 MHz to 7125 MHz adds up to close to 50 bands considering both TDD and FDD duplex modes. Considering the efforts on the simulation work and timeline to complete the work item, the bands need to be grouped together based on relevant ranges, and for each range it is needed to identify a representative antenna model and transmission frequency.

It has been agreed to prioritize bands in the upper region of FR1 with bands above 5.9 GHz in WID. However, it is not clear up to what multiple of the fundamental frequency should be considered if they hit the victim frequency.

It is not clear upto what multiples of the fundamental frequency should be considered if they hit the victim frequency above 5.9 GHz.

Table 2.3.1-1: FR1 band upto 7125 MHz

|  | Range | Simulating frequency | Duplex mode |
| --- | --- | --- | --- |
| R1 | Less than 500 MHz | 500 MHz | FDD |
| R2 | 500 MHz – 1 GHz | 1 GHz | FDD |
| R3 | 1 GHz – 2 GHz | 2 GHz | FDD/ TDD |
| R4 | 2 GHz – 6 GHz | 3.5 GHz | FDD/ TDD |
| R5 | 6 GHz – 7.125 GHz | 6 GHz | TDD |

In Table 2.3.1-1, For R1 (less than 500 MHz), the multiples of the band hitting the victim frequency range are in order of 11th and above. For R2 (between 500 MHz – 1 GHz), the multiples of the band hitting the victim frequency are in order of 5th and above. For R5 (between 6 GHz – 7.125 GHz), this is not the focus on aggressor side as per WID.

- RAN4 to prioritize 2 GHz and 3.5 GHz as Aggressor simulation frequency for the co-existence evaluation.

### 2.3.2	Victim

A band survey was performed based on current BS specifications for E-UTRA, MSR and AAS and NR. All bands defined within FR1 above 5.9 GHz as per WID are listed in Table 2.3.2-1.

Table 2.3.2-1: FR1 band above 5.9 GHz

| Band number | Uplink (UL) operating band FUL,low – FUL,high (MHz) | Downlink (DL) operating band FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| n96 | 5925 to 7125 | 5925 to 7125 | TDD |
| n102 | 5925 to 6425 | 5925 to 6425 | TDD |
| n104 | 6425 to 7125 | 6425 to 7125 | TDD |

- RAN4 to prioritize 6 GHz as Victim simulation frequency for the co-existence evaluation.

## 2.4	Possible Methodologies

### 2.4.1	Evaluation metric - Minimum Coupling Loss vs Coupling Loss CDF

When the coupling for BS-to-UE was established, the worst case was based on the Micro network deployment scenario. Now with BS classes introduced in specifications it is not obvious that Micro network MCL is a relevant assumption for Wide Area BS intended for Macro deployment. To derive the MCL, we should derive the relevant CDF of the coupling loss between the aggressor BS and the victim UE or BS due to adaptive beamforming adjusting the beam patterns to maximize the reception and transmission of signals and derive its minimum value. This value, however, is at the queue of the CDF, which is very sensitive to the simulation and usually hard to coordinate in calibration among companies. As a result, we suggest discussing on the possible range of 95th-99th percentile of the CDF and to define then a “Coupling Loss” approach, instead of continuing focusing on the MCL.

- Define a Coupling Loss approach, similar to the legacy MCL approach, where we discuss the possible range on the 95th-99th percentile of the Coupling Loss CDF, instead of the minimum value.

### 2.4.2	BS-to-UE coupling

In the Figure 2.4.2.-1, we share an initial CDF plot for the BS-to-UE coupling loss case for Macro scenario, considering simulation assumptions discussed later in Section 3. Here the link is defined considering the beam points towards the served UE but the coupling loss is towards the victim UE.

For the BS-to-UE coupling, the link is defined considering the beam points towards the served UE, but the coupling loss is calculated towards the victim UE

Proposed detailed assumptions can be found in Section 3. The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 2 GHz

  - Antenna model – AAS with 2x8 array and 6x1 subarray antenna configuration

  - Gain at Tx – both array and beamforming gain

  - Propagation model – TR 38.901

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 6 GHz

  - Isotropic UE with 0 dBi gain and without beamforming

  - Propagation model – TR 38.901

When analyzing the coupling loss, one should also decide what component of the array and beamforming gain (in case of AAS) are considered at Tx, to model the gain at which the spurious emission is transmitted from the aggressor BS. In case of non-AAS at the aggressor BS, only the array gain would be considered. In case of AAS, there is the option to consider the array gain or/ and the beamforming gain.

- RAN4 to consider the following in BS-to-UE coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain.

Figure 2.4.2-1 BS-to-UE Coupling Loss CDF with 2 GHz AAS BS towards 6 GHz isotropic UE for Macro scenario.

In general, when statistical coupling losses in terms of CDF curve are considered, the tail of the curve is very sensitive to the simulation assumptions and results. So, it’s important to agree on a percentile within RAN4, which is easier to calibrate between the companies (possibly in the range of 95%ile to 99%ile).

### 2.4.3	BS-to-BS coupling

For BS-to-BS coupling loss case when Macro scenario is concerned, range of 50% - 100% grid shifts between two base stations operating in different networks in the same geographical area is a reasonable assumption. Co-location related work in terms of coupling loss is handled in separate discussion in RAN4.

- Considering the requirement levels must be defined based on co-existence evaluation and not co-location, range of 50% - 100% grid shift is a reasonable assumption for this work.

Other proposed detailed simulation assumptions for BS-to-BS coupling loss analysis can be found in Section 3.

When analyzing the coupling loss, one should also decide what component of the array and beamforming gain (in case of AAS) are considered at Tx, to model the gain at which the spurious emission is transmitted from the aggressor BS. In case of non-AAS at the aggressor BS, only the array gain would be considered. In case of AAS, there is the option to consider the array gain or/ and the beamforming gain.

- RAN4 to consider the following in BS-to-BS coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain.

In general, when statistical coupling losses in terms of CDF curve are considered, the tail of the curve is very sensitive to the simulation assumptions and results. So, it’s important to agree on a percentile within RAN4, which is easier to calibrate between the companies (possibly in the range of 95%ile to 99%ile).

# 3	Simulation assumptions

## 3.1	Co-existence simulation scenarios

Table 3.1-1 summarizes the initial proposed simulation scenarios.

Table 3.1-1: Summary of simulation scenarios

| No. | Aggressor Simulating frequency | Victim Simulating frequency | Direction | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 GHz/ 3.5 GHz | 6 GHz | BS DL to UE UL | Urban macro | First priority |
| 2 | 2 GHz/ 3.5 GHz | 6 GHz | BS DL to BS UL | Urban macro | First priority |
| 3 | 2 GHz/ 3.5 GHz | 6 GHz | BS DL to UE UL | Micro | Second priority |
| 4 | 2 GHz/ 3.5 GHz | 6 GHz | BS DL to BS UL | Micro | Second priority |
| Note: RAN4 to prioritize Macro deployments. |  |  |  |  |  |

## 3.2	Network layout model

### 3.2.1	Urban macro

Details on urban macro network layout model are listed in Table 3.2.1-1 and 3.2.1-2.

Table 3.2.1-1 : Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 450 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor |  |
|  | Indoor UE ratio | 0% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 3.2.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | Un-coordinated operation (50-100%% grid shift) |  |

Figure 6.1.2.1.1-1: Uncoordinated operation

## 3.2	Propagation Model

### 3.2.1	Pathloss

The pathloss models are summarized in Table 7.4.1-1 from TR 38.901.

### 3.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1. from TR 38.901.

### 3.2.3	O-to-I penetration loss

The pathloss incorporating O2I building penetration loss is described in Clause 7.4.3.1 from TR 38.901.

## 3.3	Antenna configuration

### 3.3.1	AAS Macro BS antenna modelling

Table 3.3.1-1 : AAS Macro BS antenna configuration

|  | AAS Urban Macro |  |  |
| --- | --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) | 6 GHz (Victim) |
| Parameter |  |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 2x8 | 4x8 | 8x16 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 6x1 | 3x1 | 3x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 28.1 | 28.25 | 22.2 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 | 6 |

### 3.3.2	Non-AAS Macro BS antenna modelling

Table 3.3.2-1 : Non-AAS Macro BS antenna configuration

|  | Non- AAS Urban Macro |  |
| --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) |
| Parameter |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 10x1 | 10x1 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1x1 | 1x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | N/A |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 38 | 38 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 |

### 3.3.3	UE antenna modelling

The UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming

## 3.4	Other simulation parameters

Table 3.4-1: Other simulation parameters (Common for all simulation frequency)

| Parameters | Urban macro |
| --- | --- |
| Channel bandwidth | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | 100 MHz |
| Scheduled channel bandwidth per UE (UL) | 100 MHz |
| The number of active UE (DL) | Same as the number of BS beam |
| The number of active UE (UL) | 1 UE per slot |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm (2 polarizations) | 46dBm |
| UE max TX power in dBm | 23dBm |
| UE min TX power in dBm | -40dBm |
| BS Noise figure in dB | 8 |
| UE Noise figure in dB | 9 |
| Handover margin | 3dB |

- RAN4 to consider the simulation assumptions in this clause and agree to use them as reference for the simulation evaluation work.

# Conclusion

In the previous sections we made the following observations:

Observation 1	It is a protection requirement. The spurious emission limits in TS 38.104 define the maximum emissions that an aggressor BS is allowed to emit towards the concerned band stated in each row of Table 6.6.5.2.3-1 of TS 38.104.

Observation 2	The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive antennas with 17 dBi antenna gain

Observation 3	With the introduction of AAS BS in frequencies up to 7125 MHz, the impact of beams pointing towards the UE and considerably larger antenna gain, sub-array antenna structures and relevant propagation models should be considered in the coupling loss evaluation.

Observation 4	The first preference should be to validate the existing spurious emission requirements with relevant AAS models for the different bands under considerations, and relevant propagation models

Observation 5	The requirement levels may differ, depending on the duplex method for BS-to-BS and BS-to-UE coupling.

Observation 6	It can be observed that MCL equal to 53 dB relevant for Microcell environment is used for requirement derivation of requirements for Wide Area BS in TR 25.942.

Observation 7	It can be noticed that the UE noise figure assumed for 2 GHz in TR 25.942 is not relevant for frequencies in the upper region of FR1.

Observation 8	It can be observed that the requirement level for BS-to-BS co-existence spurious emission is based on the same link budget as co-location spurious emission but with a different assumption on coupling.

Observation 9	It is essential to define re-evaluation methodology and agree on simulation parameters at an early stage of the work, since the Release-19 time schedule is limited.

Observation 10	With the variables – High number of bands, Deployment scenarios, Duplexing mode, AAS/ Non-AAS antenna model assumptions, it is important to balance the scope of existence work with the available time to conclude on this work item.

Observation 11	It is not clear upto what multiples of the fundamental frequency should be considered if they hit the victim frequency above 5.9 GHz.

Observation 12	For the BS-to-UE coupling, the link is defined considering the beam points towards the served UE, but the coupling loss is calculated towards the victim UE

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to differentiate the spurious emission requirements based on BS classes as per the current BS RF specification.

Proposal 2	RAN4 to prioritize Macro deployment scenarios for the co-existence evaluation.

Proposal 3	RAN4 to prioritize 2 GHz and 3.5 GHz as Aggressor simulation frequency for the co-existence evaluation.

Proposal 4	RAN4 to prioritize 6 GHz as Victim simulation frequency for the co-existence evaluation.

Proposal 5	Define a Coupling Loss approach, similar to the legacy MCL approach, where we discuss the possible range on the 95th-99th percentile of the Coupling Loss CDF, instead of the minimum value.

Proposal 6	RAN4 to consider the following in BS-to-UE coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain.

Proposal 7	Considering the requirement levels must be defined based on co-existence evaluation and not co-location, range of 50% - 100% grid shift is a reasonable assumption for this work.

Proposal 8	RAN4 to consider the following in BS-to-BS coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain.

Proposal 9	RAN4 to consider the simulation assumptions in this clause and agree to use them as reference for the simulation evaluation work.

# References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[3]	  R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

[4]	RP-243288, “Moderator's summary of discussion about R19-RAN4-BSRF”, Huawei

[5]	R4-2500880, “On the topic of BS RF specification structure improvements”, Ericsson

[6]	R4-99a01, “BS TDD Spurious Emission Requirements for Co-Existence UTRA-FDD/ UTRA-TDD”, SIEMENS

[7]	R4-030641, “General corrections on co-existence and co-location requirements for UTRA-FDD BS”, Nortel Networks, Ericsson, Siemens, Motorola

[8]	R4-2500881, “General aspects related to BS RF evolution WI”, Ericsson

[9]	R4-2503726, “Rel-19 BS RF evolution overview”, Ericsson