3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504291

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              General discussion on XR RRM requirement

Agenda item:			7.28.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some extensively discussions on XR. In this meeting, we will give some further discussions for some open issues captured in the agreed WF especially semi-static schemes [1]. The following working assumption was confirmed in RAN1 in last meeting.

2 Discussion Agreement

Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Observation 1: Only the following working assumption was confirmed in RAN1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements

2 Discussion

Issue 1-1 Deployment scenarios

The following agreements were achieved in last meeting

< Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC

From our understanding, DC scenario is more complex due to the consideration of two independent PHY/MAC schedulers and two independent measurement object configurations. And from RAN1 scope perspective, the whole Rel-19 WI on MG skipping for XR is a minor optimization, they did not even study whether XR requirements can be met with DC, in this sense, there is no need to bring more complexity into the RAN4 discussions.

Proposal 1: Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

Issue 3-2/3-3: Formulas for measurement delay extension/ Conditions for measurement skipping

The following agreements were achieved in last meeting

Issue 3-2 Formulas for measurement delay extension <Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. Issue 3-3 Conditions for measurement skipping <Way forward>: - Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

From our understanding, in the NR-U scenario, the UE is not expected the regular reception of RSs due to the uncertainty of the outcome LBT procedure. From this, the RRM measurement and reporting will be impacted by the missed measurement opportunities of RSs due to LBT failure. While, to maintain the same accuracy, the UE still needs to obtain the same/sufficient number of measurement samples when performing measurement. Accordingly, the measurement period is extended based on those added samples/measurement opportunities. Follow this logic, seems option 2 is more reasonable, since it is +Nskip rather than Nskip ×, i.e., scaling fator. But more details: e.g., 1) The maximum number of skipped MG occasions in the measurement period; 2) CSSFinter should be discussed further

Proposal 2: Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

- FFS: The maximum number of skipped MG occasions in the measurement period to limit Nskip

- FFS: CSSFinter

From our understanding, for measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified since too relaxed delay requirements will bring negative impact on UE connectivity and/or mobility, the QoS performance degradation issue may occur

Proposal 3: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified

Issue 6-1 semi-static schemes

The following agreements were achieved in last meeting

<Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

From our understanding, DCI solution is enough, no need to consider extra solutions including RRC and/or MAC-CE.

- RRC is not needed. Periodic pattern is configured based on long-term XR traffic status, while in practice, the XR traffic is likely to vary over time, and the MG occasion skipping is somewhat like “one-shot” pattern, and considering the unpredictable, quasi-periodic arrival and jitter characteristics of the XR data, limited and stable pre-defined MG occasion skipping pattern can not suit all the PDB. In this sense, RRC pattern-based become meaningless. And RRC also has an additional relatively long signalling processing time compared with DCI indication. And there is also ambiguity on the exact time for applicability.

- No advantage is foreseen for MAC-CE. Since DCI has been agreed so there is a dynamic signalling based solution. Such redundant introduction of MAC-CE bitmap for skipping indication will increase unnecessary spec workload and UE complexity.

- Some companies have concerns that although 1 bit dynamic indication is very suitable for the XR data from NW side, it is hard to determine the measurement period/delay requirement from UE side. However, from our understanding, the same issue was happned in NR-U scenario, the same principle can be considered to define the requirements, that is issue 3-2, accordingly, it is more appropriate to put more efforts on solving such issue.

Proposal 4: Do not consider semi-static solutions

3 Conclusion

In this contribution, we provided our viewpoints on XR requirement related issues. The following observations and proposals are obtained:

Proposal 1: Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

Proposal 2: Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

- FFS: The maximum number of skipped MG occasions in the measurement period to limit Nskip

- FFS: CSSFinter

Proposal 3: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified

Proposal 4: Do not consider semi-static solutions

4 References

[1] R4-2502700 “WF on RRM requirements for NR_XR_Ph3”, Nokia