3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504299

Wuhan, China, 7 – 11 April, 2025

Source: 	Huawei

Title: 	OTA test case reduction

Agenda Item:	7.13.5.2

Document for:	Approval

1 Introduction

New WID on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at RAN#103 meeting. The work item concentrates on 2 work areas where the current BS specification may be optimised, i.e. Expected EIRP mask for upper 6GHz and OTA test enhancement. One of major identified RF requirements with the aim to shorten the test time is TX IM.

The considerations on test case reduction for TX IM test reduction was discussed in RAN4#113 [1] and the WF is captured in [2]. In this contribution we provide further discussion and proposals for the objective.

2 Test scope reduction for Tx requirement

At RAN4#113 meeting, for TX test case reduction it is agreed to focus on Tx IMD requirement and some agreements on the applicable interferer offsets and test configurations.

Issue 1-1-1: test scope reduction for Tx requirement

- Agreement:

  - Focus on test scope reduction for Tx IMD requirement

  - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements;

    - 1ST interference freq offset for ACLR;

    - 3rd interference freq offset for spurious emission;

    - TBD interference freq offset for OBUE requirement

Issue 1-1-2: Reduction of test configuration for Tx requirement

- Agreement:

  - Proposal 1: for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.

  - FFS on the maximum output power for the conformance testing.

2.1	Interferer offsets

6 interferer offsets are tested for each and 12 if AAS supports NC operation, these offsets date back to the UTRA requirements and have been carried through for each subsequent RAT however not they are generating such excessive test time so it is appropriate to see if they are all still necessary or can be reduced.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

At last meeting, the applicable interference freq offset for ACLR and spurious emission requirements is agreed, and it is FFS for OBUE.

OBUE

LTE/NR OBUE is generally considered that in the 1st adjacent channel the ALCR requirement dominates and as the emission moves away from the carrier the OBUE is tougher. They have a tougher requirement beyond 10 MHz. As we are testing at M only, the band filter can be ignored. Hence 3xCBW case is probably tougher.

Proposal 1: it is proposed to use 3xCBW offset to check OBUE limits.

The following update is made for the test requirements.

Table 6.8.5.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | For OTA ACLR requirements in clause 6.7.3, n=1. For OTA OBUE in clause 6.7.4 and OTA spurious emission in clause 6.7.5, n=3. |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

3 Out of band blocking

At RAN4#114 meeting, for RX test case reduction it is agreed to focus on out of band blocking and focus on the measurement step size for Rx OOBB requirement.

Issue 1-2-1: Out of band blocking

- Agreement:

  - To increase the step size for OOBB conformance testing for FR1 BS

  - FFS on the step size for OOBB conformance testing

RAN4 had similar discussion for FR2 in Rel-15 and finally agreed to reduce the amount of time by increasing the interferer step size. The less than 1/3 BW approach is adopted when define the measurement step.

Table 7.6.4.2.3-1: Interferer signal step size for FR2-1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

For FR1 similar approach can be considered. We propose to set the measurement step size according to the supported BS channel bandwidth declared by the manufacture.

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

Proposal 2: It is proposed to set the measurement step size according to the minimum supported BS channel bandwidth declared by the manufacture, as shown in Table 3-1.

4	Summary

Proposal 1: it is proposed to use 3xCBW offset to check OBUE limits.

Table 6.8.5.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | For OTA ACLR requirements in clause 6.7.3, n=1. For OTA OBUE in clause 6.7.4 and OTA spurious emission in clause 6.7.5, n=3. |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

Proposal 2: It is proposed to set the measurement step size according to the minimum supported BS channel bandwidth declared by the manufacture, as shown in Table 3-1

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

3	Reference

[1] R4-2419243, Test case reduction for TX IM, Huawei

[2] R4-2419828, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[3] R4-2501854, OTA test case reduction, Huawei

[4] R4-2502375, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation