3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504303

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	RF requirements for CW

Agenda Item:	7.24.2.3

Document for:	Approval

1 Introduction

At last RAN4#114 meeting, the CW RF requirements have been discussed and agreements can be found in [2].

In this paper we provide further discussion and proposals for open issues.

2 Discussion

Per Study Item discussion, output power, frequency error, unwanted emission including spurious emission are potentially needed. Some progress was made at RAN4#114, the progress is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not (outcome of SI) | WF in R4-2502859 at RAN4#114 |
| --- | --- | --- |
| Output power | YES | CW output power [33] dBm |
| ON/OFF power | TBD in WI phase | FFS for OFF power |
| Frequency error | YES | Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW |
| Phase noise | No for inside topology TBD in WI phase for outside topology | Further discuss phase noise requirements |
| Unwanted emission | YES | Specify emission requirements, values FFS |
| Spurious emissions | YES | NR spurious emission requirement can be considered as baseline for R19 AIoT CW node |
| Intermodulation | TBD in WI phase | FFS |

Output power

The CW maximum output power is [33] dBm from previous discussion.

In the release, CW node is deployed separately from the base station reader, using UL spectrum. Different to that for the UE different power classes are defined based on UE capability, single out power limit is preferred for the CW node. From TS 45.005, we can find that up to 33 dBm is supported for GSM 700, GSM 850 and GSM 900. And for RFID, the max ERP limit is 33 dBm for 800M and 36 dBm for 900M (ETSI EN 302 208). From the competition to RFID and flexibility to various deployment scenarios perspective, it is proposed to remove the [] and define the maximum output power of the CW node as 33 dBm.

Table 4.1-2 in 45.005 MS maximum power at other modulations

| Power | GSM 400 and GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 | GSM 400 & GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 |  | DCS 1 800 | PCS 1 900 | DCS 1 800 & PCS 1 900 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class | Nominal Maximum output | Tolerance (dB) for conditions |  | Nominal Maximum output | Nominal Maximum output | Tolerance (dB) for conditions |  |
|  | Power | normal | extreme | power | power | normal | extreme |
| E1 | 33 dBm | ±2 | ±2,5 | 30 dBm | 30 dBm | ±2 | ±2,5 |
| E2 | 27 dBm | ±3 | ±4 | 26 dBm | 26 dBm | -4/+3 | -4,5/+4 |
| E3 | 23 dBm | 3 | ±4 | 22 dBm | 22 dBm | 3 | 4 |
| E4 | 19 dBm1) | 2 | ±2,5 | 18 dBm | 18 dBm | 3 | ±3,5 |
| NOTE 1: Power class E4 applies only to GSM 850 and E-GSM 900. |  |  |  |  |  |  |  |

Proposal 1: The CW maximum output power is 33 dBm.

Phase noise

Based on the technical analysis on Phase Noise requirements in SI phase, for inside topology, no explicit phase noise requirement is required. Rationale behind is that the internal CW signal enables convenient single tone signal and phase noise cancellation through calibration mechanisms, eliminating the need for stringent phase noise requirements. And for outside topology, phase noise requirements become solution-dependent and must align with system implementation. For example, if direct RF calibration cable exist between the CW node and Base Station node for real-time cancellation, phase noise impacts on uplink demodulation are significantly mitigated, which is the same situation as the case for inside topology. In such an implementation, there is no need to define phase noise requirements. In our view, phase noise should be treated as an internal requirement within solution architectures rather than being standardized in 3GPP. This approach maintains implementation flexibility while ensuring system-level performance objectives.

Figure 2-1: example block diagram of the A-IoT indoor coverage solution

Proposal 2: phase noise requirement is solution-dependent and do not define phase noise requirement for CW node.

Unwanted emission

CW node transmits singe tone signal. Except the spurs in the discrete spectrum, the emission in the frequency range immediately outside the CW is phase noise dominate. Table 2.2-1 show one example phase noise performance with carrier frequency 900 MHz. Assuming the CW occupies one PRB, the integrate noise phase at the adjacent PRB is less than -25 dBm/30 kHz, which is already close to the GSM MS emission level (-27 dBm/30 kHz) at 400 kHz offset.

Table 2.2-1: example phase noise performance @ 900 MHz

| Offset | Phase Noise |
| --- | --- |
| 300Hz | -78.5dBc/Hz |
| 500Hz | -83.2dBc/Hz |
| 1kHz | -91dBc/Hz |
| 10kHz | -101dBc/Hz |
| 100kHz | -103dBc/Hz |
| 1MHz | -115dBc/Hz |

Table 2.2-2: GSM emission mask

Table 2.2-3: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

For narrow band signal in sub-1 GHz, NB-IOT and GSM share similar case, and can be used as a reference. The mask of NB-IoT and GSM spectrum emission mask are shown above Table 2.2-2 and Table 2.2-3. The two masks are almost the same in the close end and a little adjust for the far end. And for Local Oscillator, there will be some spurs appear caused by either leakage or mismatch of the charge pump. One example is shown in below figure [3]. The typically value of spur is around 60 dBc below the reference output. Considering this implementation restriction, the low limit of emission can be set to 33-60 =-27 dBm.

Assuming one PRB resource is reserved for CW, the emission mask starts from adjacent PRB. The Δf (foffset) is defined from the center of the CW transmission, the spectrum emission mask for CW node is as defined in Table 2.2-4. Equivalently, the ACLR for 1st adjacent PRB is 40 dBc, 2nd adjacent PRB is 45 dBc and others is 52 dBc. Alternatively defining it using ACLR approach with similar values is also ok.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

Proposal 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Spurious emissions

Referring to boundary definition of BS, the spectrum emission mask for CW are defined from 10 MHz below the lowest frequency of the supported uplink operating band up to 10 MHz above the highest frequency of the supported uplink operating band.

Proposal 4：The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as F = 10 MHz

At last meeting, it was agreed that NR spurious emission requirement can be considered as baseline for R19 AIoT CW node. If proposal 3 is agreed, the spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of the supported uplink operating band, up to 10 MHz above the highest frequency of the supported uplink operating band. Spurious emission limits are given in ITU recommendation ITU-R M.1581 and are captured in Table 2.2-5.

Table 2.2-5: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |

Proposal 5: Spurious emissions limits is defined as in Table 2.2-5

Intermodulation

And for intermodulation, since CW node transmits singe tone signal in the Release, the requirement is not needed.

Proposal 6: Intermodulation are not needed for CW node

3 Conclusion

In this paper we provide discussion and proposals for CW RF requirements.

Proposal 1: The CW maximum output power is 33 dBm.

Proposal 2: phase noise requirement is solution-dependent and do not define phase noise requirement for CW node.

Proposal 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

Proposal 4：The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as F = 10 MHz.

Proposal 5: Spurious emissions limits is defined as in Table 2.2-5.

Proposal 6: Intermodulation are not needed for CW node.

References

[1]	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]	R4-2502859, WF on requirements for A-IoT BS and CW, Moderator (Huawei)

[3] PLL Performance, Simulation, and Design