3GPP TSG-RAN WG4 Meeting #114bis	  R4-2504317

Wuhan, China, April 7 – 11, 2025

Agenda Item:	7.19.2

Source:	Huawei, HiSilicon

Title:	Discussion on CSI reporting requirement framework for AIML CSI prediction

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #114 [1], we discuss the remaining issues related to the RRM core requirement for AIML positioning.

- Testing setup

RAN4 #113bis [2]: Channel model for CSI prediction tests For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. RAN4 #114 Baseline Simulation assumptions for AIML based CSI prediction [3]: Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction Parameter; Value. Duplex, Waveform; FDD, TDD, OFDM. Carrier frequency; 2GHz for FDD 4GHz for TDD. Bandwidth; 10MHz for FDD, 40MHz for TDD. Symbol; 1 symbol. Number of subbands; 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD. Subcarrier spacing; 15kHz for 2GHz, 30kHz for 4GHz. gNB TX antennas; Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements. UE RX antennas; 4. Channel model; TDLA30 as baseline with XP medium correlation. Doppler spread; 20Hz, 50Hz and 100Hz. Delay spread; 30ns. Channel estimation; Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration). Rank per UE; Rank 1 and 2. CSI feedback assumption; Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements. CSI feedback overhead; Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later. Baseline; Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements). KPI; Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback. Model input type; Raw channel matrix. Model out type; Raw channel matrix. Observation window; Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms. Prediction window; Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms.

It is observed that in RAN4 legacy, TDL channel models are used for PMI reporting testing. Spatial channel model for demodulation performance requirements is still under study in RAN4. It is suggested that RAN4 will not consider CDL channel model in AI CSI prediction.

Proposal 1: RAN4 will not consider CDL channel model in AI CSI prediction.

For generalization test, there are two options to verify the performance of AI CSI prediction. One is to define several requirements under different test configurations/conditions. If the DUT can pass all the tests, then it means that the performance of CSI prediction can be guaranteed in a generalized scenario consisting of these configurations/conditions. This method is what RAN4 legacy uses.

RAN4 #113 WF: Testing conditions Use static conditions as baseline. - FFS on whether to have non-static condition

Another is that the test configurations/conditions vary over time during testing. This method has never been used in RAN4 before. Firstly, the intention is not clear to introduce more complicated testing compared legacy for generalized performance verification. Secondly, when model switching is involved under non-static condition to ensure the performance, LCM procedure testing and generalized performance testing are coupled together in one testing.

Proposal 2: For generalization testing, there is no need to use non-static condition.

For LCM procedure, if model/performance monitoring and management are performed by NW, there is no need to introduce non-static condition for UE testing either. When to activate/deactivate AI is up to NW implementation. For UE-side monitoring, two types are on the table in RAN1. One is to report performance metric, another is to report performance monitoring output. The former one needs to define the performance metric and corresponding report mechanism. The latter one needs to define the performance metric, threshold criterion and corresponding report mechanism. The performance metric and threshold definition may be subject to configurations/conditions, which has not been discussed yet in other WGs.

Proposal 3: RAN4 will start to discuss whether/how to define non-static condition during testing for UE-side monitoring after other WGs achieve sufficient progress.

- Simulation results

According to the baseline simulation assumptions for AI CSI prediction, we provide the simulation results in the following.

Proposal 4: SGCS performance results of AI CSI prediction.

Table 1. SGCS performance results of AI CSI prediction

|  | 20Hz | 50Hz | 100Hz |
| --- | --- | --- | --- |
| Rank 1 | 0.9985413 | 0.8673333 | 0.301114 |
| Rank 2 | 0.99800384 | 0.8166052 | 0.233405 |

Note: gNB TX antennas is Option 2.

It is observed that acceptable SGCS performance can be achieved with 20Hz Doppler spread. There is a degradation in SGCS performance at 50Hz. At 100Hz, there is an extremely significant degradation in SGCS performance.

Observation 1: Significant degradation in SGCS performance can be observed with 100Hz Doppler spread.

Proposal 5: RAN4 will not define performance requirement for AI CSI prediction with 100Hz Doppler spread.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 will not consider CDL channel model in AI CSI prediction.

Proposal 2: For generalization testing, there is no need to use non-static condition.

Proposal 3: RAN4 will start to discuss whether/how to define non-static condition during testing for UE-side monitoring after other WGs achieve sufficient progress.

Proposal 4: SGCS performance results of AI CSI prediction.

Table 1. SGCS performance results of AI CSI prediction

|  | 20Hz | 50Hz | 100Hz |
| --- | --- | --- | --- |
| Rank 1 | 0.9985413 | 0.8673333 | 0.301114 |
| Rank 2 | 0.99800384 | 0.8166052 | 0.233405 |

Note: gNB TX antennas is Option 2.

Observation 1: Significant degradation in SGCS performance can be observed with 100Hz Doppler spread.

Proposal 5: RAN4 will not define performance requirement for AI CSI prediction with 100Hz Doppler spread.

References

- R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP RAN4 #114, Athens, Greece, 17th – 21st February, 2025.

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.

- R4-2502970, “Baseline Simulation assumptions for AIML based CSI prediction”, 3GPP RAN4 #114, Athens, Greece, 17th – 21st February, 2025.