3GPP TSG-RAN WG4 Meeting #114bis	  R4-2504318

Wuhan, China, April 7 – 11, 2025

Agenda Item:	7.19.3

Source:	Huawei, HiSilicon

Title:	Discussion on RRM core requirement and testing framework for AIML beam management

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #114 [1], we discuss the remaining issues related to AIML beam management.

- Test setup for AI-BM

RAN4 #113 WF: Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. RAN4 #112 WF: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

For AI BM Case 1 and Case 2, measured RSRP is the model input. Both AoA and AoD are not considered as the model input. Therefore, in RAN4, only the RSRP related to different beam needs to be emulated.

Observation 1: AoA and AoD are not used as model input in AI BM Case 1 and Case 2.

Proposal 1: Use single AoA as the channel model for AI BM testing.

RAN4 #114 WF: Issue 2-9: Test setup requirements - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model - Use CDL-based channel model as starting point

For CDL-based channel model, the determination of Set A should take TE capability into account, including the power range, the maximum number of beams and the number of probes. Currently, RAN4 agreed to start the testability study with Set A = 32 and Set B = 8.

- Measurement error impact on prediction accuracy

According to the WF and the updated simulation assumptions in [2], companies are encouraged to investigate whether and how the measurement error will impact the RSRP prediction accuracy with system-level simulation assumptions. The measurement error modeling is generated by link-level simulations.

3.1 Measurement error modeling

In the following, we provide the relationship between SNR and measurement error under Gaussian distribution assumption.

Figure 1. Relationship between SNR and baseband measurement error

Observation 2: With Gaussian distribution assumption of baseband measurement error, under different SNRs, the mean values are all close to 0, while the variance increases as the SNR drops.

Based on the generated error distribution, the error is sampled and then added for each individual Tx-Rx beam pair according to the SNR. For RF error, we employ the assumption described in [2], i.e., truncated Gaussian distribution under ±4dB (μ=0,  σ=2), regardless of the SNR.

3.2 Simulation results

In [2], two categories of KPI are considered. The first one is related to beam index prediction, while the second on is related to RSRP prediction.

Updated simulation assumptions for beam prediction [2] KPI 1: - Top K/1 without considering some margin - the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", K=1,2,3,4,5 - Top K/1 with considering some margin; The margin can be implemented as: - Alt 1: Extend Top K/1 to Top K/N - the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams", K/N= 2/2, 2/4, 3/2, 3/4, 4/4, 5/2; - the percentage of "the Top-N genie-aided beams are two of the Top-K predicted beams", K/N= 2/2, 2/4, 3/2, 3/4, 4/4, 5/2, which means among the Top-K predicted beams, there are two beams are parts of top-N genie aided beams. - Alt 2: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB KPI 2: - 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam - 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams Note: 90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

In the following, we provide the simulation results regarding to KPI 1, i.e., Top K/1 without considering some margin and with considering some margin.

Proposal 2: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 1.

Table 1. Measurement error impact on prediction accuracy regarding to KPI 1

| KPI-1 |  | Model input w/o measurement error | Model input w measurement error |
| --- | --- | --- | --- |
| Top K/1 w/o margin | Top_1_1 | 64.42% | 42.46% |
|  | Top_2_1 | 86.69% | 65.21% |
|  | Top_3_1 | 93.00% | 76.97% |
|  | Top_4_1 | 95.43% | 83.59% |
|  | Top_5_1 | 96.75% | 87.78% |
| Top-N genie-aided beam is one of the Top-K predicted beams | Top_2_2_1 | 94.58% | 79.33% |
|  | Top_2_4_1 | 98.60% | 92.10% |
|  | Top_3_2_1 | 96.81% | 85.99% |
|  | Top_3_4_1 | 99.44% | 95.64% |
|  | Top_4_4_1 | 99.75% | 97.57% |
|  | Top_5_2_1 | 98.60% | 93.45% |
| Top-N genie-aided beam is two of the Top-K predicted beams | Top_2_2_2 | 67.09% | 42.35% |
|  | Top_2_4_2 | 92.64% | 74.61% |
|  | Top_3_2_2 | 86.39% | 63.95% |
|  | Top_3_4_2 | 96.70% | 84.85% |
|  | Top_4_4_2 | 98.31% | 90.88% |
|  | Top_5_2_2 | 94.20% | 80.73% |
| maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB | x = 1dB | 83.02% | 60.75% |
|  | x = 2dB | 92.49% | 72.87% |
|  | x = 3dB | 96.29% | 81.84% |

From Table 1, we observe that measurement error impacts prediction accuracy, and the more stringent the KPIs are, the greater the impact.

Observation 3: The more stringent the KPIs are, the greater the impact of the measurement error on the prediction accuracy will be.

In the following, we provide the simulation results regarding to KPI 2, i.e., 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam/the ideal L1-RSRP of the same beams.

Proposal 3: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 2.

Table 2. Measurement error impact on prediction accuracy regarding to KPI 2

| KPI-2 |  |  | Model input w/o measurement error | Model input w measurement error |
| --- | --- | --- | --- | --- |
| 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam | Avg. | CDF_strong_1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_3 | 2.345dB | 5.392dB |
|  |  | CDF_strong_5 | 2.209dB | 4.893dB |
|  | w/o avg. | CDF_strong_Top1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_Top2 | 2.380dB | 5.492dB |
|  |  | CDF_strong_Top3 | 2.139dB | 4.781dB |
|  |  | CDF_strong_Top4 | 2.049dB | 4.445dB |
|  |  | CDF_strong_Top5 | 1.967dB | 4.044dB |
| 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams | Avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top1&2&3 | 2.236dB | 7.406dB |
|  |  | CDF_same_Top1&2&3&4&5 | 2.419dB | 7.444dB |
|  | w/o avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top2 | 2.200dB | 7.314dB |
|  |  | CDF_same_Top3 | 2.435dB | 7.702dB |
|  |  | CDF_same_Top4 | 2.674dB | 7.730dB |
|  |  | CDF_same_Top5 | 2.689dB | 7.287dB |

Besides reaching the same observations as under KPI 1, we note that regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam.

Observation 4: Regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam.

Proposal 4: RAN4 define requirements with measurement error as model input.

Proposal 5: RAN4 will not select KPI under which acceptable performance cannot be achieved, including

- the KPI under which the prediction accuracy is less than 80%, e.g, Top K/1 with K<4 without RSRP margin.

- the KPI under which the 90%-tile L1-RSRP difference is larger than 5dB, e.g, maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB with x<3dB.

- Training and testing consistency

It is observed that the performance of the model may also be impacted by other factors, including the gNB-side beampattern (which is gNB-side proprietary information), the channel conditions (e.g., the scatter in the physical environment), the receiving gain at UE (i.e., different UE implementation, UE rotation). Therefore, whether and how to define a common training/testing dataset is an open issue.

If the training dataset is not specified, how to ensure the consistency between training and testing needs to be studied. One possible solution is to let the DUT to collect training dataset and complete the model training, followed by performing model test. However, according to R18 study outcome, the training procedure is not considered in RAN4 test due to many reasons, such as test cost/time.

Even if RAN4 revisits the training procedure, how to ensure the following principle identified in R18 is also an open issue:

The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field. [2]

This is due to the reason that the parameters considered in the test set up which limit the model generalization may be totally different from real deployment. In this case, a DUT has an AI model to pass RAN4 test does not mean it can work well in the field.

Proposal 6: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to avoid that a UE can pass the test but performs poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

After achieving clear answers of the aforementioned questions, we can start to discuss how to define the setup, where the exiting FR2 OTA test set up should be considered as the baseline. Any enhancements on top of existing fancy enough framework should be justified.

Proposal 7: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

- Considerations on requirement definition under LLS

We understand that in RAN4 legacy, only the link-level channel model is used for defining test cases and requirements. Since there may exist a gap between the impact derived from system-level simulations and the impact derived from link-level simulations, how to use the evaluation results with system-level simulation assumptions for requirement definition study is still an open issue.

- Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: AoA and AoD are not used as model input in AI BM Case 1 and Case 2.

Proposal 1: Use single AoA as the channel model for AI BM testing.

Observation 2: With Gaussian distribution assumption of baseband measurement error, under different SNRs, the mean values are all close to 0, while the variance increases as the SNR drops.

Proposal 2: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 1.

Table 1. Measurement error impact on prediction accuracy regarding to KPI 1

| KPI-1 |  | Model input w/o measurement error | Model input w measurement error |
| --- | --- | --- | --- |
| Top K/1 w/o margin | Top_1_1 | 64.42% | 42.46% |
|  | Top_2_1 | 86.69% | 65.21% |
|  | Top_3_1 | 93.00% | 76.97% |
|  | Top_4_1 | 95.43% | 83.59% |
|  | Top_5_1 | 96.75% | 87.78% |
| Top-N genie-aided beam is one of the Top-K predicted beams | Top_2_2_1 | 94.58% | 79.33% |
|  | Top_2_4_1 | 98.60% | 92.10% |
|  | Top_3_2_1 | 96.81% | 85.99% |
|  | Top_3_4_1 | 99.44% | 95.64% |
|  | Top_4_4_1 | 99.75% | 97.57% |
|  | Top_5_2_1 | 98.60% | 93.45% |
| Top-N genie-aided beam is two of the Top-K predicted beams | Top_2_2_2 | 67.09% | 42.35% |
|  | Top_2_4_2 | 92.64% | 74.61% |
|  | Top_3_2_2 | 86.39% | 63.95% |
|  | Top_3_4_2 | 96.70% | 84.85% |
|  | Top_4_4_2 | 98.31% | 90.88% |
|  | Top_5_2_2 | 94.20% | 80.73% |
| maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB | x = 1dB | 83.02% | 60.75% |
|  | x = 2dB | 92.49% | 72.87% |
|  | x = 3dB | 96.29% | 81.84% |

Observation 3: The more stringent the KPIs are, the greater the impact of the measurement error on the prediction accuracy will be.

Proposal 3: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 2.

Table 2. Measurement error impact on prediction accuracy regarding to KPI 2

| KPI-2 |  |  | Model input w/o measurement error | Model input w measurement error |
| --- | --- | --- | --- | --- |
| 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam | Avg. | CDF_strong_1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_3 | 2.345dB | 5.392dB |
|  |  | CDF_strong_5 | 2.209dB | 4.893dB |
|  | w/o avg. | CDF_strong_Top1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_Top2 | 2.380dB | 5.492dB |
|  |  | CDF_strong_Top3 | 2.139dB | 4.781dB |
|  |  | CDF_strong_Top4 | 2.049dB | 4.445dB |
|  |  | CDF_strong_Top5 | 1.967dB | 4.044dB |
| 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams | Avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top1&2&3 | 2.236dB | 7.406dB |
|  |  | CDF_same_Top1&2&3&4&5 | 2.419dB | 7.444dB |
|  | w/o avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top2 | 2.200dB | 7.314dB |
|  |  | CDF_same_Top3 | 2.435dB | 7.702dB |
|  |  | CDF_same_Top4 | 2.674dB | 7.730dB |
|  |  | CDF_same_Top5 | 2.689dB | 7.287dB |

Observation 4: Regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam.

Proposal 4: RAN4 define requirements with measurement error as model input.

Proposal 5: RAN4 will not select KPI under which acceptable performance cannot be achieved, including

- the KPI under which the prediction accuracy is less than 80%, e.g, Top K/1 with K<4 without RSRP margin.

- the KPI under which the 90%-tile L1-RSRP difference is larger than 5dB, e.g, maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB with x<3dB.

Proposal 6: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to avoid that a UE can pass the test but performs poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

Proposal 7: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

References

- R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP RAN4 #114, Athens, Greece, 17th – 21st February, 2025.

- R4-2502969, “Updated simulation assumptions for beam prediction”, 3GPP RAN4 #114, Athens, Greece, 17th – 21st February, 2025.

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.