3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504332

# Wuhan, China, 7th - 11th April, 2025

Title:	Summary of NTN HPUE co-existence study

Source:	Samsung

Agenda item:	7.10.2.1

Document for:	Approval

1	Introduction

In last #114 meeting, there’re several contributions updating their co-ex results received, and the co-ex results were further updated and documented in [1].

In this paper, we take the agreements of WF[2] into account and try to summarize the co-ex study for better reading.

2	Co-ex study assumption

In #112 meeting, a stable version of co-ex study assumption was adopted [3]. And after #112 meeting, the following agreements reached in #112bis, #113 and #114 which has minor impacts on the co-ex assumption items.

And also it’s worth noting that in #114 meeting, the conclusion for co-ex study has reached, and as the working approaches the end, we believe it’s time to give a final updates to incorporate all the related agreements to a document, and it will also be helpful to the readers who would like to look into the details of the study.

The agreements reached in #112bis[4], #113 [5] and #114 [2] after the [3]:

Co-ex assumption related agreements in [4]: Co-existence studies Issue 1-1: Isolation distance Agreement: - The baseline is to use 2 x ISD (ISD = 750 meters) as the isolation distance Issue 1-2: Worst case scenario Agreement: - RAN4 to focus on scenario 4, and to skip scenario 5 Issue 1-3: Wrap around assumption for TN network Agreement: - Follow the previous agreement. Issue 2-3-2 TN network topology - Agreement: Reuse existing method from TR 38.863 as baseline, and encourage companies to further evaluate any performance impact with and w/o wrap-around functionality.. Issue 1-4 ~ Issue 1-6: Agreement: - Take option 2 in Issue 1-4 as an optional parameters, and the options in Issue 1-5 and Issue 1-6 as a package under this option. It’s shown in the table below. Scenarios; NR NTN for LEO; NR NTN for GEO; NB IoT NTN for LEO; NB IoT NTN for GEO. Baseline; Optional; Baseline; Baseline; Baseline; Optional. Orbits; LEO-1200 prioritized LEO-600 not precluded; LEO-1200 (Option 1 in Issue 1-5); GEO; LEO-1200 prioritized LEO-600 not precluded; GEO; GEO (Option 2 in Issue 1-5). UL BW; 2RB; 5MHz (Option 2 in Issue 1-4); 2RB; Single tone; Single tone; [0.18] MHz (Option 2 in Issue 1-4). SNR target (dB); 15; 3 (Option 2 in Issue 1-4); 15; R4-2217473; R4-2217473; 0 (Option 2 in Issue 1-4). ACLR model; TR 38.863;  For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s).  For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s). (Option 1 in Issue 1-6); TR 38.863; R4-2217473; R4-2217473; R4-2217473. Issue 1-7: HPUE ratio Agreement: - RAN4 to consider all NR NTN/NB-IoT NTN UEs as HPUEs, taking it as the worst-case scenario. Co-ex assumption related agreements in [5]: Topic #1: Co-existence study Sub-topic 1-1 How to treat different options for results discussions Issue 1-1: Basic principles for co-ex conclusion Agreement: For co-ex conclusion discussion, to follow similar principles in deriving ACIR as documented in TR 38.863 in Rel-17. It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case. Moreover, the following considerations are adopted to deal with major disputes for the selected cases’ results in each scenario: - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. - If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.. Issue 1-2: TN deployment scenario for co-ex conclusion Agreement: For co-ex study result conclusion discussion, in principle, to use Urban results instead of Rural results which aligns with previous agreement and worst case. Issue 1-3: TN BS antenna type for co-ex conclusion Agreement: For co-ex study result conclusion discussion, in principle, to use AAS results instead of non-AAS results as worst case. Issue 1-4: Wrap-around outside TN cells, and Issue 1-5: Isolation distance between NTN UE and TN cluster edge Agreement: Companies can provide the simulation results with or without wrap-around, and TR will capture the results with or without wrap-around separately. - With wrap-around: Enable wrap-around assuming 1.5km isolation. - Without wrap-around: Disable wrap-around assuming X isolation. - FFS whether X should be 0km or 0/1.5km. Co-ex assumption related agreements in [2]: Co-existence study Issue 1-1: Results handling with different isolation distances Agreement: - Agree to capture all submitted results in the TR, and to separate results with different isolation. - Use 1.5km isolation results to draw ACLR conclusion from co-ex study perspective.

In a spearate tdoc to this meeting [6], we present the latest version of the co-ex study assumptions for companies to review.

Proposal 1: To agree on the separate tdoc which updated the content to include latest agreements based on previously agreed [3] baseline text.

3	Co-ex study results summary

In this section, we try to summarize the submitted results considering the principles we reached in previous meetings. All companies results will be captured in the TR as agreed, but we find the results in the current excel file are too many. And these results are under different options, including wrap-around, isolation distance, etc.

Hence, based on previously agreement that to capture all submitted results in the TR, we propose to include them in the attached excel file as a new annex to the TR 38.863.

Proposal 2: To include all submitted results from NR-NTN HPUE coex studies as an excel attachment to TR 38.863, and introduce a new annex to accomodate it.

And we propose the following structure to accmodate the NTN HPUE coex studies in TR 38.863:

6 Co-existence study 23 6.1 Co-existence simulation scenario 23 6.2 Co-existence simulation assumption 25 6.3 Co-existence simulation methodology 42 6.4 Co-existence simulation results 44 6.5 Summary of co-existence study 69 6a Co-existence study for 17/27GHz 71 6a.1 Co-existence simulation scenario for 17/27GHz 71 6a.2 Co-existence simulation assumption for 17/27GHz 73 6a.3 Co-existence simulation methodology for 17/27GHz 83 6a.4 Co-existence simulation results for 17/27GHz 84 6a.5 Summary of co-existence study 113 6b Co-existence study for NTN HPUE in FR1 6b.1 Co-existence simulation scenario for NTN HPUE in FR1 6b.2 Co-existence simulation assumption for NTN HPUE in FR1 6b.3 Co-existence simulation methodology for NTN HPUE in FR1 6b.4 Co-existence simulation results for NTN HPUE in FR1 6b.5 Summary of co-existence study … Annex F Simulation results of NTN HPUE co-existence study

Proposal 3: To introduce new section ’6b’ and corresponding sub-sections to accomodate NTN HPUE co-ex studies in TR 38.863.

Given all the results are attached, as we proposed, in this new Annex F, we would like to further discuss the content in the above proposed 6b.4 and 6b.5.

The agreements we could follow are to divide results between with isolation distance and without isolation distance. And the co-ex study results are too large to be captured, and not to mention that each result are with different assumptions, which are also somehow divided in different aspects.

On the other hand, it is better to provide some results or data processing information in the main content of this newly proposed section 6b.4 in order to explain how the conclusions are derived. Thus, we propose the following as an example to capture the results in 6b.4.

<Below is an example to capture reults for each power class in section 6b.4> 6b.4 Co-existence simulation results for NTN HPUE in FR1 6b.4.1 Scenario 4: NR-NTN PC2 The co-existence results from all concerned options in this scenario were evaluated, and the detailed results can be found in [Annex F] to this TR. The number in below table had taken into account all the submitted results from these contributing companies for both average throughput loss and cell edge loss, and picked from the worst results from each contributing company. Table 6b4.1-1 Interpolated required ACLR values for Scenario 4 with isolation distance 1.5km assumed Company; Interpolated required ACLR. Samsung; <20. Vivo; <20. Ericsson; <20. ZTE; 30.6. MTK; <20. Thales; <20. Xiaomi; <20. Qualcomm; <20. Table 6b4.1-2 Interpolated required ACLR values for Scenario 4 without isolation distance assumed Company; Interpolated required ACLR. Samsung; 25.9. Vivo; 27.1. Ericsson; 30.1. MTK; 33. Huawei; 23.7. Xiaomi; 42.6. Qualcomm; 20. 6b.4.2 Scenario 4: NR-NTN PC1.5 [Place holder] 6b.4.3 Scenario 4: NR-NTN PC1 [Place holder] 6b.4.4 Scenario 4: IoT-NTN PC2 [Place holder] 6b.4.5 Scenario 4: IoT-NTN PC1 [Place holder] 6b.5 Summary of co-existence study The averaged interpolate ACLR values for each power classes of NR-NTN and IoT-NTN in Scenario 4 are presented in Table 6b.5-1 below. It should be noted that only co-ex study results assuming 1.5km isolation distance are taken into account in this summary. Table 6b.5-1: Average ACLR values for each power class Power classes; PC 2; PC 1.5; PC 1. ACLR values [dB]; NR-NTN; <20; [TBA]; [TBA]. IoT-NTN; [TBA]; N/A; [TBA].

Proposal 4: To use above example structure and methods to include results and summary to the TR 38.863.

4	Conclusion

For NTN HPUE Co-ex study aspects, we propose the following:

Proposal 1: To agree on the separate tdoc[6] which updated the content to include latest agreements based on previously agreed [3] baseline text.

Proposal 2: To include all submitted results from NR-NTN HPUE coex studies as an excel attachment to TR 38.863, and introduce a new annex to accomodate it.

Proposal 3: To introduce new section ’6b’ and corresponding sub-sections to accomodate NTN HPUE co-ex studies in TR 38.863.

Proposal 4: To use above example structure and methods to include results and summary to the TR 38.863.

5	Reference

[1] R4-2502997, Collected co-existence results, Samsung

[2] R4-2502954, WF on HPUE requirements for NR-NTN, Samsung

[3] R4-2414276, Simulation assumption for co-existence study, Samsung

[4] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung

[5] R4-2420403, WF on NR NTN HPUE, Samsung

[6] R4-2504331, Simulation assumption for co-existence study, Samsung