3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504338

# Wuhan, China, 7th - 11th April, 2025

Title:	Discussion on Ku-band coex study

Source:	Samsung

Agenda item:	7.11.2

Document for:	Approval

1	Introduction

In last RAN4 #114 meeting, the WF[1] and a dedicated co-ex assumption document[2] were adopted with latest agreements for co-ex study.

In this paper, we continue to discuss remaining open issues.

2	Discussion

Given the heavy workload of this completely new frequency range for NTN co-existence study, and limited remaining time, we propose to consider down scope the scenarios that currently listed in [2].

| No | Combination | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | 14 GHz | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | 14 GHz | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | 11 GHz | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | 11 GHz | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |  |

Given the outcomes from previous Rel-17 and Rel-18 NTN studies, we propose to prioritize following scenarios for certain targeted RF requirements:

For VSAT ACLR, we could prioritize Scenario 1 over Scenario 3, if time is limited. Because the VSAT usually pointed to the sky, while the TN UE usually assumed on the ground, compared to the interference from VSAT to the BS which located on the rooftop, the impact to the UE is less concerned.

Proposal 1: For VSAT ACLR, to prioritize Scenario 1 over Scenario 3 if time is limited.

For VSAT ACS, we could prioritize Scenario 8 over Scenario 5, especially when considering the TN UE equipped with omni-directional antenna. Due to the highly concentrated antenna with good directivity, the BS interference usually cause less impacts than the TN UE with omnidirectional antenna.

Proposal 2: For VSAT ACS, to prioritize Scenario 8 over Scenario 5 if time is limited.

For SAN ACS, we could prioritize Scenario 4 over Scenario 2, given the TN UE power is quite small compared to the TN BS.

Proposal 3: For SAN ACS, to prioritize Scenario 4 over Scenario 2, if time is limited.

For VSAT phase array antenna characteristics:

| 1 | VSAT Phased array antenna Characteristics |  |
| --- | --- | --- |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 1) | 0/ 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular (RHCP or LHCP; as a starting point) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | TBD (dependent on decisions on EIRP) |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). |  |  |

| Characteristics | Phased array antenna |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO) and 6 dB (LEO); Other options not precluded |
| Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO |
| Tx antenna gain | Option 1 - 36.6 dBi, Option 2 – 36.2 dBi |
| Antenna elevation angle | 25˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

If above two tables from [2] are valid, then the peak EIRP would be the sum of 50W and 36.6 dBi or 36.2 dBi for GEO, and 10 W and 36.6/36.2 dBi for NGSO. And then with each option, we break them down below:

For array size with 45x45, 36.6 or 36.2 dBi total antenna gain can be reached with element gain as 3.1 dBi or 3.5 dBi.

The peak antenna beamforming gain  = 10*log10(#of array) + element gain

Proposal 4: For 45x45 antenna array, the Tx antenna gain of 36.6 dBi and 36.2 dBi can be reached with an element gain of 3.1 dBi and 3.5 dBi respectively.

With the Tx transmit power as 50W for GSO and 10W for NGSO, and together with the antenna gain of 36.6 dBi and 36.2 dBi, the peak eirp will sums up to 83.6 / 83.4 dBm for GSO and 76.6 / 76.2 dBm for NGSO.

Proposal 5: To clarify the understanding that with 50W for GSO and 10W for NGSO, and antenna gain of 36.6 /36.2 dBi as given in [2], the peak eirp will sums up to 83.6 / 83.4 dBm for GSO and 76.6 / 76.2 dBm for NGSO.

For a phase array antenna, the total transmit power usually equals to:

Total transmit power = Power_per_element – Ohmic_loss + 10*log10(Array size)

With 45x45 array size, and 2dB ohmic loss, as given in above table, the power per element for GSO can be:

Power_per_element_GSO = 15.9 dBm

And Power per element of NGSO = 8.9 dBm.

Proposal 6: For 45x45 antenna array, with Tx power of 50W for GSO and 10W for NGSO, the power per element will be 15.9 dBm and 8.9 dBm.

Also considering the circular polarization are assumed for SAN and VSAT in ku-band co-ex, we propose to clarify the polarization discrimination between the NTN link with TN link.

Proposal 7: To clarify the polarization loss between circular and linear polarizations.

3	Conclusion

In this contribution, we provide views on co-ex study for Ku bands NTN:

Proposal 1: For VSAT ACLR, to prioritize Scenario 1 over Scenario 3 if time is limited.

Proposal 2: For VSAT ACS, to prioritize Scenario 8 over Scenario 5 if time is limited.

Proposal 3: For SAN ACS, to prioritize Scenario 4 over Scenario 2, if time is limited.

Proposal 4: For 45x45 antenna array, the Tx antenna gain of 36.6 dBi and 36.2 dBi can be reached with an element gain of 3.1 dBi and 3.5 dBi respectively.

Proposal 5: To clarify the understanding that with 50W for GSO and 10W for NGSO, and antenna gain of 36.6 /36.2 dBi as given in [2], the peak eirp will sums up to 83.6 / 83.4 dBm for GSO and 76.6 / 76.2 dBm for NGSO.

Proposal 6: For 45x45 antenna array, with Tx power of 50W for GSO and 10W for NGSO, the power per element will be 15.9 dBm and 8.9 dBm.

Proposal 7: To clarify the polarization loss between circular and linear polarizations.

4	Reference

[1] R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document)