# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504341

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.2.1

Source: 	Qualcomm Incorporated

Title: 	Views on new requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#114, a WF was agreed in [2]. In this contribution we share our views on existing BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

## Requirements for Transient period between SBFD and non-SBFD

In [2], the following was agreed regarding the location of transient period between SBFD and non-SBFD symbols as follows:

- Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

It is worth noting that from the deployment point of view, cases B and C are the relevant ones since we are not expecting to have transition from SBFD symbol to DL symbol or from UL symbol to SBFD symbol. Moreover, regarding the location of the transient period for cases B and D, our first preference that RAN4 would not define the location of the transient period between SBFD and non-SBFD symbols, since it is gNB implementation aspect.

Proposal 1: RAN4 to not define the location of the transient period between SBFD and non-SBFD symbol in the core specification.

In case companies within RAN4 would like to define the location in RAN4 specs, we believe that defining the location in the SBFD symbol, as blanked symbols, is the only viable way as defining them in the non-SBFD symbol will harm the legacy TDD symbols.

Proposal 2: In case RAN4 agreed to define the location of the transient period, RAN4 to agree to define the guard symbols in the blanked SBFD symbol in the beginning of the SBFD UL transmission for case C and at the end of the SBFD transmission for case B.

## In-channel adjacent subband leakage ratio

In [2] the following was agreed regarding general aspects of the newly introduced in-channel adjacent subband leakage ratio

- In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

During the study item phase, legacy ACLR and ACS was assumed for inter-channel leakage and inter-channel selectivity, this was mainly assumed to progress the work on the coexistence between legacy TDD and new SBFD deployments for the different simulation scenarios. In our views, it is a viable way forward to follow what was done in the study item as a baseline and further agree on methodologies that could be agreed within RAN4 to confirm the feasibility of such way forward. One challenge for the in-channel new requirements is the dependency on the self-interference capability yields the definition of a new inter-subband requirement difficult to agree between different companies. As observed in Section 9 in TR 38.853, different implementations can be assumed among vendors, leading to different self-interference, co-sector interference, and inter-site interference mitigation capabilities. Moreover, employed technologies and mitigation techniques in the SBFD-capable gNB vary within RAN4, and thus, room for implementation differences should be considered when defining the new inter-subband leakage and blocking requirements.

Observation 1: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent.

Proposal 3: RAN4 to agree on reusing ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the new in-channel adjacent subband leakage.

## In-channel adjacent subband blocking requirement

One of the main differences between the adjacent blocking requirement that are being discussed for SBFD-capable gNB and the in-channel adjacent subband blocking is the impact of the inter-gNB CLI mitigation/ handling techniques. Since in-channel emissions are being investigated, CLI handling techniques are a cornerstone of enabling intra-operator deployment of SBFD, as they offer solutions to mitigate intra-operator inter-gNB CLI. Accordingly, it is recommended that RAN4 consider the impact of the CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

Proposal 4: RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

# Conclusion

In this contribution, we provided our views on new BS RF requirements for SBFD with the following observations and proposals:

Proposal 1: RAN4 to not define the location of the transient period between SBFD and non-SBFD symbol in the core specification.

Proposal 2: In case RAN4 agreed to define the location of the transient period, RAN4 to agree to define the guard symbols in the blanked SBFD symbol in the beginning of the SBFD UL transmission for case C and at the end of the SBFD transmission for case B.

Observation 1: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent.

Proposal 3: RAN4 to agree on reusing ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the new in-channel adjacent subband leakage.

Proposal 4: RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2502310, “Way Forward for [113][306] NR_duplex_evo_General”, RAN4#114, Samsung.