3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504355

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.24.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for CW node

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Issue 3-1: CW Output power (for ahoc discussion) - Agreement: - CW output power [33] dBm. - Issue 3-2: ON/OFF power - Agreement: - FFS for OFF power - Issue 3-3: Frequency error - Agreement: - Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW. - Issue 3-4: Phase noise - Agreement: - Further discuss phase noise requirements - Issue 3-5: Unwanted emission - Agreement: - Specify emission requirements, values FFS - Issue 3-6: Spurious emissions - Agreement: - NR spurious emission requirement can be considered as baseline for R19 AIoT CW node - Issue 3-7: Intermodulation - Agreement: - FFS

Discussion

- In the previous RAN4 meeting and RAN task, there were some discussion on the feasibility study for A-IoT BS especially from CW cancellation similar as SBFD DL sub-band to UL sub-band interference analysis. As we know that A-IoT BS will receive the CW signal from the transmitter side. CW interference could be mainly dominated by phase noise model. In the following figure 2.3-1, we proposed the phase noise model according to FR2 phase model and update it according to its operating frequency according to the Lesson theory. Based on the initial analysis from phase noise model according to the following figure 2.3-1, the CW leakage would be roughly around 50dB (0dB-[-91+10*log10(15*10^3)]) which is much better than 28dBc assumption at UE side.

- In addition, the spatial isolation for D1T1-B could be considered.

- Figure 2.3-1. Phase noise model at 1GHz [7.5kHz: -117dBc/Hz，90kHz: -128dBc/Hz]

- During the last RAN4 meeting, there were some discussion on how to define the spectral emission mask for CW node, e.g.

- Reusing NB1/NB2 UE spectral emission mask based on UE transmission power as 23/20/16dBm

- Reusing GSM UE spectral emission mask based on UE transmission power as 33dBm

Table 6.6.2F.1-1: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Table 2.2-2: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

- If we reuse GSM UE spectral mask for CW node as following, as some Guard band each to D2R 180KHz need to reserved, other words, at least the 2PRBs should be reserved for D2R/CW transmission, or 3 PRBs should be reserved for D2R/CW transmission.

Figure 2.1. CW spectral mask

- Observation 1: if we reuse GSM UE spectral emission mask for CW node, then at least 2 PRBs or 3 PRBs should be reserved for CW transmission due to the reserved guard band next to 180KHz D2R bandwidth;

- Proposal 1: propose to further the guard band (e.g 1PRB or 2 PRBs) if GSM UE spectral mask is reused for CW node transmission.

- Issue 3-2: ON/OFF power

- Agreement:

  - FFS for OFF power

- In the last RAN4 meeting, there were some discussion how to define ON/OFF power for CW node since the CW transmission is not always on or contiguous transmission, therefore some ON/OFF power might be needed. From our understanding, the UE OFF power as defined in the 38.101-1 could be updated as following:

|  | Channel bandwidth / Transmit OFF power / Measurement bandwidth |
| --- | --- |
|  | 0.18MHz |
| Transmit OFF power | -50 dBm |
| MBW | 0.18MHz |

- Proposal 2: propose to define the OFF power for CW node transmission as -50dBm/0.18MHz.

- Regarding the Tx intermodulation requirement for CW node, from our understanding, it depends on the following factors:

- Whether CW node will transmit multiple tone CW signal in addition to single tone, for multiple tone CW transmission, then Tx intermodulation requirement should be defined which is different from out of carrier interference and also tested

- Whether UE uplink transmission reached the antenna connector of CW node, if the power is still 30dBc lower than wanted signal, then Tx intermodulation requirement should be needed, however considering the following MCL 53dBc for MR case and maximum UE transmission power as 23dBm, then the interference reached the antenna connector of CW node is -30dBm which is 63dBc lower than wanted signal. From this perspective, the Tx IMD requirement should be not needed.

- Proposal 3:  if CW node will transmit the multiple tone CW signal in addition to single tone CW signal, then Tx IMD requirement are still need as intra-system TxD requirement.

Conclusions

In this contribution, we shared some initial views on the RF requirement for CW node in D1T1-B and proposals are made as following:

- Observation 1: if we reuse GSM UE spectral emission mask for CW node, then at least 2 PRBs or 3 PRBs should be reserved for CW transmission due to the reserved guard band next to 180KHz D2R bandwidth;

- Proposal 1: propose to further the guard band (e.g 1PRB or 2 PRBs) if GSM UE spectral mask is reused for CW node transmission.

- Proposal 2: propose to define the OFF power for CW node transmission as -50dBm/0.18MHz.

- Proposal 3: if CW node will transmit the multiple tone CW signal in addition to single tone CW signal, then Tx IMD requirement are still need as intra-system TxD requirement.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282	RF requirements for Ambient IoT BS, Huawei,

- R4-2500782, RF requirements for A-IoT BS, Huawei, Noted.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2501863, RF requirements for CW, Huawei, approved.