3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504368

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.23.1

Source: 	ZTE Corporation

Title: 	Discussion on system parameters for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. During the last RAN4 meeting, we have some initial discussions on work plan and general aspects for SBFD BS, however there are still lots of open issues not addressed yet. In this contribution, we would like to share further views on these remaining issues.

- Issue 1-1-4: Declaration and flexibility for SBFD operation Agreement: - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS - Sub-topic 1-2: DL/UL subband configurations support for SBFD - Issue 1-2-1: Restriction on the number of UL subband sizes supported - No new agreement achieved. - Issue 1-2-2: Restriction on the number of DL subband sizes supported - No new agreement achieved. - Issue 1-2-3: Restriction on guardband size for SBFD - No new agreement achieved. - Issue 1-2-4: Other restriction on DL subband size - No new agreement achieved. - Sub-topic 1-3: Requirement Applicability for SBFD - Issue 1-3-1: Requirement set applicability table for SBFD-capable BS Way Forward: - RAN4 further study the applicability of RAN4 requirement for SBFD capable BS, by taking the table provided in R4-2501967 as example to be further study on. NOTE: The table does not imply that the BS can be declared 1-H for non-SBFD and 1-O for SBFD, or vice versa. - Sub-topic 1-4: Fair coexistence between SBFD and TDD networks - Issue 1-4-1: Limiting Maximum UE Transmit power in SBFD symbols/slots. Way Forward: - No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of limiting Maximum UE Transmit power in SBFD symbols/slots - Issue 1-4-2: Using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots Way Forward: - No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots

Discussion

- Issue 1-1-4: Clarification on declaration and flexibility

- Regarding the declaration and flexibility, most controversial part is about the necessity of having the analog filtering after the LNA at the receiver side, if there are no such kind of analog filter for the protection of receiver’s saturation, then the RAN4 standard should be flexible to have different DL-UL sub-band configuration and only test the worst case for the conformance testing. For FR1 WA SBFD BS, it is clear that some analog filtering are needed at the receiver, this is part of reasoning why BS could only operate under the tested sub-band configuration. However for FR1 MR SBFD BS, from our understanding, the analog filtering is not needed which is similar as FR1 LA SBFD BS.

- Proposal 1: regarding the declaration and flexibility for SBFD operation, we propose to define the conformance testing as following way:

- FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration;

- For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration)

- Issue 1-3-2: DL/UL subband configurations support for SBFD

Agreement:

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

    - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

  - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

    - FFS different sizes for specific UL subband for DUD and DU/UD.

    - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth.

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

- Regarding the maximum specific UL sub-band size, it is still open. From our understanding, we don’t see the necessity to restrict to 2 specific UL sub-band. From the standardization perspective, to specify the maximum UL sub-band size as 3 could leave more flexibility for manufacture declaration if it is capable to support.

- Proposal 2: regarding the DL/UL subband configurations support for SBFD, to set the maximum UL sub-band size as 3 for FR1 WA SFBD and don’t see this necessity to apply this feature to other SBFD BS class.

- Issue 1-3-6: Other restriction on DL subband size

Way Forward:

- FFS the following restriction on DL subband size:

  - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

  - FR2 minimum CBW to support SBFD is 100MHz.

- The above proposal is mainly motivated by sub-band DL symbols overlapping with SSB symbols in the time domain, if this is overlapped, then such minimum channel bandwidth restriction is needed, however SBFD symbols could also overlap with other DL symbols. In short, we don’t see the necessity to mandate such kind of restriction for DL sub-band. This should be totally up to the manufacture declaration.

- Proposal 3: propose not to mandate the additional restriction for DL sub-band band.

## Topic 2: PUSCH with resource muting for CLI handling

- In the previous RAN1 meeting, it was agreed to configure 1~2 OFDM symbols for PUSCH channel for CLI muting with comb pattern in the freq domain (every other REs to be muted). In the previous RAN4 meeting, there were some initial discussions on it, however there were no consensus reached so far. Technically speaking, the MPR evaluation might be needed for PUSCH with resource muting for CLI handling.

Simulation assumptions for FR1 MPR evaluation for CLI muting solution.

- Non-linearity

PA is calibrated to 1dB MPR point according to FR1 MPR evaluation convention to meet ACLR, SEM and EVM requirements.

PA model: Polynomial model (Other PA models are not precluded)

Reference waveform: 20MHz CBW, 15kHz SCS, QPSK, 100RB3, DFT-s-OFDM

- Tx impairment

IQ image and carrier leakage: 28dBc

CIM3 = 60dBc

Simulation assumptions for FR2 MPR evaluation for CLI muting solution.

- Non-linearity

PA model: ~28GHz CMOS model as captured in TR 38.803 (Other PA models are not precluded)

- Tx impairment

IQ image and carrier leakage:

25dBc for pi/2 BPSK, QPSK and 16QAM

28dBc for 64QAM

36dBc for 256QAM

More simulation results would be provided later on.

Proposal 4: propose to conduct the MPR evaluation for PUSCH with resource muting for CLI solution.

- Topic 3: Location of transient period between SBFD and non-SBFD

For the transition period between SBFD and non-SBFD symbols, there are still two open issues left for further discussion.

Case B:

For the case B, it is not clear whether this guard period should be configured as SBFD symbols or not. From our understanding, both DL symbols and flexible symbols could be configured as symbol symbols from standardization perspective, the guard symbols could be configured by flexible symbols. In other words, guard symbols could be also configured by SBFD symbols in theory from the pure signalling perspective, however we don’t see such kind of the necessity for it,

Proposal 5: for Case B, propose to configure the guard symbols following the the legacy TDD DL-UL configuration.

Case C:

The guard period between DL symbols and also SBFD symbols are still needed to allow the RF reconfiguration from BS side and also allow the UE to do the Rx to Tx switching etc, however whether these guard period could be contained within DL symbols or SBFD symbols which could be left up to the implementation. Meanwhile the TDD SBFD DL/UL configuration for SBFD conformance testing, it should be still made it to be clear e.g. as shown in the following table and figure 1.

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |
| SBFD-StartingSlotIndex | TBD | TBD | ? |
| SBFD-StartingSymbolIndex | TBD | TBD | ? |
| SBFD-EndingSlotIndex | TBD | TBD | ? |
| SBFD-EndingSymbolIndex | TBD | TBD | ? |
| scs-SpecificfirstDLSubbandrList | TBD | TBD | ? |
| scs-SpecificsecondDLSubbandrList | TBD | TBD | ? |
| scs-SpecificULSubbandrList | TBD | TBD | ? |
| SubbandlocationAndBandwidth | TBD | TBD | ? |
| NOTE: 60KHz SCS is optional in FR1 and not supported by the commercial UE yet. |  |  |  |

Figure 1. some potential sub-band configuration in the time domain for the conformance testing.

Proposal 6: for Case C, whether the guard period to be contained within DL symbols or SBFD symbols could be left up to the implementation, however this should be made clear in the conformance testing specification for testing.

- Topic 4: HPUE related updates due to the SBFD configuration

- For SBFD was agreed to be specified in FR1/FR2 TDD bands only and there are lots of HPUE related duty cycle specified for the single TDD band and also EN-DC or NR-CA. For the legacy dutycycle evaluation, the calculation is based on the legacy uplink symbols, however considering the SBFD feature would allow the DL/flexible symbols to be configured as SBFD symbols with both DL sub-band and also UL sub-band, this will change the dutycycle evaluation at the UE side, please find the following updates for HPUE in single band in clause 6.2 in TS 38.101-1.

- Some initial updates for HPUE in clause 6.2 in TS38.101-1.

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols and SBFD symbols with UL transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

Proposal 7: propose to consider the SBFD symbols together with the legacy UL symbols for dutycycle evaluation for HPUE cases including HPUE in single band, EN-DC or NR-DC.

Conclusions

In this contribution, we want to share some further views on general aspects for SBFD BS and proposals/observations are made as following:

- Proposal 1: regarding the declaration and flexibility for SBFD operation, we propose to define the conformance testing as following way:

- FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration;

- For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration)

- Proposal 2: regarding the DL/UL subband configurations support for SBFD, to set the maximum UL sub-band size as 3 for FR1 WA SFBD and don’t see this necessity to apply this feature to other SBFD BS class.

- Proposal 3: propose not to mandate the additional restriction for DL sub-band band.

Proposal 4: propose to conduct the MPR evaluation for PUSCH with resource muting for CLI solution.

Proposal 5: for Case B, propose to configure the guard symbols following the the legacy TDD DL-UL configuration.

Proposal 6: for Case C, whether the guard period to be contained within DL symbols or SBFD symbols could be left up to the implementation, however this should be made clear in the conformance testing specification for testing.

Proposal 7: propose to consider the SBFD symbols together with the legacy UL symbols for dutycycle evaluation for HPUE cases including HPUE in single band, EN-DC or NR-DC.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2406107, Way Forward for [110bis][313] NR_duplex_evo

- R4-2409958, Way forward for [110bis][313] NR_duplex_evo, Approved

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General, Approved.

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General

- R4-1801031, WF on NR BS RF FRCs, approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2501967	Further discussion on general aspects for SBFD system, Samsung

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.