3GPP TSG-RAN WG4 Meeting #114bis									                      R4-2504374

Wuhan, China, April 7th – 11th, 2025

Agenda item:			7.1.2

Source:	Ericsson

Title:	Discussions on RRM performance requirements of Rel-19 WI on UE RF enhancements

Document for:	Discussions

# Introduction

In this paper we discuss the RRM performance impact of 6Rx UE introduced in [1]:

High power UE (HPUE) for CA in terrestrial network (TN) - Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO - Example band combinations: - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO - Example band combinations: - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA - Focus on the SAR solution - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE - Identify and update the requirements if necessary - Only PC3 is considered for LTE FDD in EN-DC - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified - Introduce the signaling to support the above objectives, if needed. - Consider release independency, if needed Power boosting and/or MPR reduction - Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Introduce 6 MIMO layers support as an optional feature only for FWA. - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. General for all the areas - Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible.

# Discussions

It is recalled that no additional RRM core requirements were defined for 6Rx UEs and it was concluded that the existing interruption requirements at SRS antenna port switching can be reused. Hence, no specification updates were made in the core requirements.

- Existing interruption requirements at SRS antenna port switching were reused for 6Rx UEs.

Introducing support for 6Rx capable UE is expected to trigger some RRM performance work. These impacts are discussed below.

Impact on antenna configurations for 6Rx capable UEs

Annex 3 of TS 38.133 contains antenna configurations specifically defined for 4Rx capable UEs and 8Rx capable UEs in FR1. The configuration includes principle of testing specifying which of existing test cases are applicable and with which setting, e. g. the modified SNR level for the existing test cases. For the RLM and BFD test cases, RAN4 needs to discuss whether the existing (2 Rx, or 4 Rx requirements) SNR levels can be reused. Due to that the UE is capable of 6Rx, it is expected that the different RLM/BFD events (in-sync, out-of-sync, BFD, CBD) can be triggered at different SNR levels compared to those defined for the legacy UEs assuming 2Rx. It is noted that for similar reason, the RLM/BFD tests are run with modified SNR levels for 4Rx and 8Rx capable UEs.

Since scope of the WI is 6Rx capable UEs for FR1 in single carrier scenario, the antenna configuration shall only focus on testing rules for single carrier tests. Therefore, DC and CA related tests can be excluded. In addition, the scenario of introducing 6Rx capable is similar to 4 Rx, it is therefore proposed to use the existing 4Rx requirements as baseline and do the necessary adaptation for 6 Rx.

- New antenna configurations are to be defined for 6 Rx capable UEs targeting single carrier operation using the existing 4Rx configurations as reference.

- RAN4 to discuss the modified SNR levels for RLM and BFD tests for 6 Rx capable UEs.

# Conclusion

In the previous sections, we made the following observations:

- Existing interruption requirements at SRS antenna port switching were reused for 6Rx UEs.

- New antenna configurations are to be defined for 6 Rx capable UEs targeting single carrier operation using the existing 4Rx configurations as reference.

- RAN4 to discuss the modified SNR levels for RLM and BFD tests for 6 Rx capable UEs.

# References

- RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.