3GPP TSG-RAN WG4 Meeting #114bis						        	 		R4-2504386

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Results for PC1.5 contiguous ULCA with comparison to PC2 and 6dB PSD imbalance

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.2	Intra-band contiguous and non-contiguous UL CA with PC1.5	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In R4#114 we provided a preliminary set of measurement results [1] for PC1.5 contiguous intra-band ULCA, and based on results from other companies, a tentative MPR for contiguous allocation was agreed in [2]. At the time of R4#114, we did not have measurements for PC2 2Tx nor PC1.5 with 6dB PSD differences, so we were not able to comment on the proposed values in [2] as we usually rely on relative back-off values with direct comparisons between PC1.5 2Tx versus PC2 2Tx. Similarly, we needed to compare PC1.5 with equal PSD versus +/-6dB PSD imbalance. In this contribution, with all the measurements available to make a fair assessment versus the existing 2Tx PC2 MPR, we make MPR values proposals for both contiguous and non-contiguous allocations.

# Discussion

## In meeting R4#114, several were captured in [2] with tentative values in [] for PC1.5 contiguous ULCA with contiguous allocation MPR. These agreements are reproduced here in italics.

Agreement:

- Define separate MPR/A-MPR requirements for handheld and FWA

- Companies are encouraged to bring A-MPR values in following meetings

- The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2

- Having the following PC1.5(2Tx) MPR values for equal PSD as starting point to be further checked in upcoming meetings

  - Companies can further evaluate the unequal PSD (6dB) impacts in next meeting.

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [7] | [9] | [6.5] | [8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [9] | [9.5] | [8.5] | [9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [6-6.5] | [9.5] | [5.5-6] | [9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [8.5] | [9.5] | [8.5] | [9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

Agreement:

- The MPR requirements defined are carrier power and are PSD balance/imbalance agnostic

- The MPR values are derived with equal PSD condition with margin reserved to accommodate unequal PSD cases

- Send LS to RAN5 on PSD condition RAN4 assumed to derive MPR requirements, if needed

In this contribution, we will first discuss the tentative values for contiguous values and then provide preliminary input for non-contiguous allocations.

## 2.1 Measurement Data

The measured data is provided in the annex and is the raw data without correction including:

- Measured back-off values needed to meet ACLR, SEM and spurious emission. EVM and IBE are not measured as they do not dictate the MPR and for intra-band CA. Furthermore, the MPR specification does not account for the impact of cross CC interference in one CC IBE and EVM due to the other CC impairments.

- PC1.5 2Tx based on two PC2 PAs calibrated for 31dB ACLR DFT-s-OFDM QPSK 20MHz 100RB0 waveform.

  - A set of inner/outer contiguous and inner/outer1/outer2 non-contiguous allocations with equal PSD for two CCs of 80MHz

  - A subset of the above allocations with equal +6dB or -6dB PSD imbalance for comparison.

- PC2 2Tx based on two PC3 PAs (same PA but different biasing) calibrated for 30dB ACLR DFT-s-OFDM QPSK 20MHz 100RB0 waveform.

  - A subset of the PC1.5 2Tx allocations with equal PSD for comparison.

- All measurements use 10dB antenna coupling so a 0.5dB MPR relaxation (as captured in [2] should be allowed for FWA based on 20dB antenna coupling.

- For the non-contiguous allocations where back-off above 6dB is found at nominal voltage, additional measurements may be needed with lower voltage on the PA to mimic what happens for ET and APT PAs at moderate back-off. This means that back-offs in the >5dB range in the current set of measurements probably need a 1 to 2dB additional back-off.

- With the choice of two CCs of 80MHz, both CCs are 30kHz SCS. For non-contiguous allocations with 1RB+1RB, a 15kHz SCS would result in a slightly higher PSD and thus, in absence of 15kHz based measurements, we suggest adding 1dB back-off for 1RB+1RB cases.

- The measurements were only performed when allocation was present in each CC since in R19 we agreed that the single CC MPR applies when there are RB allocated in only one CC.

## 2.1 Contiguous Allocations

First, we analyze the impact of the +/-6dB imbalance on the back-off based on the measurements in Annex Table A1.

Observations:

- Except for the cases where there is margin to all the OOBE which are limited by the measurement floor (but correspond to a negative MPR), all the allocations requiring back off are limited by ACLR.

- Worst case inner allocation:

  - The back-off for PC1.5 with equal PSB is 0.9dB for CP and 0.3dB for DFT, but in-band requirement means that the MPR should at least be equal to the single CC case.

  - The 6dB imbalance impact is about 0.5dB

- Worst case outer allocation:

  - The back-off for PC1.5 with equal PSB is 4.5dB for CP and 3.8dB for DFT. This is less than 5dB, so values should be valid even for ET/APT based solution where the supply is reduced.

  - The 6dB imbalance impact is about 0.5dB.

Then, we analyze the back-off difference between PC1.5 and PC 2Tx with equal PSD based on the measurements in Annex Table A2.

Observations:

- Except for the cases where there is margin to all the OOBE which are limited by the measurement floor (but correspond to a negative MPR), all the allocations requiring back off are limited by ACLR.

- Worst case inner allocation: The back-off for PC2 is about 0.5dB worse than PC1.5 which is consistent with the limitation being ACLR and PC2 PAs being less linear than PC1.5 PAs (30dB ACLR target vs 31dB).

- Worst case outer allocation: The back-off for PC2 is about 1dB worse than PC1.5 which is consistent with the limitation being ACLR and PC2 PAs being less linear than PC1.5 PAs (30dB ACLR target vs 31dB). With larger transmitted BW, that difference becomes more important.

- Based on the above, the fact that PC2 2Tx MPR already accounts for unequal PSD, and the limited impact of 6dB PSD difference measured, there is no strong justification for the PC1.5 MPR to be larger than the PC2 MPR for contiguous allocation in contiguous ULCA

If we can accept that our measurements may represent better absolute performance than other solutions, we have difficulties understanding why a PC1.5 2Tx MPR would be worse than the agreed PC2 2Tx MPR given that PC2 2Tx uses two PC3 PAs that are less linear that the two PC2 PAs used in PC1.5 2Tx. Since all the limiting requirements are relative (ACLR, IBE, EVM) the improved PA linearity should benefit the PC1.5 MPR. We thus suggest that other companies initiate a further verification on which limitations drive their PC1.5 MPR results.

In any case, we do not see why we should adopt the worst values in a range proposed in [2], and as a compromise propose to discuss a range between the PC2 2Tx MPR and the best value proposed in [2] highlighted in yellow.

Proposal on PC1.5 2Tx contiguous ULCA contiguous allocations MPR: We propose to discuss the ranges highlighted in yellow and in [] in the following table with contributing companies. PC3 1Tx and PC2 2Tx are provided for reference.

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [3.5-4.5] | [8-9] | [3-4] | [7.5-8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [3.5-4.5] | [8-9] | [3-4] | [7.5-8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [3.5-4.5] | [8-9] | [3-4] | [7.5-8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [6-7] | [8-9] | [5.5-6.5] | [7.5-8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [8-9] | [8.5-9.5] | [7.5-8.5] | [8-9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [4-5] | [8.5-9.5] | [3.5-4.5] | [8-9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [4-5] | [8.5-9.5] | [3.5-4.5] | [8-9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [5.5-6] | [8.5-9.5] | [4.5-5.5] | [8-9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [7.5-8.5] | [8.5-9.5] | [7-8.5] | [8-9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

## 2.1 Non-contiguous Allocations

First, we analyze the impact of the +/-6dB imbalance on the back-off based on the measurements in Annex Table A3.

Observations:

- Inner allocations are either limited by ACLR (relative requirement that should improve for PC1.5 2Tx vs PC2 2Tx) or the -10dBm/MHz or the -13dBm/MHz SEM regions (absolute requirement that becomes 3dB harder for PC1.5 versus PC2)

- Worst case inner allocation is limited by the first MHz OOB SEM:

  - The back-off for PC1.5 with equal PSB is 2.9dB for CP and 2.2dB for DFT.

  - The 6dB imbalance impact is about 0.5dB

- Outer1 allocations are either limited by ACLR or the -13dBm/MHz SEM regions.

- Worst case outer1 allocation:

  - The back-off for PC1.5 with equal PSB is 6.5dB for CP and 6dB for DFT. This is more than 5dB so values should be validated with measurements at reduced supply to account for the ET/APT PA.

  - The 6dB imbalance impact may be up to 2dB

- Outer2 allocations are either limited by ACLR or the -13dBm/MHz SEM regions or the -30dBm/MHz spurious emission region.

- Worst case outer1 allocation:

  - The back-off for PC1.5 with equal PSB is 7.5dB for CP and 6.6dB for DFT. This is more than 5dB, so values should be validated with measurements at reduced supply to account for the ET/APT PA.

  - The 6dB imbalance impact is about 1dB.

Then, we analyze the back-off difference between PC1.5 and PC 2Tx with equal PSD based on the measurements in Annex Table A4.

Observations:

- Worst case inner allocation is limited by the -10dBm/MHz SEM region: The back-off for PC2 is about 2dB better than PC1.5 which is consistent with the limitation being absolute and 3dB more stringent. Note that allocations that are ACLR limited (relative) see the PC2 back-off being about 1 to 1.5dB worse than for PC1.5.

- Worst case outer1 allocation is limited by the -13dBm/MHz SEM region: The back-off for PC2 is about 0.5dB better than PC1.5 which is consistent with the limitation being absolute. Note that some allocations that are ACLR limited in PC2 become SEM limited in PC1.5. In the cases where both PC1.5 and PC2 are ACLR limited, the back-off is worse by about 1dB for PC2 versus PC1.5

- Worst case outer2 allocation: The back-off for PC2 is about 1dB worse than PC1.5 which is consistent with the limitation being ACLR and PC2 PAs being less linear than PC1.5 PAs (30dB ACLR target versus 31dB). With larger transmitted BW that difference gets more important.

Even for non-contiguous allocations, it is confirmed that PC1.5 back-off is lower than PC2 when the requirement is relative (ACLR). Nevertheless, the worst-case back off for non-contiguous allocations is always limited by the absolute SEM or SE requirement. Thus, the MPR for the PC1.5 non-contiguous allocations should be larger than for PC2 2Tx and the observed difference can be the basis for an MPR proposal.

It should be noted that for most back-off values >5dB additional low voltage measurements should be conducted which is why our proposal is tentative and based on the fact that PC2 MPR is also affected by the same issue. Thus, applying the measured delta at nominal voltage seems appropriate. At this time, we do not have a clear view on how much the FWA case can benefit from a lower coupling of the antennas.

Proposal on PC1.5 2Tx contiguous ULCA non-contiguous allocations MPR: We propose to discuss the ranges highlighted in yellow and in [] in the following table with contributing companies. PC3 1Tx and PC2 2Tx are provided for reference. FWA case is FFS.

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 33 | 7.5 | 13 | [5] | [8] | [13-14] |
|  | QPSK | 2.5 | 6 | 11.5 | 33 | 7.5 | 13 | [5] | [8] | [13-14] |
|  | 16QAM | 3 | 6 | 11.5 | 33 | 7.5 | 13 | [5] | [8] | [13-14] |
|  | 64QAM | 5 | 6 | 11.5 | 5 | 7.5 | 13 | [5.5] | [8] | [13-14] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 6.5 | 7.5 | 14 | [6.5] | [8] | [13-14] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 3.53 | 8 | 14 | [5.5] | [8.5] | [14-15] |
|  | 16QAM | 3.5 | 7 | 12 | 3.53 | 8 | 14 | [5.5] | [8.5] | [14-15] |
|  | 64QAM | 5 | 7 | 12 | 5 | 8 | 14 | [6] | [8.5] | [14-15] |
|  | 256QAM | 7.5 | 7.5 | 12 | 7.5 | 8 | 14 | [7.5] | [8.5] | [14-15] |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we further provided a larger batch of measurements for PC1.5 contiguous intra-band ULCA with PC2 and 6dB imbalance additional data for comparison. This allows a solid MPR proposal for contiguous allocations and a good basis for a tentative MPR for non-contiguous allocation pending verification at low PA voltage due to large required back-off.

Proposal on PC1.5 2Tx contiguous ULCA contiguous allocations MPR: We propose to discuss the ranges highlighted in yellow and in [] in the following table with contributing companies. PC3 1Tx and PC2 2Tx are provided for reference.

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [3.5-4.5] | [8-9] | [3-4] | [7.5-8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [3.5-4.5] | [8-9] | [3-4] | [7.5-8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [3.5-4.5] | [8-9] | [3-4] | [7.5-8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [6-7] | [8-9] | [5.5-6.5] | [7.5-8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [8-9] | [8.5-9.5] | [7.5-8.5] | [8-9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [4-5] | [8.5-9.5] | [3.5-4.5] | [8-9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [4-5] | [8.5-9.5] | [3.5-4.5] | [8-9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [5.5-6] | [8.5-9.5] | [4.5-5.5] | [8-9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [7.5-8.5] | [8.5-9.5] | [7-8.5] | [8-9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

Proposal on PC1.5 2Tx contiguous ULCA non-contiguous allocations MPR: We propose to discuss the ranges highlighted in yellow and in [] in the following table with contributing companies. PC3 1Tx and PC2 2Tx are provided for reference. FWA case is FFS.

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 33 | 7.5 | 13 | [5] | [8] | [13-14] |
|  | QPSK | 2.5 | 6 | 11.5 | 33 | 7.5 | 13 | [5] | [8] | [13-14] |
|  | 16QAM | 3 | 6 | 11.5 | 33 | 7.5 | 13 | [5] | [8] | [13-14] |
|  | 64QAM | 5 | 6 | 11.5 | 5 | 7.5 | 13 | [5.5] | [8] | [13-14] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 6.5 | 7.5 | 14 | [6.5] | [8] | [13-14] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 3.53 | 8 | 14 | [5.5] | [8.5] | [14-15] |
|  | 16QAM | 3.5 | 7 | 12 | 3.53 | 8 | 14 | [5.5] | [8.5] | [14-15] |
|  | 64QAM | 5 | 7 | 12 | 5 | 8 | 14 | [6] | [8.5] | [14-15] |
|  | 256QAM | 7.5 | 7.5 | 12 | 7.5 | 8 | 14 | [7.5] | [8.5] | [14-15] |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. |  |  |  |  |  |  |  |  |  |  |

# References

[1] 2502067 Measurement results for PC1.5 intra-band contiguous ULCA, Skyworks Solutions Inc., R4#114

[2] R4-2502863 WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, R4#114

# Annex Raw measured data

The measured data provided in this annex is the raw data without correction and includes:

- Measured back-off values needed to meet ACLR, SEM and spurious emission. EVM and IBE are not measured as they do not dictate the MPR for intra-band CA. Furthermore, the MPR specification does not account for impact of cross CC interference in one CC IBE and EVM due to the other CC impairments.

- PC1.5 2Tx based on two PC2 PAs calibrated for 31dB ACLR DFT-s-OFDM QPSK 20MHz 100RB0 waveform.

  - A set of inner/outer contiguous and inner/outer1/outre2 non-contiguous allocations with equal PSD for two CCs of 80MHz

  - A subset of the above allocations with equal +6dB or -6dB PSD imbalance for comparison.

- PC2 2Tx based on two PC3 PAs (same PA but different biasing) calibrated for 30dB ACLR DFT-s-OFDM QPSK 20MHz 100RB0 waveform.

  - A subset of the PC1.5 2Tx allocations with equal PSD for comparison.

- All measurements use 10dB antenna coupling so a 0.5dB MPR relaxation (as captured in [2] should be allowed for FWA based on 20dB antenna coupling.

- For the non-contiguous allocations where back-off above 6dB is found at nominal voltage, additional measurements may be needed with lower voltage on the PA to mimic what happens for ET and APT PAs at moderate back-off. This means that back-offs in the >5dB range in the current set of measurements probably need a 1 to 2dB additional back-off.

- With the choice of two CCs of 80MHz both CCs are 30kHz SCS. Non-contiguous allocations with 1RB+1RB a 15kHz SCS would result in a slightly higher PSD and thus, in absence of 15kHz based measurement. we suggest adding 1dB back-off for 1RB+1RB cases.

- The measurements were only performed when allocation was present in each CC since in R19 we agreed that the single CC MPR applies when there are RB allocated in only one CC.

The tables provide the following information:

- The first five column provide:

  - Allocation type with CCI for contiguous inner, CCO for contiguous outer, NCI for non-contiguous inner, NO1 for non-contiguous outer1 and NO2 for non-contiguous outer2.

  - Number of RB (#RB1) and RBstart (RB#1) for CC1 then the number of RB (#RB1) and RBstart for CC2 (RB#1)

- The next three columns are the CP-OFDM PC1.5 with equal PSD measurement results with:

  - For a positive value, BO is for the back off needed to meet all emissions requirements. A negative value provides the virtual negative back-off when there is margin to emissions at PCmax (the value is margin/3dB)

  - The limit column provides which emission requirement is dictating the back-off (or has the lowest margin): ac is ACLR, s4 is the spurious region at -30dBm/MHz, s3 is the SEM region at -25dBm/MHz, s2 is the SEM region at -13dBm/MHz, s1 is the SEM region at -10dBm/MHz and s0 is the SEM limit in the first MHz. Finally, fl means the limit is due to the measurement noise floor (equivalent of a -4dB MPR or 4dB boost).

  - The source column is the source of the emissions: floor is for the noise floor of the setup (only when there is already margin a Pmax), i3/i5 is for IMD3/5 respectively of the allocations in each CC.

  - Finally, the PSD column provides the PSD imbalance between the two CCs at the output (while the input waveforms are with equal PSD or +/6dB PSD imbalance)

- The last three columns are the DFT-s-OFDM PC1.5 with equal PSD measurement results with the same parameters.

- For the tables including PC1.5 with PSD imbalance on top of PC1.5 with equal PSD, the next twelve columns follow the same pattern as the previous PC1.5 with equal PSD six columns.

  - The last four columns show the difference between equal PSD Back-off and the +6dB and -6dB PSD imbalance back-off. A negative value means that PC1.5 with equal PSD has lower back-off than its 6dB imbalance counterpart.

- For the tables including PC2 on top of PC1.5 with equal PSD, the next six columns follow the same pattern than the previous PC1.5 with equal PSD six columns.

  - The last two columns show the difference between PC2 and PC1.5 back-off. A negative value means that PC1.5 with equal PSD has lower back-off than its PC2 counterpart.

Table A 1: Contiguous allocations measurements for PC1.5 +/-6dB PSD imbalance (25/9 allocations measured)

Table A 2: Contiguous allocations measurements for PC1.5 and PC2 with equal PSD (9 allocations compared)

Table A 3: Non-contiguous allocations measurements for PC1.5 +/-6dB PSD imbalance (153/68 allocations measured)

Table A 4: Non-contiguous allocations measurements for PC1.5 and PC2 with equal PSD (52 allocations compared)