3GPP TSG-RAN WG4 Meeting #114bis	R4-2504391

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	6.14.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN B252 UE RF requirements

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for IoT-NTN B252 UE RF requirements is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable.

2.2		Previous Meeting Agreements

In RAN4#114, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

Issue 1-1-1: UE to UE Coexistence with band n2/n25 Agreement: - Agree to two different additional spurious emission requirements, each associated to an NS value, and corresponding A-MPR, for protection of band b25/n25/b2/n2 - -30 dBm/MHz (NS_XX) - -40 dBm/MHz (NS_YY) Issue 1-2-4: UE Out-of-band Blocking - Agreement: - Further discuss the Out-of-Band Blocking requirements for band n252 based on the options following - Option 1, with no co-banding assumption with n256 and n65 Table 2.2-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; -30; -15. n252; Finterferer (CW); MHz; -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750. n2542; Finterferer (CW); MHz; -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750. n255; Finterferer (CW); MHz; -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750. n2561; Finterferer (CW); MHz; -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750. NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void. - Option 2 with assumption of n252 co-banding with n256 and n65 Table 1: Out of-band blocking for n252 Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; -30; -15. n2521; Finterferer (CW); MHz; -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750. n2561; Finterferer (CW); MHz; -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750. NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations. - Option3: reuse the OOB requirements of band B23 Issue 2-1-2: UE Out of band Blocking for Cat M1 - Agreement - Aligned with the assumptions agreed for NR. Issue 2-1-3: UE Out of band Blocking for Cat NB1/NB2 - Agreement: - Aligned with the assumptions agreed for NR.

2.3		UE RF requirements

2.3.1	A-MPR for UE B252 Coexistence with B2/B25

Proposal 1: Regarding -30dBm/MHz (NS_XX) or -40dBm/MHz (NS_YY) for protecting B2/B25, consider that B252 TX A-MPR is unnecessary as shown below.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_XX | 6.5A.4.4.6 | 252 | Table 5.3A-1 | N/A |
| NS_YY | 6.5A.4.4.6 | 252 | Table 5.3A-1 | N/A |

Table 6.5A.4.4.6-1: Additional requirements for "NS_XX"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| B25, B2 | -30 | 1MHz |

Table 6.5A.4.4.6-2: Additional requirements for "NS_YY"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| B25, B2 | -40 | 1MHz |

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_XX | 6.5B.4.4.6 | 252 | N/A |
| NS_YY | 6.5B.4.4.6 | 252 | N/A |

Table 6.5B.4.4.6-1: Additional requirements for "NS_XX"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| B25, B2 | -30 | 1MHz |

Table 6.5B.4.4.6-2: Additional requirements for "NS_YY"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| B25, B2 | -40 | 1MHz |

2.3.2	Out-of-band Blocking for B252 for Cat-M1 and Cat-NB1/NB2

Currently, the new duplexer filter could be used for only supporting B256 and B252.

Observation 1: The provided duplexer filter could be used for both IoT-NTN B256 and B252 without supporting TN B65.

Proposal 2: Before further discussing the co-banding duplexer covering B65, B256 and B252, consider to study/check the duplexer response.

Proposal 3: If there is no duplexer response provided for covering B65, B256 and B252, further discuss out-of-band blocking requirements below for B252 based on obtainable duplexer response.

- Option1:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

- Option2:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 252 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

3 Conclusion

In this paper, our views on IoT-NTN B252 UE RF requirements are provided.

Proposal 1: Regarding -30dBm/MHz (NS_XX) or -40dBm/MHz (NS_YY) for protecting B2/B25, consider that B252 TX A-MPR is unnecessary as shown below.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_XX | 6.5A.4.4.6 | 252 | Table 5.3A-1 | N/A |
| NS_YY | 6.5A.4.4.6 | 252 | Table 5.3A-1 | N/A |

Table 6.5A.4.4.6-1: Additional requirements for "NS_XX"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| B25, B2 | -30 | 1MHz |

Table 6.5A.4.4.6-2: Additional requirements for "NS_YY"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| B25, B2 | -40 | 1MHz |

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_XX | 6.5B.4.4.6 | 252 | N/A |
| NS_YY | 6.5B.4.4.6 | 252 | N/A |

Table 6.5B.4.4.6-1: Additional requirements for "NS_XX"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| B25, B2 | -30 | 1MHz |

Table 6.5B.4.4.6-2: Additional requirements for "NS_YY"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| B25, B2 | -40 | 1MHz |

Observation 1: The provided duplexer filter could be used for both IoT-NTN B256 and B252 without supporting TN B65.

Proposal 2: Before further discussing the co-banding duplexer covering B65, B256 and B252, consider to study/check the duplexer response.

Proposal 3: If there is no duplexer response provided for covering B65, B256 and B252, further discuss out-of-band blocking requirements below for B252 based on obtainable duplexer response.

- Option1:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

- Option2:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 252 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

4 Reference

- RP-241661, “New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2502850, “WF on NR and IoT NTN bands”, Aalyria