# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504403

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			RedCap UE RF impact on HD-FDD

Agenda item:		7.29.2.1

Document for:		Approval

# Introduction

In this paper, we present our views on the RedCap UE RF remaining issue.

# Discussion

In WF[1], the remaining issue is below:

## Issue 1-1: PC3 output power for (e)RedCap

Agreement:

  - Rel-18 power boosting is applicable for NR NTN including (e)RedCap

    - No reference sensitivity specification impact (e.g. RSD) from power boosting.

  - No changes to the PC3 tolerance for HD-FDD and FD-FDD compared non-RedCap UE NR NTN

  - FFS on solutions on additional new output power enhancement

    - supported modulation orders

    - potential downselection between HD-FDD and FD-FDD

    - simultaneous support with rel-18 power boosting

    - Background: output power enhancement (e.g., FE IL improvement)

    - Other solutions not precluded

Rel-18 coverage enhancement work item introduced the power boosting feature for PC2 and PC3. It has introduced two capabilities (powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18). The difference of these two capabilities is that whether the spectrum flatness requirements would be relaxed compared to the legacy spectrum flatness requirements. For UE supporting capability of powerBoosting-pi2BPSK-QPSK-r18, the enhanced inner region (reduced RB ranged within the legacy inner region), no need to relax the spectrum flatness while for UE supporting powerBoosting-pi2BPSK-QPSK-Modified-r18, the spectrum flatness is relaxed and tested against the clause 6.4.2.4.2. No matter which capabilities will be supported by Rel-18 NR UE, either legacy RB inner region or enhanced inner region will be applied and outer RB region is excluded in the power boosting feature.  Also the Rel-18 coverage enhancement is generic feature and it is possible for (e) RedCap UE to support based on capabilities reporting.

For UE supporting Rel-18 power boosting capabilities only inner or enhanced inner region apply and outer region is excluded for power boosting.

Rel-18 (e) RedCap UE can support the Rel-18 power boosting feature.

A Rel-19 NTN (e) RedCap UE can also support the Rel-18 power boosting feature as the feature is generic for NR UE as per agreement in WF [1]. Moreover, as the power increase due to the reduction of the insertion loss of the Tx filter, such power boosting is different with Rel-18 power boosting scope where a transparent scheme would be applied, e.g FDSS, peak cancellation, clipping + filtering etc.  Though power increase by reducing the insertion loss of filter can be rendered using the Rel-18 power boosting capabilities, it can also be used independently with the Rel-18 power boosting feature enabled with FDSS. Therefore, a (e)RedCap UE could increase its output power above the power level when such UE also enabling the Rel-18 power boosting feature simultaneously by reducing the filter insertion loss and at the same time retaining the same PA and baseband (e.g FDSS or peak cancellation still apply) if the unwanted emission requirements still be met. Therefore, a new output power enhancement can be supported simultaneously with rel-18 power boosting.

Additional new output power enhancement can be supported simultaneously with rel-18 power boosting.

For example, for a UE supporting Rel-18 power boosting, the boosted power is 1 dB for PC3 and 0.5 dB for PC2. With insertion loss reduction, the power boosting delta relative to the UE nominal power could be stretched to 1+ X dB for PC3 and 0.5 + Y for PC2 , where X dB for PC3 or Y dB for PC2 is due to the insert loss reduction and it could apply to all modulation scheme which is good property compared to the Rel-18 power boosting which only apply to BPSK/QPSK. On top of this, such power increase (X dB for Pc3 and Y dB for PC2) can apply to all RB allocation without limiting to only inner or enhanced inner region, considering the power boosting mechanism could be different for a (e)RedCap UE. A (e)RedCap UE can support power boosting for outer region with a different power boosting capability on top of Rel-18 power boosting capabilities. As power boosting features are optional features, the below options could be possible for a (e) RedCap UE:

- A (e)RedCap UE operating in HD-FDD mode (indicating the halfDuplexFDD-TypeA-RedCap-r17) supports only Rel-18 power boosting capabilities (powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18)

- A (e)RedCap UE operating HD-FDD mode and Rel-19 new power boosting enhancement capability supports only the power boosting by boosting the power with X dB for PC3 and Y dB for PC2.

- A (e)RedCap UE operating HD-FDD mode supports both Rel-18 power boosting enhancement capability and Rel-19 new power enhancement X dB for PC3 and Y dB for PC2 due to insertion loss reduction. The total configured output power could be two options below:

  - Either Rel-18 power boosting (1 dB for PC3 and 0.5 dB for PC2) or Rel-19 new power enhancement (X dB for PC3 and Y dB for PC2) can be configured, not simultaneously.

  - Both Rel-18 power boosting and Rel-19 new power enhancement can be configured simultaneously and the total output power boosting could be 1 + X dB for PC3 and 0.5 + Y for PC2.

If the option 3 above would be possible, it should further discuss whether a tightened ALCR compared to PC3 ACLR should apply.

Discuss the (e) RedCap power boosting with the three options listed above.

A (e)RedCap UE can support the power boosting with X dB for PC3 and Y dB for PC2 with insertion loss reduction and apply to both outer region and inner region, also apply to other modulation than BPSK/QPSK if unwanted emission would be met.

Power boosting due to the insertion loss reduction could apply both FD-FDD and HD-FDD.

There is a need to discuss the applied RB region and modulation scheme though. The Rel-18 power boosting feature only applies to the inner RB region where 0 MPR is applied. As a starting point, this region should be assumed when Rel-18 power boosting and Rel-19 new power enhancement would be enabled with legacy power boosting simultaneously.  There is also a possibility to extend the RB region to outer RB allocation as the transparent scheme applied can reduce the MPR at outer region also. This is illustrated in Figure 1.

By applying the transparent scheme, Rel-19 power boosting enhancement could be extended to outer region when insertion less reduction is enabled simultaneously.

In principle, if a new power enhancement is to be specified for Rel-19 NTN (e) RedCap, either insertion loss reduction or another transparent scheme could be applied. Whether or not to apply the different schemes also depend on the target of the power boosting. In this sense, we think RAN4 can discuss to set a target power boosting value and further discuss if the requirement impact.  For such purpose, we suggest X = Y=0.5 dB for a starting point for Rel-19 new power enhancement.

Target X=Y=0.5 dB for both PC3 and PC2 for Rel-19 new power enhancement.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 02 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 2: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, thereference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Figure 1 : OBO with power boosting for 100MHz BW for different transparent schemes[2]

# Summary

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Observation 1 For UE supporting Rel-18 power boosting capabilities only inner or enhanced inner region apply and outer region is excluded for power boosting.

Observation 2 Rel-18 (e) RedCap UE can support the Rel-18 power boosting feature.

Observation 3 Additional new output power enhancement can be supported simultaneously with rel-18 power boosting.

Observation 4 A (e)RedCap UE can support the power boosting with X dB for PC3 and Y dB for PC2 with insertion loss reduction and apply to both outer region and inner region, also apply to other modulation than BPSK/QPSK if unwanted emission would be met.

Observation 5 By applying the transparent scheme, Rel-19 power boosting enhancement could be extended to outer region when insertion less reduction is enabled simultaneously.

Proposal-1: Discuss the (e) RedCap power boosting with the three options listed above.

Proposal-2: Power boosting due to the insertion loss reduction could apply both FD-FDD and HD-FDD.

Proposal-3: Target X=Y=0.5 dB for both PC3 and PC2 for Rel-19 new power enhancement.

# References

- R4-2502281, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc

- R4-2313475, RF spec impact for transparent MPR reduction scheme, Ericsson