3GPP TSG-RAN WG4 Meeting #114bis	R4-2504408

Wuhan, China, 7th – 11st, April 2025

Source:	Ericsson

Title:	Specification impact on spectrum sharing by multiple RATs

Agenda item:	9.2

Document for:	Discussion

# 1	Introduction

RAN task RAN4 to study the principle of selected specification series in case of spectrum sharing by multiple RATs (related to approval of RP-250611), this paper presents our view on it.

# 2	Discussion

As a basic principle, RAN4 uses MSR specification (37 series) when multiple RATs shall be supported (e.g. UTRA and E-UTRA, E-UTRA and NR, …).

The NB-IoT is regarded as a dedicated RAT and the quoted text in [1]:

Justification

In GERAN#62, a study item on “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things” was approved in GP-140421. The aim was to study both the possibility of evolving current GERAN system and the design of a new access system towards low complexity and low throughput radio access technology to address the requirements of cellular internet of things. The objectives of the study were: improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimised) network architecture.

NB-IoT should then have been specified in a separate TS and the MSR specifications (37 series) should have been used when operating with UTRA. E-UTRA, …

Nevertheless, when NB-IoT is operating in E-UTRA in-band or guard band, the E-UTRA RF Tx requirements still apply (ACLR, OBUE, …), and not the NB-IoT ones: the 2 RATs could then be perceived as a single “merged” E-UTRA RAT. Considering this singularity, to keep the specifications coherent, RAN decided to specify NB-IoT within the E-UTRA specifications (36 series). And this should be considered as an exception when handling MSR configuration.

Observation: Due to the singularity of NB-IoT when operating in E-UTRA in-band or guard band, the NB-IoT requirements were specified in the 36 series.

It should be noted that each RAT, NB-IoT and E-UTRA, has its own RF Rx requirements. Still, to maintain all NB-IoT RF requirements in one specification, NB-IoT Rx requirements have then been specified in the 36 series.

When NB-IoT operating in NR in-band was introduced, RAN4 concluded that there was no impact on the existing NB-IoT RF requirements, only on the NR RF requirements, keeping then NB-IoT RF requirements in the 36 series.

For NTN, as NTN NB-IoT standalone was specified in the 36 series (TS 36.102), it was then making sense to specify the NB-IoT operating in-band requirements in the 36 series, keeping all NB-IoT requirements (standalone and in-band) in one specification.

Proposal: To keep consistent with the TN approach, NTN NB_IoT RF Rx requirements when operating in NTN NR in-band should be specified in the 36 series, i.e. 36.102.

# 3 	Summary

In this paper, we present our view with below observation and proposal:

Observation: Due to the singularity of NB-IoT when operating in E-UTRA in-band or guard band, the NB-IoT requirements were specified in the 36 series.

Proposal: To keep consistent with the TN approach, NTN NB_IoT RF Rx requirements when operating in NTN NR in-band should be specified in the 36 series, i.e. 36.102.

# References

[1] RP-152284, Revised Work Item: Narrowband IoT (NB-IoT), Huawei