3GPP TSG-RAN WG4 Meeting #114bis	R4-2504415

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Other RX requirements for LP-WUR

Agenda item:	7.26.2.3

Document for:	Discussion

Introduction

In this document we present our views on issues regarding other LP-WUR specific requirements. Following agreements were made in the last RAN4 meeting [1].

# Topic #3: Other RF requirements ### Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF Issue 3-1-1: IBB, OBB requirements Way forward: - Same interference level as MR - FFS at least for one set of Rx requirements. - FFS on wanted signal in blocking test. Issue 3-1-2: detailed table for test parameters of IBB and OBB test case Way forward: - Encourage companies to provide input and analysis on IBB and OBB requirements ### Sub-topic 3-2 Other Rx requirements Issue 3-2-2: Maximum input level requirements for FR1 LP-WUR Way forward: - FFS. Issue 3-2-3: Maximum input level test case for FR1 Way Forward: - Further check and decide whether the max. input power level is specified for all channel bandwidths. Issue 3-2-4: spurious response requirements for FR2 Way Forward - Further check next meeting on whether need to specify spurious response requirements for FR2 LP-WUR.

Discussion

Maximum input level

LP-WUR and main receiver (MR) will observe the same radio environment. Thus, for LP-WUR to reliably operate and co-exist with legacy NR signals, it should be able to handle large input signals.

The value of maximum input level depends on the maximum gNB output power and the minimum coupling loss (MCL) between the gNB and the UE. The maximum gNB output power has not changed and is same for LP-WUS and NR signal. Further, there is no change on the assumptions regarding MCL. Thus, the value of maximum input level for MR can be used for LP-WUR.

There hasn’t been any change in assumptions regarding MCL and gNB output power.

For the main receiver, the requirement power level varies based on channel bandwidth and modulation type. Further, it is a single value of -25 dBm for channel bandwidths of 3, 5, 10, 15, and 20 MHz, for 64 QAM modulation. There is no impact of SCS on the requirement. As possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz, a single value is sufficient and there is no need to scale the current value. Given the LP_WUS signal generated by the gNB is a new one and not present in the legacy specification, so the test signal has to be defined.

Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

SCS has no impact on the requirement.

A new test signal has to be defined for testing the maximum input level requirement.

No need to scale-down the requirement based on LP-WUS RBs.

Use maximum input level of -25 dBm for LP-WUR.

Define a new test signal and side conditions for the maximum input level requirement.

Blocking requirements

Blocking characteristics are measure of the receiver ability to receive the wanted signal in presence of interference on frequencies other than its designated frequencies. Given LP-WUR will observe the same radio environment as MR, it should be able to withstand interferer with similar level as of MR. Thus, the interferer power levels, and configuration should be the same as of MR.

Reuse Table 7.6.2-2, 7.6.2-4, 7.6.3-2, 7.6.3-4 for in-band and out-of-band blocking levels for LP-WUR.

It is known that dynamic range of the LP-WUR will be smaller than the dynamic range of main radio. Thus, it is possible to relax the desired signal level in blocking test cases. There shall be no extra relaxation for the UEs that target ultra-low power consumptions as the power consumption target is a UE implementation issue.

Agree to use larger signal power in blocking test cases compared to MR blocking test cases.

UE power saving target shall have no impact on the blocking requirements.

Spurious emission

Spurious emissions are the narrow band CW emissions appearing at the UE antenna connector. Receiver spurious emissions mainly come from the LO in the receiver. As illustrated in Figure 1 marked with the green line can the LO leakage end at the antenna or antenna connector. The main LO frequency will dominate in-band spurious emissions, and the harmonic will dominate out-band spurious emissions. In a design with potential low isolation from LO to RF input an LNA will be required to attenuate the in-band spurious and a band pass filter to ensure out-band spurious is suppressed. Given that the current requirements are regulatory in nature, they should apply to the LP-WUR.

Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

Figure 1Illustration of a DC receiver.

Conclusion

The following Observations and Proposals were made:

Observation 1: There hasn’t been any change in assumptions regarding MCL and gNB output power.

Observation 2: Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

Observation 3: SCS has no impact on the requirement.

Observation 4: A new test signal has to be defined for testing the maximum input level requirement.

Proposal 1: No need to scale-down the requirement based on LP-WUS RBs.

Proposal 2: Use maximum input level of -25 dBm for LP-WUR.

Proposal 3: Define a new test signal and side conditions for the maximum input level requirement.

Proposal 4: Reuse Table 7.6.2-2, 7.6.2-4, 7.6.3-2, 7.6.3-4 for in-band and out-of-band blocking levels for LP-WUR.

Proposal 5: Agree to use larger signal power in blocking test cases compared to MR blocking test cases.

Proposal 6: UE power saving target shall have no impact on the blocking requirements.

Proposal 7: Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

- References

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.