3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504431

Wuhan, China, 7 – 11th April 2025

Source:	BT, Ericsson

Title:	PMI statistics for measured MIMO channels in band n78

Agenda item:	7.16.1

Document for:	Discussion

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN4#112, RAN4#112b, RAN4#113 and RAN4#114 approved way forward agreements [2-5] respectively.

In this contribution we provide observations of spatial characteristics of measured MIMO channels from typical deployments, focusing on the spatial preferences of channels. The purpose of this contribution is to support the group with identification of important properties of typical propagation environments to be reflected as close as reasonably possible in a spatial channel model.

This contribution presents an empirical analysis of PMI statistics based on measurement data. The analysis provides insights into expected spatial preference of a UE in n78 band sample deployment scenarios.  Other observations of basic properties of MIMO channels in sample typical deployments are provided in [6, 7, 8] focusing on 2100 and 2600 MHz bands.

Field measurements from typical deployments

Measurement setup and scope

Uplink sounding reference signals (SRS) were used for evaluation of respective MIMO channel between a typical commercial MIMO base station and a UE.  Measurements were conducted using a 40MHz channel in n78 band (3680 – 3720 MHz) in four locations A to D, as shown in Figure 1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup. The output of the measurements was reconstructed into channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. The H matrices were used in an in-house simulator to derive the precoder matrix indicator (PMI) statistics for four locations A to D.

Locations A and C can be characterised as having a dominant line-of-sight component, yet with several reflections from surrounding buildings. Locations B and D can be characterised as non-line-of-sight due to obstructions from surrounding buildings. However, the LoS or NLoS determination is indicative. The determination is based on analysis of power delay profiles from previous test campaigns utilising same locations. Sample typical PDP for downlink direction for Location A is provided in Figure 3 of [7].

The SCM should have sufficient flexibility to accurately capture the observed behavior, ensuring a realistic representation of multipath propagation and MIMO channel characteristics.

PMI statistics

- Figure 2 presents the PMI statistics for four locations. This analysis aims to determine if there is a spatial preference experienced by a UE, as indicated by the PMI values in a channel state information (CSI) report. We input the time samples of the channel matrix from the first subband (captured by the SRS) into an in-house simulator. The simulator processes each H matrix to generate an NR 3GPP Release-15 Type-1 report. From this report, we extract the port-to-beam mapping (represented by PMI1) and the beam-to-layer mapping (represented by PMI2). The actual precoder for the PDSCH is the product of PMI1 and PMI2.

Figure 1. Measurement point locations for the sample MIMO channels.

(a) Location A.

(b) Location B.

(c): Location C.

(d) Location D.

Figure 2: PMI Statistics Across Four Locations Highlighting Spatial Preferences in UE.

- The results indicate that the UE prefers the highest rank (rank 4) for transmission among the PMI hypotheses [1,2,3,4]. For a rank of 4, there are 512 possible PMI1 hypotheses and two corresponding PMI2 hypotheses for each PMI1. The histograms reveal that the UE reports a subset of PMI1s, with one particular PMI being reported significantly more frequently than others. This suggests a spatial preference consistent with the PMI statistics shown in Figure 17 of [9]. Furthermore, this observation is consistent with the Angle of Arrival (AoA) analysis detailed in [8], where the primary AoA remained constant over time.

Conclusion

Observations and proposals for the SCM are summarised below.

Observation 1: The UE's preference for a specific PMI subset is observed in measured MIMO channels from typical deployments.

Adopt the expected PMI statistic behaviour as a metric for assessment of channel model candidates in the SCM study.

- Acknowledgements

The measurements and data processing for this contribution have been conducted by Ericsson Research UK and BT Research teams under UK DSIT REASON project and UKRI Future Leaders Fellowship (MR/Y020065/1).

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.

- R4-2502378, Way Forward [114][322] NR_SCM, RAN4#114, Feb 2025.

- R4-2411557, “Scenarios and requirements for spatial channel modelling”, BT, RAN4#112, Aug 2024.

- R4-2415673, “Further observations from deployment and MU-MIMO use cases for SCM evaluation”, BT, Orange, RAN4#112b, Oct 2024.

- R4-2419338, “Further observations on the properties of MIMO channels in typical deployments”, BT, Ericsson, RAN4#113, Nov 2024.

- R4-2418043, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113, Nov 2024.