3GPP TSG-RAN WG4 Meeting #114bis	R4-2504439

Wuhan, China, April 7th – 11th 2025

Agenda Item:	7.20.2

Source:	Ericsson

Title:	2-sided CSI modelling

Document for:	Discussion

# 1	Introduction

During RAN4#114, first simulation results were presented for the study on development of a test solution based on a frozen/standardized test decoder (option 3). The results were generated based on exchange of datasets and models between companies. With these initial results, a number of interesting observations could be made:

- It was possible for companies to upload encoders/decoders using ONNX and for other companies to download and use the models for training and inference.

- For track 1 (frozen decoder trained using company’s own dataset), with at least one frozen decoder, several companies could train their own encoder and observe comparable results.

- However, in two other cases of frozen decoder, other companies were not able to train their own encoders and obtain consistent results. This was most likely due to differences between the datasets of the companies that generated the frozen decoders.

- For track 2, with a mixed datasets, companies took the first step of training an encoder and decoder. There was pretty close alignment between results using the mixed dataset.

- It was observed that 5 companies datasets seemed to produce similar results, whereas 2 datasets seemed to have statistical differences to the others.

These initial results provide an initial demonstration that exchange of models and use of a frozen “test” encoder may be viable.

In order to progress further with the investigation, several further steps were agreed:

- For both track 1 and track 2, an explicit quantization and dimension for the output of the encoder and input of the decoder were agreed.

- For track 1, companies should submit new models based on the agreements and adjustments to datasets.

- For track 2, a new mixed dataset was agreed, and companies were invited to provide new encoder/decoder pairs, such that a frozen decoder could be selected to use for further analysis towards this meeting.

In addition, an agreement to provide a reference model for option 3 was agreed, although the details remained FFS.

# 2	Option 3

## 2.1	Track 1 updated results

### 2.1.1	Inference of Track 1 models with cross testing datasets

Post RAN4 114 meeting, companies are submitting models for Track 1 (where the model is trained using respected companies’ datasets). In the following tables, we summarize the inference of the submitted models. Table 1 is for the unquantized models and Table 2 is for quantized models. The individual test sets are obtained from the test set collected for Track 2 study. Note that in the tables, the Mediatek models are not tested against the Mediatek test set, and the Nokia models are not tested against the Nokia test set. This is because it is not clear whether the test set used for Track 2 study is included as a training dataset for Track 1 model generation.

Table 1	Inference of unquantized models for Track 1 study, tested with test set for Track 2 study.

| Test set | Ericsson | CATT | Huawei | Mediatek | Nokia | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
| Ericsson | 0.7591 | 0.7095 | 0.5251 | 0.6348 | 0.7356 | 0.7257 |
| Mediatek | 0.6335 | 0.5597 | 0.3967 |  | 0.6489 | 0.6510 |
| Nokia | 0.7596 | 0.7161 | 0.5507 | 0.6612 |  | 0.7496 |
| Oppo | 0.7636 | 0.7237 | 0.5721 | 0.6710 | 0.7613 | 0.7589 |
| Vivo | 0.7375 | 0.6871 | 0.5046 | 0.6297 | 0.7296 | 0.7202 |

Table 2	Inference of quantized models for Track 1 study, tested with test set for Track 2 study.

| Test set | Ericsson | CATT | Huawei | Mediatek | Nokia | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
| Ericsson | 0.6933 | 0.6614 | 0.5327 | 0.5703 | 0.6886 | 0.6768 |
| Mediatek | 0.5701 | 0.4978 | 0.4319 |  | 0.5771 | 0.5646 |
| Nokia | 0.7018 | 0.6676 | 0.5621 | 0.5829 |  | 0.7009 |
| Oppo | 0.7107 | 0.6792 | 0.5842 | 0.5943 | 0.7173 | 0.7174 |
| Vivo | 0.6728 | 0.6367 | 0.5192 | 0.5603 | 0.6766 | 0.6664 |

From the above tables we can observe some differences on the SGCS performance when a Track 1 model is tested with dataset from various companies. Vice versa, there are also differences on the SGCS performances when a testing dataset is used to test various models. This implies that the test set and the training dataset used to train the models are having different characteristics/distribution. The dataset from Mediatek, in particular, may be quite different than other companies’ datasets. Tested with Mediatek test set, models from companies perform noticeably lower compared to being tested with other testing datasets. Mediatek Track 1 model also performs relatively low when being tested with other companies’ test set compared to other models (except Huawei). This aspect shall be considered if the conclusion of the feasibility study on the inter-operability shall be drawn. This shall also be considered in determining, if needed, the reference/standardized datasets that shall be used for model training or testing.It is particularly interesting to note here that the datasets are all based on the same simulation assumptions, however still an alignment effort is needed; this implies strong alignment will be needed in a WI.

For track 1, there is variation in performance between different companies that depends on the training set and the test set.

### 2.1.2	Training an encoder with Ericsson dataset and Samsung decoder

For this evaluation, we take Samsung Track 1 (both the unquantized and the quantized models) to train an encoder with Ericsson’s dataset. An encoder-decoder pair (Encoder 0 – Decoder 0) trained jointly (from scratch) with Ericsson dataset is given for a comparison. The test set is the Ericsson’s test set. The loss is calculated compared to the Encoder 0 – Decoder 0 pair. The evaluation results can be seen in the following table.

Table 3	Evaluation results for Track 1 with Samsung Track 1 decoder

| Encoder | Decoder | Unquantized |  | Quantized |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Loss | SGCS | Loss |  |
| Encoder 0 | Decoder 0 | 0.7591 |  | 0.6933 |  | Encoder 0 and Decoder 0 are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.7229 | -4.8% | 0.6752 | -2.6% | Encoder 1 and Decoder 1 are jointly trained with Samsung dataset |
| Encoder 2 | Decoder 1 | 0.7238 | -4.6% | 0.6765 | -2.4% | Encoder 2 is trained with Decoder 1 using Ericsson dataset |

From the above table, we can observe that training with a nominal/reference decoder is feasible. Note that the results shall only represent results for a feasibility study. The loss/gain may, if needed, may require further study. For example, the gain/loss of training an encoder with a new dataset may depend on how differ the original dataset and the new dataset, etc.

We also note that in this example, when an encoder is trained on the Ericsson dataset but using the Samsung decoder, the SGCS did not improve back to the level of a jointly trained Ericsson encoder/decoder on the Ericsson dataset.

Based on the Samsung decoder, option 3 track 1 seems basically feasible, at least where companies have similar datasets.

## 2.2	Track 2 updated results

### 2.2.1	General performance for Track 2

In Track 2, a mixed dataset is created from several companies. For this meeting, a dataset from 5 companies is being used (100K samples from each companies). The dataset is divided into 400K samples of training dataset, 50K samples for validation dataset, and 50K samples for testing dataset. We present, for the Ericsson model, both the mean SGCS across all 50K samples (labelled as mixed in the table) and across each company 10K samples (labelled with Company’s name). The result of training with mixed dataset are presented in the below table. The results of Track 2 from the previous meeting (700K samples) are also presented for comparison. Table 4 shows the results for the unquantized model while Table 5 shows the results for the quantized model. Note that for the RAN4 #113 models, there is no inverse sigmoid in the quantized models.

Table 4	SGCS for Track 2 unquantized Ericsson model

| Test set | RAN4 #114 |  | RAN4 #113 |  |
| --- | --- | --- | --- | --- |
|  | SGCS | Gap to the mixed test set | SGCS | Gap to the mixed test set |
| Mixed | 0.7561 |  | 0.7460 |  |
| Ericsson | 0.7463 | -1.3% | 0.7203 | -3.4% |
| Mediatek | 0.7753 | 2.5% | 0.7616 | 2.1% |
| Nokia | 0.7571 | 0.1% | 0.7425 | -0.5% |
| Oppo | 0.7633 | 1.0% | 0.7523 | 0.8% |
| Vivo | 0.7382 | -2.4% | 0.7169 | -3.9% |
| CATT |  |  | 0.6984 | -6.4% |
| Qualcomm |  |  | 0.8299 | 11.2% |

Table 5	SGCS for Track 2 quantized Ericsson model

| Test set | RAN4 #114 |  | RAN4 #113 |  |
| --- | --- | --- | --- | --- |
|  | SGCS | Gap to the mixed test set | SGCS | Gap to the mixed test set |
| Mixed | 0.6936 |  | 0.6932 |  |
| Ericsson | 0.6789 | -2.1% | 0.6679 | -3.6% |
| Mediatek | 0.7119 | 2.6% | 0.7041 | 1.6% |
| Nokia | 0.6961 | 0.4% | 0.6897 | -0.5% |
| Oppo | 0.7122 | 2.7% | 0.7063 | 1.9% |
| Vivo | 0.6691 | -3.5% | 0.6590 | -4.9% |
| CATT |  |  | 0.6528 | -5.8% |
| Qualcomm |  |  | 0.7728 | 11.5% |

From the above table, we can observe that the performance of Track 2 under the current dataset setup (500K) is, in average, equal or better than the previous dataset setup (700K). This is likely due to the elimination of Qualcomm and CATT dataset which were quite different from the other dataset. The ML training is simplified as the dataset are more similar to each other, but the performance is also reduced because the Qualcomm test dataset was “easy” compared to the other.

From a per-company testing dataset point of view, however, the performance of the Track 2 models with the current dataset setup is generally better than the previous setup. From our evaluation, this is caused from the fact that the mean value with the previous dataset setup is somewhat skewed by Qualcomm dataset with a very high SGCS, causing the mean SGCS value across all samples to be dragged up.

From the variance of the SGCS value, we can observe a range of mean SGCS value of 4.9% and 6.2% for the unquantized model and the quantized model, respectively. While the range is noticeably lower compared to the previous dataset setup, this range shows that agreement/discussion on the dataset is crucial, in particular for the discussion related to test set that will be used for the conformance test and, if supported, the discussion on the standardized reference models.

The variation between the mixed testset performance and individual companies testsets for track 2 has decreased but is still significant.

### 2.2.2	Model inference for companies’ submitted models

In the below tables, we present the mean SGCS value for the inference of the submitted ONNX models. Similar to the above tables, we present both the mean SGCS across all 50K samples (labelled as mixed in the table) and across each company 10K samples (labelled with Company’s name). Table 6 shows the results for the unquantized model while Table 7 shows the results for the quantized model.

Table 6	SGCS for Track 2 unquantized models submitted by companies

| Test set | ERIC | CATT | HW | MTK | NOK | SAM | VIVO | Mean |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mixed | 0.7561 | 0.7726 | 0.7536 | 0.7389 | 0.7572 | 0.7532 | 0.7522 | 0.7548 |
| Ericsson | 0.7463 | 0.7601 | 0.7421 | 0.7274 | 0.7460 | 0.7392 | 0.7397 | 0.7430 |
| Mediatek | 0.7753 | 0.7951 | 0.7738 | 0.7561 | 0.7774 | 0.7762 | 0.7728 | 0.7752 |
| Nokia | 0.7571 | 0.7739 | 0.7556 | 0.7410 | 0.7576 | 0.7554 | 0.7545 | 0.7564 |
| Oppo | 0.7633 | 0.7765 | 0.7621 | 0.7509 | 0.7658 | 0.7628 | 0.7611 | 0.7632 |
| Vivo | 0.7382 | 0.7573 | 0.7345 | 0.7193 | 0.7390 | 0.7323 | 0.7328 | 0.7362 |

Table 7	SGCS for Track 2 quantized models submitted by companies

| Test set | ERIC | CATT | HW | MTK | NOK | SAM | VIVO | Mean |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mixed | 0.6936 | 0.7171 | 0.7050 | 0.6949 | 0.6979 | 0.7018 | 0.6962 | 0.7009 |
| Ericsson | 0.6789 | 0.7058 | 0.6928 | 0.6788 | 0.6845 | 0.6882 | 0.6821 | 0.6873 |
| Mediatek | 0.7119 | 0.7337 | 0.7218 | 0.7141 | 0.7147 | 0.7193 | 0.7128 | 0.7183 |
| Nokia | 0.6961 | 0.7201 | 0.7068 | 0.6974 | 0.6999 | 0.7042 | 0.6985 | 0.7033 |
| Oppo | 0.7122 | 0.7275 | 0.7197 | 0.7139 | 0.7149 | 0.7183 | 0.7140 | 0.7172 |
| Vivo | 0.6691 | 0.6986 | 0.6839 | 0.6705 | 0.6755 | 0.6791 | 0.6737 | 0.6782 |

The above results can be seen in at least from two point of views, i.e., the performance variation due to the model training (first row) and performance variation due to the test set (last column).

From the first row of the unquantized models, we can observe that compared to the average value across different models (0.7548), the lowest performance (0.7389) and the highest performance (0.7726), has a gap to the average value of -2.1% and 2.4%, accordingly. For the quantized models, compared to the average value across different models, (0.7009), the lowest performance (0.6936) and the highest performance (0.7171), has a gap to the average value of -2.5% and 2.7%, accordingly.

From the last column of the unquantized models, we can observe that compared to the average value across different test set (0.7548) the test set that gives the lowest performance across models (0.7362) and the test set that gives the highest performance across models (0.7752), has a gap of -2.5% and 2.7% accordingly. For the quantized models, compared to the average value across different test set (0.7009) the test set that gives the lowest performance across models (0.6782) and the test set that gives the highest performance across models (0.7183), has a gap of -3.2% and 2.5% accordingly.

The difference on the results shown for different test set (i.e., by comparing the rows of a column) is caused by different dataset distribution/characteristic (although the dataset is generated under the same simulation assumption). One possible way to see the dataset characteristic is by observing the similarity across subbands of the eigenvector. In the below figure, the similarity (SGCS) between all subbands and the subband index 6 is plotted.

Figure 1	SGCS between all subbands with subband index 6.

From the above figure, we can observe that there is some correlation between the similarity across subbands of the dataset with the model performance. Seeing the last column of Table 6 and Table 7, for example, Vivo dataset has the lowest similarity and thus, such dataset results in the lowest average performance across models. Similarly, Mediatek dataset has the highest similarity and thus, such dataset results in the highest average performance across models.

In general, however, the datasets and models using the current Track 2 dataset are generally quite aligned. The experience and knowledge obtained from Track 2 study generally confirm the importance of having discussion/alignment on the datasets.

Variations between different companies datasets (and the mixed dataset) appears to be related to differing levels of correlation between the SGCS observed for different sub-bands.

### 2.2.3	Evaluation with Samsung Track 2 decoder

In the below, we conduct evaluations similar to Track 1 but using Track 2 decoders. We consider two scenarios. In the first scenario, an encoder is trained with Samsung Track 2 decoder using the mixed dataset and tested with mixed dataset while in the second scenario, an encoder is trained with Samsung Track 2 decoder using the Ericsson dataset and tested with Ericsson dataset.

The results for the first scenario can be seen in the following table.

| Encoder | Decoder | Unquantized |  | Quantized |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Loss | SGCS | Loss |  |
| Encoder 1 | Decoder 1 | 0.7532 |  | 0.7018 |  | Encoder 1 and Decoder 1 are Samsung Track 2 models |
| Encoder 2 | Decoder 1 | 0.7518 | -0.2% | 0.7000 | -0.3% | Encoder 2 is trained with Decoder 1 and using mixed dataset |

The results for the second scenario can be seen in the following table. In the table, the results of using Ericsson Track 1 models are given as a reference.

| Encoder | Decoder | Unquantized |  | Quantized |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Loss | SGCS | Loss |  |
| Encoder 0 | Decoder 0 | 0.7591 |  | 0.6933 |  | Encoder 0 and Decoder 0 are Ericsson Track 1 models |
| Encoder 1 | Decoder 1 | 0.7367 | -3.0% | 0.6857 | -1.1% | Encoder 1 and Decoder 1 are Samsung Track 2 models |
| Encoder 2 | Decoder 1 | 0.7402 | -2.5% | 0.6867 | -1.0% | Encoder 2 is trained with Decoder 1 using Ericsson dataset |

From the above tables, we can observe that training an encoder with a test decoder is generally feasible and the degradation are very small if the encoder is trained on the same dataset as the test decoder. The dataset used to train the decoder (mixed dataset), and the dataset used to train the encoder (mixed dataset or Ericsson dataset) has similar distribution/characteristic. I.e., from the previous section, it is observed that average performance wise, Ericsson dataset only has 1.6% difference and 1.9% difference for the unquantized models and the quantized models, respectively, compared to the mixed dataset.

Based on working with the Samsung decoder as a frozen decoder, option 3 track 2 works very well if the UE vendor encoder is trained with the same mixed dataset as the test decoder. If trained with a different dataset, there is a small difference in performance.

## 2.3	Other considerations

Option 3 is standardization of a test decoder in the 3GPP specifications. The current simulation work is related to assessing whether it is realistic, at least in a simulation environment, to fix a test decoder and then for companies to be able to create their own encoders with satisfactory performance. The current simulation activity, considering the viability of option 3, can both provide an initial view on the viability of option 3 as well as a learning curve on how a real standardization activity could proceed. Apart from the simulation activity, a number of other considerations arise when

Reference encoder

RAN4#114 reached the following conclusion on standardization of a reference encoder:

- To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4

  - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part

  - FFS on criterion for selection of reference encoder

  - FFS on decision whether RAN4 or RAN5 will specify it

In our view, it is important and useful to capture a reference encoder with the option 3 for the following reasons:

- For verification of the correct operation of the test decoder on test equipment

- As a useful reference for network vendors to develop decoders

- It may also be used by UE vendors; in the most extreme case if they prefer not to optimize the encoder then the reference encoder can be used and should be capable of meeting RAN4 requirements if operated correctly.

Regarding the open issues in the previous meeting agreement, our view is as follows. Regarding capturing in a TS or TR, it is important to bear in mind that unless the TR is an external TR, a TR will not be updated to new specification releases. Since the reference encoder is closely linked to the test decoder, in our view it should be maintained together with the test decoder. For this purpose, it would be preferable to capture the encoder as part of a TS.

However, clearly AI models will not directly be part of a TS in the sense of being captured within a word document. Presumably, AI models will need to be stored and maintained in a specific location and referred to from the TS or similar. If this is the case, then it may be suitable to store the encoder at the same managed location as the decoder.

If there is a need for it to be part of a TS or TR, then preferably the reference encoder should be part of a TS. However, it may be suitable that it is stored in the same managed location as the test decoder.

The second open FFS is the criterion for the selection of the reference encoder. In our view, the reference encoder should be the paired encoder/test decoder which are trained together using the same training dataset.

The reference encoder should be the encoder that is paired with, i.e. trained together with, the test decoder using the same training data.

The third FFS is on whether the reference encoder should be captured in RAN4 or RAN5 specifications. Since one of the major purposes of the reference encoder is to verify test equipment, there could be some argument for inclusion in the RAN5 specification. However, it is of great usefulness to keep the paired test decoder and reference encoder together in the same specification (i.e., RAN4). As described above, it may anyhow be the case that models are stored and maintained at a specific location and can be referred to from the specifications.

Keep the test decoder and reference encoder together, in the RAN4 specifications. Or alternatively store and maintain the models together in the same place and refer from the RAN4/5 specifications.

Training, verification and test datasets.

These datasets are defined as follows:

Training dataset: The dataset used to train the test decoder

Verification dataset: The dataset used by the TE vendor to verify that the decoder in the TE performs properly

Test dataset: The dataset used during UE conformance tests.

There was some discussion, but no agreement on standardizing or at least storing the training dataset used to develop the test decoder for option 3. An associated question is whether to standardize or store a verification dataset and the test dataset.

During the current trial, the training dataset has been based on simulation results and made available by contributing companies. If during a real WI a training set would be based on simulation (“synthetic data”) then it could be shared in a similar manner. For future development of encoders and decoders it would be useful to keep such a training dataset available. The dataset need not necessarily be normatively standardized, but it should be kept somewhere where it is clearly associated to the test decoder and reference encoder and can be maintained across releases.

If available, store the dataset used to create the test decoder such that it is linked to the test decoder and reference encoder and maintained across releases.

In case the test decoder is developed using data that cannot be shared, then of course it would not be possible to store the dataset. It may be dubious whether a standardization of the test decoder could be performed on this basis, though, although if it could be done, a “good” decoder without a dataset would be preferable to a “bad” model with a dataset.

Regarding the test dataset, this can be discussed when the requirements are developed. The test could be described by means of a channel model(s), or could be captured as a dataset. The same would be true of a verification dataset.

Discuss during development of the requirement whether to describe test and verification data by means of a channel model or an explicit dataset.

Representation of models in the specifications

For exchanging models during the study phase, ONNX has been used, as this enables swapping of models independently of AI tools used by different companies. This is useful for a study phase, however for permanent capture of a test decoder in the specifications it may not be suitable. ONNX is an open source tool that is subject to variation between versions and that is not under the control of 3GPP. A 3GPP specification needs to be open access and not be subject to becoming invalid due to variations in the platform used for representing the model. The 3GPP specification should also not be tied to a specific AI/ML platform; both because it restricts implementation freedom and also because this would again link the 3GPP specification with a third-party tool.

Thus, there may be a need to develop a 3GPP specification method for a test decoder in the TS, with the ambition to handling other future AI/ML models.

The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed (for CSI compression and future models). Possible means to achieve this may be discussed if time allows.

The 3GPP representation may also need a process for updating of the test decoder. This may be handled by the CR and release process, although some thought may be needed on managing backward compatibility. For example, if encoders would not be directly compatible with an updated decoder, then a mismatch could occur between the version of a decoder assumed by the network and the version against which UE encoders have been trained; such mismatches should be avoided or handled by signalling.

The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed.

If it is agreed to capture the dataset associated with the test decoder then there will also be a need to agree on how to capture the dataset. This is a significantly less complex task than capturing a model, however; in principle the dataset format agreed as part of the SI has proved successful and could be used.

Identification of a model structure

As part of the process of deciding on the decoder during standardization, for the simulation exercise some arbitrary assumptions were made on the decoder structure in order to obtain an alignment to check the decoder development process using simulation. During a real standardization, there would be a need to spend time and detail on agreeing the decoder structure. The starting point would be the backbone, scale and complexity of the overall model. Some overall goals would need to be agreed, such as performance target, complexity target, number of models, degree of generalizability over a wide test-set. There may also be a need to consider the extent to which different types of test decoder backbone and structure could support variation in encoder and in decoder implementations. It might be expected that a large amount of time might be needed for this discussion.

For the current study phase, it may be useful to discuss around what characteristics may be of importance when deciding on a real decoder structure. Some candidates for discussion are outlined in proposal 8.

The study should consider what key characteristics should drive the discussion on structure for a real standardization exercise. Some characteristics include:

- Backbone (CNN, transformer etc.)

- Complexity target

- Number of models (depending on scenarios etc.)

- A metric and acceptable performance target

- Ability of the agreed structure for the test decoder to allow for variation of real encoder/decoder implementation

It should be noted that a discussion is ongoing in RAN1 relating to model specification options. Although the purpose of the RAN1 discussion is different (inter-vendor collaboration rather than standardization of a test decoder), a discussion in RAN4 on test decoder structure may do well to co-ordinate with the RAN1 discussions and conclusions, in particular considering that it may be the case that aligning the test decoder structure with a structure used for collaboration may be needed.

Conclude that, if Option 3 is supported for performance testing, then the fully specified test decoder is the same as the decoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion.

Identification of a dataset

For the simulation exercise, the dataset has been based on system simulation assumptions adopted by RAN1 during the performance study. Although in principle all of the datasets should be the same due to aligned simulation assumptions, in practice during the first round of simulations some differences between the characteristics of different datasets were observed, potentially due to different amplitude/phase normalization.

If a real test decoder and RAN4 testing is devised, it is important that it is based on a dataset that is realistic for the range of real-world situations that the model could encounter. The reason for this is that the test decoder will capture an available latent space that can be used by encoders. Also, RAN4 requirements need to cover a range of situations that ensure that a model can have a reasonable expectancy of working properly in real-world scenarios.

In order to achieve this, in a real work item, there would be a need to identify a dataset that contains a sufficient diversity of data. This is regardless of how many tests RAN4 develop for ensuring generalization; even if there would be no testing of generalization in RAN4 it would be good for the test decoder to capture a latent space that can be expected to be realistic for most deployment scenarios.

One way to create such a dataset would be to accumulate data from real world measurement. However accumulating such data would be non-trivial and very likely not possible within the 3GPP RAN4 context. Alternatively, a sufficient diversity of simulation conditions would need to be agreed. Care would also need to be taken that the simulated datasets with different companies could be aligned.

Conclude that, for a real WI, a sufficient diversity of simulation assumption sets would be needed to make the training dataset for the test decoder. Also, some steps to ensure alignment of different companies’ datasets are needed.

# 3	Option 4

## 3.1	Option 4 results

Another option to resolve inter-vendor training collaboration/interoperability issues is by having a standardized reference encoder. In this evaluation, a reference encoder is first created by joint train the encoder and the decoder with a first dataset to obtain a reference encoder. The reference encoder is then used to train a decoder. The training can be done either by using a first dataset or a second dataset that is different than the first dataset. Note that in this evaluation, although the first dataset and the second datasets are different, they are generated under the same simulation assumption. Further, the trained decoder is used to train another encoder. The training may use the first or the second datasets. The different encoder-decoder pairs obtained from this training sequence is then tested with test sets associated with the first training dataset or the second training dataset. Note that in this evaluation, the same architecture as the agreed architecture is used. Note also that although the evaluation is done to Option 4b, the similar/same results are expected from Option 4a.

### 3.1.1	Evaluation for Encoder obtained from Track 1

In this evaluation, the performance of Samsung Track 1 models (Encoder 1 – Decoder 1 pair) is first evaluated with Ericsson test set. Encoder 1 is then used to train a new decoder (Decoder 2) by using Ericsson training dataset. Further, a new Encoder (Encoder 2) is trained with Decoder 2 by using Ericsson dataset. The performance of Ericsson Track 1 models (Encoder 0 – Decoder 0 pair) is also shown as a comparison.

#### 3.1.1.1	Evaluation with unquantized models

Using the above setup, the results for the unquantized model are as the following.

Table 8	SGCS for Option 4b with Samsung Track 1 Unquantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.7591 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.7229 | -4.8% | Encoder 1 and Decoder 1 are Samsung models for Track 1 |
| Encoder 1 | Decoder 2 | 0.7309 | -3.7% | Decoder 2 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.7334 | -3.4% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |

From the above table, we can observe that training with a reference encoder is feasible. Training the decoder with a training dataset that match with the test set could improve the model performance to some extent. Note, however, in the above result, the training epoch used in the Decoder 2 training is less than 50. This is because there is overfitting happens after such an epoch. One possibility is that the overfitting may happen as the decompression task during a Decoder-only training is quite light. On the other hand, the model used for the Decoder is quite large. In addition, the overfitting may also happens due to the high value of learning rate, etc.

#### 3.1.1.2	Evaluation with quantized models

The similar evaluation to Table 8 is conducted for the quantized models. We evaluate the performance with three different Encoders from Track 1, i.e., the Samsung, Nokia, and Mediatek.

Table 9	SGCS for Option 4b with Samsung Track 1 Quantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.6933 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.6752 | -2.6% | Encoder 1 and Decoder 1 are Samsung models for Track 1 |
| Encoder 1 | Decoder 2 | 0.6697 | -3.4% | Decoder 2 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.6774 | -2.3% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |

Table 10	SGCS for Option 4b with Nokia Track 1 Quantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.6933 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.6856 | -1.1% | Encoder 1 and Decoder 1 are Nokia models for Track 1 |
| Encoder 1 | Decoder 2 | 0.6740 | -2.8% | Decoder 2 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.6845 | -1.3% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |

Table 11	SGCS for Option 4b with Mediatek Track 1 Quantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.6933 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.5674 | -18.2% | Encoder 1 and Decoder 1 are Mediatek models for Track 1 |
| Encoder 1 | Decoder 2 | 0.6723 | -3.0% | Decoder 2 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.6818 | -1.7% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |

From the above tables, we can observe that training with a reference encoder is generally feasible. Similar with the evaluation for unquantized models, the number of epochs used during the Decoder 2 training is less than 50 due to overfitting after such an epoch.

The overfitting problem seems even more severe for the quantized models, i.e., performance degradation is observed if we compare the Encoder 1 – Decoder 2 pair to Encoder 1 – Decoder 1 pair in the Samsung Encoder and Nokia Encoder cases. The overfitting problem seems to be more severe in the quantized models. This may happen due to the fact that in the quantized models, the number of possible combinations of the encoder outputs are smaller compared to the unquantized models, resulting in even more lighter decompression task.

In addition to the above, the performance gain or loss of training an encoder with a reference encoder may depend on the differences between the dataset used to train the reference encoder and the dataset used to train the new decoder (which corresponds to the dataset used for testing). Comparing the Mediatek to Samsung and Nokia results for example, we can observe that for Mediatek Encoder case, we can observe a gain (instead of loss) when training a new decoder based on the reference encoder. This is because the dataset distribution between Mediatek dataset and Ericsson dataset is quite different (as can be seen if we compare the first row and the second row of Table 11 and results in Section 2.1.1.

We note that, when a new decoder is trained on the Ericsson dataset using a fixed encoder, and when a new encoder is trained using the new decoder, with the Samsung and Nokia datasets, the performance cannot recover to the level attained if an encoder/decoder is trained directly on the Ericsson dataset. However, with the MediaTek dataset a very significant improvement in performance is seen compared to the MediaTek enoder/decoder on the Ericsson testset (however still not performing exactly as well as a directly trained Ericsson encoder/decoder).

For option 4, if a frozen encoder is taken from track 1, a new decoder (trained on the frozen encoder) and encoder (trained on the new decoder) can be trained that show a small performance drop compared to the encoder/decoder trained directly.

### 3.1.2	Evaluation for Encoder obtained from Track 2

In this evaluation, an Encoder obtained from Track 2 study is used to train a new decoder. The decoder obtained from this training is then used to train another encoder. Scenarios that are considered in this evaluation are as the following:

- Scenario 1:

An Encoder obtained from Track 2 evaluation (Encoder 1) is used to train a decoder using a mixed dataset (Decoder 2). Decoder 2 is then used to train another Encoder (Encoder 2) with Ericsson dataset.

- Scenario 2:

An Encoder obtained from Track 2 evaluation (Encoder 1) is used to train a decoder using Ericsson dataset (Decoder 3). Decoder 3 is then used to train another Encoder (Encoder 3) with Ericsson dataset.

In all of those scenarios, the Encoder – Decoder pairs are evaluated with Ericsson test set. Results for Track 2 model pairs (Encoder 1 – Decoder 1) is also included in the evaluation. Both the unquantized models and the quantized models are evaluated. In the table, Ericsson Track 1 model (Encoder 0 – Decoder 0) is also included as a reference.

Table 12	SGCS for Option 4b with Samsung Track 2 Unquantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.7591 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.7367 | -3.0% | Encoder 1 and Decoder 1 are Samsung models for Track 2 |
| Encoder 1 | Decoder 2 | 0.7336 | -3.4% | Decoder 2 is trained with Encoder 1 and Mixed dataset |
| Encoder 1 | Decoder 3 | 0.7383 | -2.7% | Decoder 3 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.7385 | -2.7% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |
| Encoder 3 | Decoder 3 | 0.7451 | -1.8% | Encoder 3 is trained with Decoder 3 and Ericsson dataset |

Table 13	SGCS for Option 4b with Samsung Track 2 Quantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.6933 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.6857 | -1.1% | Encoder 1 and Decoder 1 are Samsung models for Track 2 |
| Encoder 1 | Decoder 2 | 0.6775 | -2.3% | Decoder 2 is trained with Encoder 1 and Mixed dataset |
| Encoder 1 | Decoder 3 | 0.6825 | -1.6% | Decoder 3 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.6795 | -2.0% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |
| Encoder 3 | Decoder 3 | 0.6850 | -1.2% | Encoder 3 is trained with Decoder 3 and Ericsson dataset |

Table 14	SGCS for Option 4b with Nokia Track 2 Quantized Encoder. Ericsson test set is used to evaluate the performance

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.6933 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.6812 | -1.7% | Encoder 1 and Decoder 1 are Nokia models for Track 2 |
| Encoder 1 | Decoder 2 | 0.6752 | -2.6% | Decoder 2 is trained with Encoder 1 and Mixed dataset |
| Encoder 1 | Decoder 3 | 0.6758 | -2.5% | Decoder 3 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.6777 | -2.3% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |
| Encoder 3 | Decoder 3 | 0.6828 | -1.5% | Encoder 3 is trained with Decoder 3 and Ericsson dataset |

Table 15	SGCS for Option 4b with Mediatek Track 2 Quantized Encoder. Ericsson test set is used to evaluate the performance.

| Encoder | Decoder | SGCS | Gain/ loss | Remark |
| --- | --- | --- | --- | --- |
| Encoder 0 | Decoder 0 | 0.6933 |  | The encoder and decoder are jointly trained with Ericsson dataset |
| Encoder 1 | Decoder 1 | 0.6769 | -2.4% | Encoder 1 and Decoder 1 are Mediatek models for Track 2 |
| Encoder 1 | Decoder 2 | 0.6717 | -3.1% | Decoder 2 is trained with Encoder 1 and Mixed dataset |
| Encoder 1 | Decoder 3 | 0.6765 | -2.4% | Decoder 3 is trained with Encoder 1 and Ericsson dataset |
| Encoder 2 | Decoder 2 | 0.6756 | -2.6% | Encoder 2 is trained with Decoder 2 and Ericsson dataset |
| Encoder 3 | Decoder 3 | 0.6822 | -1.6% | Encoder 3 is trained with Decoder 3 and Ericsson dataset |

From the above tables, we can observe that Option 4 is generally feasible. A better performance can be obtained with Scenario 2, where the Decoder is trained with the training dataset that match (having the same distribution/characteristics) the testing dataset. Note that this feasibility only relates on the feasibility on training a new encoder based on training a reference encoder – decoder – encoder chain. In the real implementation, it needs to be evaluated whether different Encoder – Decoder pairs obtained from different steps from the above tables could still work well.

For option 4, if a frozen encoder is taken from track 2, a new decoder (trained on the frozen encoder) and encoder (trained on the new decoder) can be trained that show a small performance drop compared to the encoder/decoder trained directly.

To test the feasibility of option 4, all companies should upload at least a “decoder 2” (decoder trained using frozen encoder and mixed dataset) and potentially also a “decoder 3” (decoder trained using frozen encoder and company’s own dataset). This will allow interoperability between UE vendor encoders and different TE vendor decoders to be emulated.

## 3.2	Other considerations

On the difference between option 4a and option 4b

According to option 4a, a dataset labelled with the encoder latent message for each dataset entry should be provided. Option 4b envisages that a reference encoder is provided. Although option 4b does not necessarily mean that a dataset needs to be captured, in reality, to obtain similar and interoperable decoders for different TE vendors, it is reasonable to expect that there will be a need to standardize the dataset with option 4b.

For option 4b, most likely a dataset needs to be standardized to enable reproducibility of the test decoder between different TE vendors.

If a dataset is standardized, then the difference between option 4a and 4b becomes whether a reference encoder is standardized to enable labelling of the dataset with the encoder latent space, or the dataset is simply already labelled with the encoder latent space.

To facilitate verification of the test decoder, and also to provide a reference for network vendors when creating real decoders, it is useful to capture a reference encoder in the specifications. Thus, in total it would make sense to merge options 4a and 4b to include standardization of a dataset and standardization of a reference encoder.

Merge options 4a and 4b to standardization of a dataset and standardization of a reference encoder.

Merging option 3 and option 4

For option 3, a test decoder is standardized, and it was agreed to capture a reference encoder. Proposal 4 in this contribution proposes to capture the dataset used to generate the test decoder.

For option 4, proposal 11 proposes to merge options 4a and 4b and capture a dataset and a reference encoder in the TR.

For both options, a verification dataset should be captured and used, together with the reference encoder, to verify that the implemented test decoder on TE is capable of working properly and achieving the RAN4 requirements.

With this in mind, if an encoder, dataset and decoder are captured in the specifications, then option 3 is to directly use the decoder, whereas option 4 enables the TE vendor to develop their own decoder.

To make such a merging, capturing of the dataset would be needed. Further consideration of the ability to capture a dataset is needed before such a merging could be decided.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For track 1, there is variation in performance between different companies that depends on the training set and the test set.

Observation 2	Based on the Samsung decoder, option 3 track 1 seems basically feasible, at least where companies have similar datasets.

Observation 3	The variation between the mixed testset performance and individual companies testsets for track 2 has decreased but is still significant.

Observation 4	Variations between different companies datasets (and the mixed dataset) appears to be related to differing levels of correlation between the SGCS observed for different sub-bands.

Observation 5	Based on working with the Samsung decoder as a frozen decoder, option 3 track 2 works very well if the UE vendor encoder is trained with the same mixed dataset as the test decoder. If trained with a different dataset, there is a small difference in performance.

Observation 6	For option 4, if a frozen encoder is taken from track 1, a new decoder (trained on the frozen encoder) and encoder (trained on the new decoder) can be trained that show a small performance drop compared to the encoder/decoder trained directly.

Observation 7	For option 4, if a frozen encoder is taken from track 2, a new decoder (trained on the frozen encoder) and encoder (trained on the new decoder) can be trained that show a small performance drop compared to the encoder/decoder trained directly.

Observation 8	For option 4b, most likely a dataset needs to be standardized to enable reproducibility of the test decoder between different TE vendors.

Based on the discussion in the previous sections we propose the following:

Proposal 1	If there is a need for it to be part of a TS or TR, then preferably the reference encoder should be part of a TS. However, it may be suitable that it is stored in the same managed location as the test decoder.

Proposal 2	The reference encoder should be the encoder that is paired with, i.e. trained together with, the test decoder using the same training data.

Proposal 3	Keep the test decoder and reference encoder together, in the RAN4 specifications. Or alternatively store and maintain the models together in the same place and refer from the RAN4/5 specifications.

Proposal 4	If available, store the dataset used to create the test decoder such that it is linked to the test decoder and reference encoder and maintained across releases.

Proposal 5	Discuss during development of the requirement whether to describe test and verification data by means of a channel model or an explicit dataset.

Proposal 6	The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed (for CSI compression and future models). Possible means to achieve this may be discussed if time allows.

Proposal 7	The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed.

Proposal 8	The study should consider what key characteristics should drive the discussion on structure for a real standardization exercise. Some characteristics include:

	Backbone (CNN, transformer etc.)

	Complexity target

	Number of models (depending on scenarios etc.)

	A metric and acceptable performance target

	Ability of the agreed structure for the test decoder to allow for variation of real encoder/decoder implementation

Proposal 9	Conclude that, if Option 3 is supported for performance testing, then the fully specified test decoder is the same as the decoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion.

Proposal 10	Conclude that, for a real WI, a sufficient diversity of simulation assumption sets would be needed to make the training dataset for the test decoder. Also, some steps to ensure alignment of different companies’ datasets are needed.

Proposal 11	To test the feasibility of option 4, all companies should upload at least a “decoder 2” (decoder trained using frozen encoder and mixed dataset) and potentially also a “decoder 3” (decoder trained using frozen encoder and company’s own dataset). This will allow interoperability between UE vendor encoders and different TE vendor decoders to be emulated.

Proposal 12	Merge options 4a and 4b to standardization of a dataset and standardization of a reference encoder.