3GPP TSG-RAN4 Meeting  #114bis	R4-2504451

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.5.3

Source:	Apple

Title:	On UE RF requirements for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues:

- Impacts on DL performance due to self-interference and blocker

- Triggering condition to enable ”One RF Rx chain” and UE indication

# 2	Discussion

Requirements for UE supporting fragmented carriers with shared RF chains

In the earlier WF [2], the point on how to specify requirements for UE supporting fragmented carriers with shared RF chains is in brackets, as shown below

- [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

It is important to have a common understanding on it. As is well understood, the current RF requirements for intra-band non-contiguous CA were derived based on a partially shared RF chain architecture captured in TR 36.823. Given the blocker in the gap, it is evident that a shared RF chain architecture cannot meet such requirements. That is the reason why the SI scope explicitly contains the bullet “Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking.”

To be clear, whatever the outcome of the SI is, the existing requirements should still apply to UEs using partially shared RF chain architecture to support fragmented carriers (or intra-band non-contiguous CA). With that, we think it is reasonable to say the adjusted requirements for UE supporting fragmented carriers with shared RF chains should be separated from existing requirement. Of course, there should be a UE capability or other means to indicate such UEs.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Discussion of impacts on DL performance for UE enabling fragmented carrier mode

In current CA receiver sensitivity requirement, there is ΔRIBNC to allow relaxation due to TX noise at RX band, i.e., TX noise at SCC, which assumes UE architecture of separate RF chains, as shown in the specification excerpts below.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |

With common RF chain architecture, the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, both the PCC and SCC performance may take some hit.

In our previous contribution [4], we provided analysis on the impact of self-interference, i.e., the NF increase due to AGC adjustment (i.e., RX gain backoff). Table 1 and Table 2 show our analysis for CA configuration CA_n3(2A) (5MHz PCC+ 5MHz SCC, Wgap = 65MHz) and CA_n25(2A) (5MHz PCC + 5MHz SCC, Wgap = 55MHz), respectively.

Table 1. Link budget of Self-interference for CA_n3(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - |
| LPF output (dBm) | -36.4 | -102 | -97.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.5 | - |
| Noise floor (including ADC noise) | - | - | - | -90.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -11.5(PCC) |
|  |  |  |  |  | -7.5(SCC) |

Table 2. Link budget of Self-interference for CA_n25(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -100.5 | -95.5 | - | - |
| LPF attenuation | 12.9 | 1 | 1 | - | - |
| LPF output (dBm) | -35.9 | -101.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -100.5 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 11 | - |
| Noise floor (including ADC noise) | - | - | - | -89.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -12(PCC) |
|  |  |  |  |  | -7.7(SCC) |

Observation 1: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), ΔRIBNC may need to be increased by about 11dB for PCC and 7dB for SCC.

In the WF at the last meeting [3], the following is captured:

#### Issue 3-3: Discussion on in-gap interference requirements - For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC. - For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC.

Next, we consider the impact of blocker in the gap in addition to the self-interference. In the analysis, we consider two cases, where the image of the blocker overlaps and does not overlap with a wanted CC.

Case 1: The image of the blocker does not overlap with a wanted CC

Based on our analysis, the results are shown in Table 3 and Table 4 for CA_n3(2A) and CA_n25(2A), respectively.

Note the following in the analysis:

- In ACS Case 1, both the receiver sensitivity (REFSENS) and blocker power are increased by the proposed ΔRIBNC adjustment due to self-interference.

- In IBB Case 1 and Case 2, only the receiver sensitivity (REFSENS) is increased by the proposed ΔRIBNC adjustment due to self-interference, but the blocker power is not. This is because in ACS Case 2, the wanted signal power is fixed at -56.5dBm and the blocker power is fixed at -25dBm in ACS Case 2, -56dBm in IBB Case 1, and -44dBm in IBB Case 2, respectively.

Table 3. Link budget of Blocker for CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 13.4 |  |
| NF increase due to AGC adjustment (dB) | 10.5 |  |
| Proposed ΔRIBNC adjustment due to self-interference | 10.5dB for PCC | 6.5dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | 13 | 13 |
| ACS Case 2 | 4.1 | 4.1 |
| IBB Case 1 | 5 | 5 |
| IBB Case 2 | 5 | 5 |

Table 4. Link budget of Blocker for CA_n25(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12.9 |  |
| NF increase due to AGC adjustment (dB) | 11 |  |
| Proposed ΔRIBNC adjustment due to self-interference | 11dB for PCC | 6.7dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | 13 | 13 |
| ACS Case 2 | 4.1 | 4.1 |
| IBB Case 1 | 5 | 5 |
| IBB Case 2 | 5 | 5 |

Based on the analysis, with the proposed ΔRIBNC adjustment due to self-interference, the ACS/IBB requirements can be met with some margin. However, we note that this conclusion may depend on the CA band combination. For instance, if the duplex gap between UL and DL bands are wide enough, in other words, the TX leakage is of little issue and little ΔRIBNC adjustment is needed to meet the REFSENS requirement, there may be some adjustment needed for ACS/IBB requirements.

Observation 2: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), with the proposed ΔRIBNC adjustment due to self-interference, the ACS/IBB requirements can be met.

Case 2: The image of the blocker overlaps with a wanted CC

When the image of the block overlaps with a wanted CC, the results are shown in Table 5 and Table 6 for CA_n3(2A) and CA_n25(2A), respectively.

Note the following for the analysis:

- Image rejection ratio at the UE is assumed to be 25dB.

- Other assumptions are the same as in Case 1 where the image of the blocker does not overlap with a wanted CC.

- For the two CA configurations, CA_n3(2A) (5MHz PCC + 5MHz SCC, Wgap = 65MHz) and CA_n25(2A) (5MHz PCC + 5MHz SCC, Wgap = 55MHz), because the PCC and SCC have the same channel bandwidth, the image of the blocker does not overlap with a wanted CC. Therefore, the analysis shown below are aimed to provide an initial assessment of the impact. Further analysis is needed to address the CA configurations where the image of the blocker does overlap with a wanted CC.

Table 5. Link budget of Blocker for CA_n3(2A)

| Image rejection ratio (IRR) (dB) | 25 |  |
| --- | --- | --- |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6.5 |
| ACS Case 2 | -6.5 | -6.5 |
| IBB Case 1 | 0.5 | 1.2 |
| IBB Case 2 | -11.5 | -10.8 |

Table 6. Link budget of Blocker for CA_n25(2A)

| Image rejection ratio (IRR) (dB) | 25 |  |
| --- | --- | --- |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6.5 |
| ACS Case 2 | -6.5 | -6.5 |
| IBB Case 1 | 1.5 | 2.2 |
| IBB Case 2 | -10.5 | -9.8 |

It can be seen that the SNR for ACS Case 1, ACS Case 2 and IBB Case 2 is 5-10dB below -1dB, showing the image of the blocker causes strong interference to the receiver.

Observation 3: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), with the proposed ΔRIBNC adjustment due to self-interference, 25dB IRR shows strong performance degradation.

Another way to address the image interference issue is to limit the blocker to wanted CC power ratio of 26dB. If this is the case, SNRs for ACS/IBB Case 1 and 2 in Table 5 and Table 6 will be -1dB or higher.

Observation 4: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), with the proposed ΔRIBNC adjustment due to self-interference, the image interference issue can be addressed by limiting the blocker to wanted CC power ratio to 26dB, assuming IRR of 25dB.

Based on Observations 3 and 4, we propose:

Proposal 2: To address the interference due to the image of the blocker, it is proposed to limit the blocker to wanted CC power ratio to 26dB.

Triggering condition to enable ”One RF Rx chain”

In the WF at the last meeting [3], the following is captured:

### Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition.

First, as shown in our contribution [4], it is feasible to measure total aggregated power of all interferences in the gap between the two wanted CCs if the UE is given such measurement opportunity, after considering the following factors:

- The gap width: As shown in Table 7.3A.2.2.-1 from 38.101-1, the maximum gap width is 65MHz, which can certainly be supported by one RX RF chain.

- UE BB capability: In NR, UE is required to measure NR carrier RSSI and hence should be able to use the same hardware/software resources to measure the blocker power.

- Maximum blocker power: this is currently being discussed and we agree that the current maximum input level in 38.101-1 can be used as a starting point to avoid receiver saturation.

- The need of measurement gap for such measurement. This may depend on UE implementation and if needed, can be further discussed.

Observation 5: It is feasible to measure total aggregated power of all interferences in the gap between two wanted CCs if the UE is given such measurement opportunity. How to enable the UE to perform such measurement is to be discussed, e.g., a dedicated measurement gap is needed.

Second, as the analysis of self-interference and blocker impact shows, there is DL performance degradation in terms of SNR when the UE operates with one Rx RF chain. In this case, there is a clear need for triggering the switching between one Rx RF chain and legacy Rx RF chain. There can be multiple ways to trigger the switching, e.g., based on configured TX power for the UE (because the TX power would determine the required ΔRIBNC), based on measurements and reporting of the blocker power at the UE (because in the field the blocker power could be much higher than the wanted CC signal), or based on a combination of both. While whether and how to specify such a triggering condition can be further discussed, some simply ways can be considered. For instance, UE only indicates to the network whether it can support non-contiguous CCs with common/fully shared RF chains or not, assuming the associated degradation is no greater than allowed degradation that is specified. It is up to the network to decide if the UE should be configured to work with one RX RF chain to support the two non-contiguous CCs and use the spared RX RF chain to support additional CCs.

Third, UE RF reconfiguration between one Rx RF chain and legacy Rx RF chain should be under network control. This is because associated with the RF reconfiguration is the change in CA configuration, which is currently configured by network.

Proposal 3: Whether and how to specify a triggering condition for the UE to work with one Rx RF chain and legacy Rx RF chain can be further discussed. Meanwhile, it is proposed to allow UE indication to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified. It is up to the network to decide if the UE should be configured to work with one RX RF chain to support the two non-contiguous CCs and use the spared RX RF chain to support additional CCs.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Observation 1: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), ΔRIBNC may need to be increased by about 11dB for PCC and 7dB for SCC.

Observation 2: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), with the proposed ΔRIBNC adjustment due to self-interference, the ACS/IBB requirements can be met.

Observation 3: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), with the proposed ΔRIBNC adjustment due to self-interference, 25dB IRR shows strong performance degradation.

Observation 4: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), with the proposed ΔRIBNC adjustment due to self-interference, the image interference issue can be addressed by limiting the blocker to wanted CC power ratio to 26dB, assuming IRR of 25dB.

Proposal 2: To address the interference due to the image of the blocker, it is proposed to limit the blocker to wanted CC power ratio to 26dB.

Observation 5: It is feasible to measure total aggregated power of all interferences in the gap between two wanted CCs if the UE is given such measurement opportunity. How to enable the UE to perform such measurement is to be discussed, e.g., a dedicated measurement gap is needed.

Proposal 3: Whether and how to specify a triggering condition for the UE to work with one Rx RF chain and legacy Rx RF chain can be further discussed. Meanwhile, it is proposed to allow UE indication to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified. It is up to the network to decide if the UE should be configured to work with one RX RF chain to support the two non-contiguous CCs and use the spared RX RF chain to support additional CCs.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2417201, "WF on NR FR1 DL fragmented carriers study," Mediatek

R4-2502873, "WF on fragmented DL carriers study," Moderator (Mediatek Inc.), OPPO, vivo, ZTE, Nokia, Ericsson, Xiaomi, Apple, Samsung, Huawei, Murata, CHTTL

R4-2500244, "On UE RF requirements for fragmented carriers," Apple