3GPP TSG-RAN WG4 Meeting #114bis		R4-2504453

Wuhan, Hubei, China, 07th – 11th April, 2024

Title:	[draft] LS on signaling aspects of UE support of fragmented carriers

Response to:

Release:	R19

Work Item:	FS_NR_FR1_DL_Frag_Carrier

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Steven Chen

steven.x.chen AT apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 has been discussing signalling solutions for UE to indicate support of DL fragmented carriers with one Rx RF chain considering pros and cons of each candidate solution, as shown in Table 1 below.

Table 1: Three alternatives of signaling solutions with pros and cons analysis

|  | How it works | Pros | Cons |
| --- | --- | --- | --- |
| Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication | UE reports to network that it is capable of Rx chain sharing for the supported band combination by additional signaling on top of existing band combination reporting. An example is the UE supports CA_n7(2A)-n25(2A) with Rx sharing capability and supports CA_n7A-n25(2A) without. Note: UE indication of Rx sharing can or cannot be supported based on some triggering condition is needed. | The current CA capability reporting is reused. | New UE capability is needed to indicate support of Rx chain sharing. |
| Alt. 2: Band combination capability update with the supported number of CCs | The UE first reports a high CA combination assuming non-continuous CCs can be handled by common RF chain. If some triggering conditions are met, such as when the blocker power is too high or self-interference due to TX leakage is too high, the UE can inform the network that only fewer CCs can be supported. Similarly, the UE first reports it supports CA_n7(2A)-n25(2A), and later if the triggering conditions are met, either the UE can use UAI or other mechanism to release CCs to CA configuration of CA_n7A-n25(2A) or the network can release CCs to CA configuration of CA_n7A-n25(2A) Note: UE indication of Rx sharing cannot be supported based on some triggering condition is needed. | The current CA capability reporting is reused. No new UE capability is introduced. |  |
| Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling | A new syntax/notation, say CA_n25(f_2A), is used for indicating support of fragmented carrier operation in for CA in band n25 with common RF chain. Depending on some triggering conditions, the UE may indicate it can or cannot support it. Note: UE indication of Rx sharing can or cannot be supported based on some triggering condition is needed. | It is visible and clear in RAN4 specification that a CA configuration is supported by UE with RF sharing capability. | A new syntax/notation is needed for both RAN4 and RAN2. |

In addition, UE may need to indicate to the network when it can be configured with Rx sharing (i.e., one Rx RF chain) for the two non-contiguous CCs and thus the spared Rx RF chain can be used support additional CCs (i.e., a higher order CA configuration).

To facilitate the future RAN4 discussion, RAN4 would like to understand from RAN2 perspective which alternative solution is preferable considering the required UE indication.

# 2	Actions

To RAN

ACTION: 	RAN4 respectfully asks RAN2 to share views on the above signaling solutions considering the required UE indication.

# 3	Dates of next TSG RAN WG4 meetings

RAN4#115	May 19 – 23, 2025		Malta, MT

RAN4#116	Aug. 25 – 29, 2025		India, IN