3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504473

Wuhan, China, 7th – 11th April 2025

Source:	Nokia

Title:	NR NTN system parameters for Ku band

Agenda item:	7.11.3

Document for:	Approval

Introduction

In RAN4#114, WF for NTN Ku band System parameters was agreed in the following.

### Sub-topic 3-6: FR1-NTN ARFCN definitions Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN NTN satellite operating band; ΔFRaster (kHz); Uplink Range of NREF (First – <Step size> – Last); Downlink Range of NREF (First – <Step size> – Last). n248; 30; 1333334 – <2> – 1366666; 1113334 – <2> – 1250000. n247; 30; 1316668 – <2> – 1333332; 1113334 – <2> – 1250000. ### Sub-topic 3-7: FR2-NTN ARFCN definitions Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN SAN operating band; ΔFRaster (kHz); Uplink range of NREF (First – <Step size> – Last); Downlink range of NREF (First – <Step size> – Last). n509; 120; 1333336 – <8> – 1366664; 1113336 – <8> – 1250000. n508; 120; 1316672 – <8> – 1333328; 1113336 – <8> – 1250000. - Sub-topic 3-8: FR1-NTN GSCN definitions Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN) NTN satellite operating band; SS Block SCS; SS Block pattern1; Range of GSCN (First – <Step size> – Last). n248; 30 kHz; Case C; 12850 – <1> – 14266. n247; 30 kHz; Case C; 12850 – <1> – 14266. - Sub-topic 3-9: FR2-NTN GSCN definitions Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN) SAN operating band; SS Block SCS; SS Block pattern (NOTE); Range of GSCN (First – <Step size> – Last). n509; 120 kHz; Case D; 12864 – <12> – 14256. n508; 120 kHz; Case D; 12864 – <12> – 14256. ### Sub-topic 3-10: Default Tx Rx frequency separation Agreement: Variable TX RX spacing is assumed, how it is specified is FFS. Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.

RAN#107 has agreed to include also 15 kHz SCS with the channel bandwidths 10MHz, 15MHz, 25MHz, 35MHz and 50MHz [1] for FR1-NTN, therefore the above agreements need to be updated for FR1-NTN.

Discussion

Channel bandwidth and Subcarrier spacing

For FR1-NTN Ku bands, 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing , and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing are supported as defined in WID [1] .

The following channel bandwidth table should be introduced for FR1-NTN.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 50 | 70 | 100 |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |

Channel bandwidth table for FR1-NTN Ku bands (n248 and n247) with 15kHz SCS is specified according to the above table.

Channel raster

SCS-based raster was agreed in RAN4#112-bis [5], i.e., 30 kHz raster for 30 kHz SCS of FR1-NTN Ku bands and 120 kHz raster for 120 kHz SCS of FR2-NTN Ku bands. Applicable NR-ARFCN were agreed in [4].

Furthermore, 15 kHz SCS is also agreed for FR1-NTN. Therefore, the following channel raster definitions for 15 kHz SCS shall be added.

For NTN satellite operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I> for FR1-NTN.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

NR-ARFCN for FR1-NTN Ku bands (n248 and n247) with 15kHz SCS is specified according to the above table.

Tx-Rx Separation

We agreed flexible duplex in RAN4#112-bis [2] and this was further confirmed in RAN4#114 in the WF [7]. If is FFS whether to specify the default duplex of the flexible duplex for Ku band.

No default Tx-Rx separation is specified in clause 5.4.4 of TS 38.101-5 for Ka band. This means that there is no restriction in Tx-Rx separation, thus it would be understood that all UEs support fully flexible duplex as far as both UL and DL channel bandwidths are within the band.

This is more likely the case for Ku band, thus we propose that no default Tx-Rx separation for Ku band is specified.

All UEs are supposed to support fully flexible Tx-Rx separation without default Tx-Rx separation.

No default Tx-Rx separation for Ku band is specified.

Sync raster

In the WF agreed in RAN4#112-bis [2], the existing sync raster design was assumed for Ku bands, meaning that the GSCN parameters in Table 5.4.3.1-1 of TS 38.101-5.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}1 | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

For the FR1-NTN Ku band with 30 kHz SCS, SS block pattern C should be assumed, instead of pattern B which is intended for a specific LTE-NR coexistence scenario.

For 15 kHz SCS, only SS block pattern A is available and GSCN ranges for n248 and n247 for both numerologies are derived in the following.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12850 – <1> – 14266 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12850 – <1> – 14266 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

GSCN for FR1-NTN Ku bands (n248 and n247) with 15kHz SCS is specified according to the above table.

Conclusion

In this contribution, the systema parameters for FR1-NTN Ku bands are proposed.

Proposal 1: Channel bandwidth table for FR1-NTN Ku bands (n248 and n247) with 15kHz SCS is specified according to the above table.

Proposal 2: NR-ARFCN for FR1-NTN Ku bands (n248 and n247) with 15kHz SCS is specified according to the above table.

Observation 1: All UEs are supposed to support fully flexible Tx-Rx separation without default Tx-Rx separation.

Proposal 3: No default Tx-Rx separation for Ku band is specified.

Proposal 4: GSCN for FR1-NTN Ku bands (n248 and n247) with 15kHz SCS is specified according to the above table.

- References

- RP-250799 Revised WID: Introduction of Ku bands for NR NTN, RAN#107

- R4-2416593 Way Forward for [112bis][313] NR_NTN_Ku_Band_General, RAN4 #112 bis

- R4-2502288 WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114

- R4-2502290 Way Forward for [114][313] NR_NTN_Ku_Band_General, RAN4#114