3GPP TSG-RAN WG4 Meeting #114-bis 	R4-2504481

Wuhan, China, April 7th – 11th, 2025

Agenda Item: 7.28.2

Source: MediaTek inc.

Title: Discussion on RRM requirements for XR

Document for: Discussion

# Introduction

The work item (WI) for XR (eXtended Reality) for NR Phase 3 has the following objectives [1]:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4]

In this contribution, the potential directions that leads to enhance the gap cancellation mechanisms are discussed then other RRM impact issues are raised, which requires some studies.

# Discussion

RAN4 RRM has discussed and identified the impact on RRM requirements as below.

# Discussion on scenarios for measurement skipping

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

Issue 1-1: Deployment scenarios:

< Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC

Due to the limited TU for this WI, RAN4 should de-prioritize NR-DC.

- RAN4 should de-prioritize NR-DC.

# Discussion on measurements with gaps

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

Issue 2-1: Types of measurement gaps to consider:

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Way forward>: Please discuss the priorities for RAN4 to work based on the following table: Cap types; First prio; Second prio; Don’t include. Type 1. Type 2. NCSG. MUSIM. Positioning. Rel 18 concurrent gap. PreConfigured.

RAN4 shall decide on whether the UE is expected to do all the measurements while operating with XR data traffic. RAN4 has already agreed that Type-1 should be considered with high priority, while all other measurements gaps/interruption can be second priority. Besides, RAN4 should discuss the necessity of having positioning and MUSIM measurement gaps while the UE is operating with XR traffic.

In addition, the solution of Type-1 can be applied for other measurement gap types at a later stage. Yet, RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types.

- RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic.

- RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types, which are Type-2, NCSG, Con-current GAP, and Pre-MG.

# Discussion on XR skipping requirements impact

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

Issue 3-1: How to capture impact of measurement skipping <Agreement from [R4-2420103]>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay ## Issue 3-1: Principles for measurement delay calculation <Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension. ## Issue 3-2: Formulas for measurement delay extension <Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. ## Issue 3-3: Conditions for measurement skipping <Way forward>: - Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

Furthermore, RAN4 discussed the impact on the measurements delay requirements due to MG cancellation. The number of samples for a single layer measurement was decided based on simulation evaluation for different channel environment. Given that the XR is meant to operate with NR devices then the focus shall be on the impact of measurement gap cancellation and how to compensate the dropped measurement samples. In previous meetings, RAN4 already agreed to extend the delay requirements to compensate the skipped measurement gap occasions, also, RAN4 agreed to follow NR-U like approach to define the requirements. In addition, the way forward captured two options for information and based on the NR-U like approach, Option 2 should be supported, i.e. ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

- RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Furthermore, cancelling multiple measurement gaps occasions might result in mobility performance degradation. Therefore, RAN4 should discuss how to limit such impact. One option is to limit the max number of cancelled gap occasions within a specific time window.

- RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

# Discussion on additional skipping schemes

Issue 6-1: Semi-static schemes:

<Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

From implementation point of view, semi-static approach that NW indicate which occasions to be skipped using RRC and MAC-CE is preferable.

- RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

Issue 6-2: Skipping indication on frequency domain:

<Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

When the UE is configured with Per-FR gap and the UE receive DCI for MG cancellation, the UE shall cancel the gap occasion in that specific FR associated with that Per-FR gap. Otherwise, when the UE is configured with Per-UE gap and the UE receive DCI for MG skipping, the UE skips the gap occasions for both FR1 and FR2.

- When a UE is configured with Per-FR gap, the MG skipping applies only to the FR associated with that Per-FR gap, otherwise, when Per-UR gap is configured, the MG skipping applies to both FRs.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 should de-prioritize NR-DC.

Proposal 2: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic.

Proposal 3: RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types, which are Type-2, NCSG, Con-current GAP, and Pre-MG.

Proposal 4: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

Proposal 5: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of  skipped MG occasions in the measurement period.

Proposal 6: RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

Proposal 7: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

Proposal 8: When a UE is configured with Per-FR gap, the MG skipping applies only to the FR associated with that Per-FR gap, otherwise, when Per-UR gap is configured, the MG skipping applies to both FRs.

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, MediaTek (Moderator), Dec 2023.

- R4-2502700, “WF on RRM requirements for XR_Ph3”, Athens, Greece, Feb, 2025.