3GPP TSG-RAN WG4 Meeting #114bis	R4-2504506

Wuhan, China, April 7 - 11, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SSB adaptation

Agenda item:	7.25.2.2

Document for:	Discussion

Introduction

The discussion on SSB adaptation as part of the third objective “adaptation of common signal/channel transmissions” of the Rel-19 WID on Enhancements of network energy savings for NR [1], was continued at RAN4#114 meeting [2] and the agreements were captured in the WF [3]. In this contribution, we continue the discussion on the RRM impact from SSB adaptation.

Discussion

  - Progress status in RAN4 and RAN1

At RAN4#114 meeting, the following has been agreed on RRM requirements for SSB adaptation [3].

- Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues.

Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused. - Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG Issue 1-4-2: TCI state switching requirements for SSB adaptation Agreement: - No requirements when SSB adaptation happens during TCI state switching. - Sub-topic 2-1 RRM requirements impacts for OD-SIB1 Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

Open issues for SSB adaptation are contained in [2] and previous RAN4 agreements are listed in Annex 1.

RAN1 agreements are listed in Annex 2.

  - Scenarios for SSB adaptation

In this section we consider open issues related to scenarios for SSB adaptation.

## SSB adaptation in SCell

The RAN1#120 conclusion on support of SSB adaptation for SCell is depicted below.

Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for Scell

Based on this conclusion, RAN4 needs to consider RRM impacts from SSB adaptation for SCell for NCD-SSB only.

Proposal 1: 	RAN4 to consider RRM impacts from SSB adaptation in SCell for NCD-SSB only.

## SSB periodicity range

Regarding the range of SSB periodicities, RAN1#120 achieved following conclusion.

Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst

Hence the existing specified range of SSB burst periodicities is kept for SSB periodicity adaptation.

Furthermore, there is no change of SSB burst positions or number of SSBs in SSB burst, thus periodicity is the only parameter which may be changed due to SSB adaptation in time domain.

Proposal 2: 	RAN4 to specify RRM requirements for SSB adaptation based on the existing range of SSB burst periodicities (5ms to 160ms) for SSB periodicity adaptation as the only changeable parameter.

## Signalling methods for SSB adaptation in RAN1 and RAN2

With regard to signalling methods for adapting the SSB periodicity, RAN1#120 achieved following two agreements.

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Based on the above two RAN1#120 agreements, it is obvious that SSB periodicity adaptation may be triggered by group common DCI command (foreseen to be specified by RAN1) or MAC-CE command (foreseen to be specified by RAN2). Thus, RAN4 should consider specifying RRM requirements for both signalling methods. It is expected that RAN1 will clarify usage of both signalling methods, so that there is no overlap in terms of use case scenarios.

Proposal 3: 	RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered DCI and MAC-CE signalling methods, designed by RAN1 and RAN2.

## Direct SCell activation and SSB adaptation

It is noted that RAN4 #114 discussed the scenario for OD-SSB based direct SCell activation. The following agreement was achieved [4].

Issue 6-1: OD-SSB based direct SCell activation - Background RAN1 #118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.. Agreement: - RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion.

In our view, the same scenario in the NES cell, i.e. direct SCell activation should be considered for SSB adaptation. In such case we propose to align the RRM requirements based on RRC with OD-SSB based direct SCell activation as much as possible.

Proposal 4: 	RAN4 to consider the scenario for SSB adaptation command sent together with direct SCell activation command and investigate the impact to SCell addition requirements and align them to OD-SSB based direct SCell activation as much as possible.

  - Layer 1 measurement impact

RAN4#114 achieved the agreement on requirements needing updating as listed in Annex 1. Based on this, SSB based L1 measurements in RRC CONNECTED, agreed to be affected by SSB adaptation, are identified to be the following ones:

- CBD,

- L1-RSRP,

- L1-SINR.

    - Impact to measurement/evaluation periods for non-transition periods

The impact to the above measurements for non-transition periods has been analyzed in [5].

In conclusion, for CBD, L1-RSRP and L1-SINR measurements, legacy RRM requirements are applicable for non-transition periods. The L1 measurement impact from SSB periodicity adaptation is hence limited to transition periods.

Proposal 5: 	RAN4 to consider the impact from SSB periodicity adaptation to Layer 1 measurements such as CBD, L1-RSRP and L1-SINR for active SCells during transition periods only.

## L1 transition requirements

RAN4#114 achieved following agreement related to L1 transition requirements as summarized in section 2.1 and discussed the following open issue.

Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

Thus, if measurement/evaluation period is based on SSB periodicity after adaptation from Tfirst_SSB, measurement/evaluation period for preceding SSBs is based on SSB periodicity before Tfirst_SSB, provided SSB adaptation command is received in time before SSB at Tfirst_SSB , the time offset, named SSB adaptation delay (see section 2.6), being dependent on the processing delay for the DCI or MAC-CE signaling method as well as on the reprogramming of the SSB reception.

Proposal 6: 	RAN4 to agree the measurement period for CBD, L1-RSRP and L1-SINR prior to the SSB received at Tfirst_SSB is based on the SSB periodicity before adaptation.

For L1 transition requirements, SSB adaptation delay needs to be taken into account as depicted in Figures 1 and 2 for transition to lower SSB periodicity and in Figure 3 and 4 for transition to higher SSB periodicity. Following use case scenarios are distinguished:

- Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period (MP), see Figure 1.

- Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period (MP), see Figure 2.

- Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period (MP), see Figure 4.

- Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period (MP), see Figure 4.

Figure 1: SSB adaptation to lower periodicity with early SSB adaptation command after completed L1 MP.

Figure 2: SSB adaptation to lower periodicity with late SSB adaptation command after completed L1 MP.

Figure 3: SSB adaptation to higher periodicity with early SSB adaptation command after completed L1 MP.

Figure 4: SSB adaptation to higher periodicity with late SSB adaptation command after completed L1 MP.

Thus, RAN4 should consider the above use case scenarios with SSB adaptation command received after or within the L1 measurement period and taking account the SSB adaptation delay as defined in section 2.7.

Proposal 7: 	RAN4 to agree following use case scenarios for specifying L1 transition requirements are distinguished:

-	Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period.

-	Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period.

## L1 reporting requirements

RAN4 did not yet discuss L1 reporting requirements for SSB adaptation. L1 reporting requirements are covered by CSI reporting requirements in TS 38.214, clause 5.2.1. The report configuration is based on the IE CSI-ReportConfig as defined in TS 38.331, clause 6.3.2. This serves both for periodic or semi-periodic reports sent on PUCCH or for semi-periodic and aperiodic reports sent on PUSCH triggered by DCI received on the cell in which the IE CSI-ReportConfig is included.

For SSB periodicity adaptation, we assume periodic reports being typically configured. The periodicity of the reports is defined in the IE CSI-ReportPeriodicityAndOffset as part of the IE CSI-ReportConfig being depicted hereafter.

CSI-ReportPeriodicityAndOffset ::=  CHOICE {

slots4                              INTEGER(0..3),

slots5                              INTEGER(0..4),

slots8                              INTEGER(0..7),

slots10                             INTEGER(0..9),

slots16                             INTEGER(0..15),

slots20                             INTEGER(0..19),

slots40                             INTEGER(0..39),

slots80                             INTEGER(0..79),

slots160                            INTEGER(0..159),

slots320                            INTEGER(0..319)

}

The periodicity can be selected from a range of 4 slots to 320 slots, hence e.g. from 4ms to 320ms for SCS=15 kHz. In case of SSB periodicity adaptation, the report periodicity may also be changed. For instance, if the SSB periodicity in section 2.3.2 is changed from 20ms to 40ms, then the report periodicity for the depicted case (M=3, P=1) may also change from 80ms (≥ M*P*20ms) to 160ms (≥ M*P*40ms), assuming that the latest valid and available L1 measurement is being reported. Likewise, the report periodicity may be shortened if after SSB adaptation the SSB periodicity is halved.

RRC based signaling for adjusting the CSI report configuration is then used. The reporting configuration change thus is kept independent of the DCI or MAC-CE based triggering mechanism for SSB adaptation. It is noted RAN1 is currently discussing whether and how adaptation of the SSB periodicity affects CSI measurements and reporting.

Proposal 8: 	RAN4 not to consider any impact to measurement requirements from L1 reporting for SSB adaptation, until further notice from RAN1.

  - Layer 3 measurement impact

RAN4 #112bis sent LS to RAN2 [6] asking about the impact on SMTC configuration due to SSB adaptation. The RAN2 reply is received at this RAN4 meeting [7] and is discussed below. L3 measurement impacts for deactivated SCell and activated SCell are considered subsequently. Finally, L3 measurement impacts in transition periods are investigated.

## RAN2 Reply LS on SMTC configuration

RAN2#129 sent LS to RAN4 [7] replying to RAN4’s LS [6] on SSB adaptation related to SMTC configuration for SSB adaptation. The agreement is replicated below.

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

This agreement means that further additional SMTC configuration may be configured by the network based on the adapted SSB periodicity being used for serving cell measurements in SCell, whilst neighbor cell measurements in SCell will be based on legacy SMTC. It is our understanding that this additional SMTC configuration may carry a set of SSB periodicities, from which one will be activated in the SSB adaptation command, i.e. only one SSB periodicity is active at any time in the SCell.

Proposal 9: 	RAN4 to proceed with the assumption that the additional SMTC configuration carries a set of SSB periodicities, from which one will be activated in the SSB adaptation command and only one active at any time in the SCell.

The RAN2 agreement also means that there is no impact on L3 neighbor cell measurements, since the legacy SMTC applies. Hence the network must ensure that the adapted SSB periodicity in SCell is not larger than the legacy SMTC periodicity.

Observation 1: 	The network must ensure that the adapted SSB periodicity in SCell is not larger than the legacy SMTC periodicity.

Based on the agreement depicted in the RAN2 LS [7], only serving cell measurements in SCell are subject to SSB adaptation, whilst neighbor cell measurements are based on legacy SMTC. Hence L3 measurement impact has to be distinguished for activated SCell / deactivated SCell measurements.

## L3 measurement impact to deactivated SCell

For deactivated SCell measurements, RAN4 #114 achieved following agreement:

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

Legacy requirements for L3 measurements of deactivated SCells in RRC_CONNECTED state are contained in clauses 9.2.5 and 9.2.7, being specified under intra-frequency requirements. This includes the case that NCD-SSB based L3 measurements are used in the SCell, if the NW indicates it as reference SSB.

RRM impacts exist for cell identification of unknown cells (PSS/SSS detection, time index detection) and cell measurements for known cells, both for measurements without gaps (from Rel-15) and with NCSG (from Rel-17) as the measurement period is proportional to the measCycleSCell value up to a defined limit.

In all cases, the measurement period is depending on the parameter measCycleSCell, configurable between 160 ms and 1280ms, hence it is greater or equal to the largest existing SSB periodicity (160ms). Thus, there is no misconfiguration possible between SSB periodicity and measCycleSCell. In addition, these measurements will cause interruptions in the serving cell, either unknown (of measured without gaps) or known (if measured with NCSG). Higher number of these interruptions may result in more required retransmissions in the PCell which should be minimized. Hence it is recommended to stay with the legacy UE behavior for deactivated SCell measurements, as throughput in serving cell may be degraded due to caused interruptions and both UE / network energy savings may be compromised in case of lower SSB periodicity after adaptation.

Proposal 10: 	RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG.

## L3 transition requirements

RAN4#114 did not further discuss the submitted proposals related to L3 transition requirements. The following progress is noted in [2].

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general - Proposals - Option 1: Wait for L2 inputs on whether there is impact on SMTC. (QC, Xiaomi, LGE, CATT, Huawei, Ericsson, Vivo, Samsung) - Option 2: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and NR intra-frequency time index delay, as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell. (Nokia) - Option 3: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation. (Oppo) - Option 4: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi) - Option 5: For adaptation of SSB transmissions, suggest RAN4 deprioritize the discussion on RRM requirement impact on deactivated SCell measurement/detection (Samsung) - If deactivated SCell measurement/detection is needed based on SSB sources for adaptation, RAN4 to discuss whether the principle defined for OD-SSB can be reused - Recommended WF: - Wait for LS reply on whether there is impact on SMTC. - Discuss whether to define requirements at transition for L3 measurement base on LS reply.

In fact, the LS from RAN2 [7] indicates SSB adaptation is based on the additional SMTC configuration for SCell for serving cell measurements only. It may be used for SCell activation, e.g. to fasten the measurement, and after SCell activation, e.g. to slow down the measurement, until the SCell is deactivated.

If SSB periodicity in SCell is adapted, then measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower.

Proposal 11: 	RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower.

  - SCell activation requirements

For measurements related to SCell activation, RAN4 #114 achieved following agreement:

Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG

For SCell activation in RRC_CONNECTED state, with current requirements specified in TS 38.133, clause 8.3.2, SSB periodicity adaptation to lower SSB periodicity can be used to fasten the serving cell measurements required to perform SCell activation based on lower time of TFirstSSB. This includes the case that NCD-SSB based measurements are used in the SCell, if the NW indicates it as reference SSB.

If the SSB adaptation command comes together with SCell activation command in the RRC message, the SCell activa-tion delay may be somewhat higher due to additional processing for SSB adaptation as part of Tactivation_time, such as

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1.

However, no SSB adaptation delay is needed in this case, as discussed in next section. In this case, the lower SSB periodicity, part of the additional SMTC configuration, is immediately used after SCell activation and will fasten L3 measurements in the activated SCell.

Proposal 12: 	RAN4 to define measurement requirements for SCell activation in case of concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1.

  - SSB adaptation delay requirements

RAN4#114 did not further discuss the impact to other requirements due to SSB adaptation as provided in the recommended way forward [2].

Issue 1-4-3: Other requirements impacts by SSB adaptation - Proposals - Option 1: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1/RAN2, to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay. (Nokia) - Option 2: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi) - Recommended WF: - Discuss above proposal

In fact, as SSB adaptation represents a network-controlled procedure, as agreed under issue 1-1-3 at RAN4 #113, see Annex 1, the UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on DCI, MAC-CE or RRC). Hence some processing time needs to be defined, until the UE is ready to receive SSBs with the adjusted SSB burst periodicity, which can be identified as SSB adaptation delay.

Proposal 13: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1 (DCI), RAN2 (MAC-CE) and RAN4 (RRC), in order to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay.

As RAN2 is supposed to include SSB adaptation command and OD-SSB activation command in the same MAC-CE message, MAC-CE based processing delay is assumed to be same as for OD-SSB.

Proposal 14: 	RAN4 to define the MAC-CE based processing delay for SSB adaptation same as for OD-SSB.

- Conclusion

In this contribution, as part of the objective on “adaptation of common signal/channel transmissions” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed scenarios for investigating the RRM impact from SSB periodicity adaptation in time domain based on RAN1/2 progress and RAN4#114 agreements and presented our views on the potential RRM impact.

Following observation is made.

Observation 1: 	The network must ensure that the adapted SSB periodicity in SCell is not larger than the legacy SMTC periodicity.

And following proposals are made.

Scenarios for SSB adaptation

Proposal 1: 	RAN4 to consider RRM impacts from SSB adaptation in SCell for NCD-SSB only.

Proposal 2: 	RAN4 to specify RRM requirements for SSB adaptation based on the existing range of SSB burst periodicities (5ms to 160ms) for SSB periodicity adaptation as the only changeable parameter.

Proposal 3: 	RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered DCI and MAC-CE signaling methods, designed by RAN1 and RAN2.

Proposal 4: 	RAN4 to consider the scenario for SSB adaptation command sent together with direct SCell activation command and investigate the impact to SCell addition requirements and align them to OD-SSB based direct SCell activation as much as possible.

Layer 1 measurement impact

Proposal 5: 	RAN4 to consider the impact from SSB periodicity adaptation to Layer 1 measurements such as CBD, L1-RSRP and L1-SINR for active SCells during transition periods only.

Proposal 6: 	RAN4 to agree the measurement period for CBD, L1-RSRP and L1-SINR prior to the SSB received at Tfirst_SSB is based on the SSB periodicity before adaptation.

Proposal 7: 	RAN4 to agree following use case scenarios for specifying L1 transition requirements are distinguished:

-	Scenario 1: SSB adaptation to lower periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 2: SSB adaptation to lower periodicity with late SSB adaptation command received after completed L1 measurement period.

-	Scenario 3: SSB adaptation to higher periodicity with early SSB adaptation command received after completed L1 measurement period.

-	Scenario 4: SSB adaptation to higher periodicity with late SSB adaptation command received after completed L1 measurement period.

Proposal 8: 	RAN4 not to consider any impact to measurement requirements from L1 reporting for SSB adaptation, until further notice from RAN1.

Layer 3 measurement impact

Proposal 9: 	RAN4 to proceed with the assumption that the additional SMTC configuration carries a set of SSB periodicities, from which one will be activated in the SSB adaptation command and only one active at any time in the SCell.

Proposal 10:	RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG.

L3 transition requirements

Proposal 11: RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower.

SCell activation requirements

Proposal 12: 	RAN4 to define measurement requirements for SCell activation in case of concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1.

SSB adaptation delay requirements

Proposal 13: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1 (DCI), RAN2 (MAC-CE) and RAN4 (RRC), in order to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay.

Proposal 14: 	RAN4 to define the MAC-CE based processing delay for SSB adaptation same as for OD-SSB.

- References

[1] 	RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson

[2]	 R4-2500538, “Topic summary for [114][224] Netw_Energy_NR_enh_Part2”, Moderator (Huawei)

[3]	R4-2502689, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei

[4] 	R4-2502598, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

[5]	R4-2502240, “RRM requirements for SSB adaptation”, Nokia, Nokia Shanghai Bell

[6]	R4-2416911, “LS on SSB adaptation”, RAN4, RAN4#112bis

[7]	R2-2501484, “Reply LS on SSB adaptation”, RAN2, RAN2#129

- Annex 1: Previous RAN4 agreements on SSB adaptation

RAN4 has discussed the SSB adaptation enhancement in their last meetings. At RAN4#113 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Issue 1-1-2: Scenario clarification- SSB periodicity Agreement: - Postpone the discussion and wait for RAN1 progress. Issue 1-1-3: Scenario clarification- SSB adaptation triggering Agreement: - RAN4 to focus on NW controlled SSB adaptation firstly. Issue 1-1-4: Scenario clarification- SSB adaptation mechanism Agreement: - Postpone the discussion and wait for RAN1 progress. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC.

At RAN4#112bis meeting, the following has been agreed on RRM requirements for SSB adaptation.

### Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

At RAN4#112 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

- Annex 2: RAN1 agreements on SSB adaptation

RAN1 has discussed the SSB adaptation enhancement in their last meetings. At RAN1#120 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#120) Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for Scell Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

At RAN1#119 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#119) Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode).

At RAN1#118bis, the following conclusion was reached:

Agreements (RAN1#118bis) [Adaptation of SSB in time domain] Conclusion​ There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19​. Proposal 2.1.4-rev1​ For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).​ - No spec impact

At RAN1 #118 meeting, the following agreements were reached:

Agreements (RAN1#118) [Adaptation of SSB in time domain] Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB