# 3GPP TSG-RAN WG4 Meeting #114bis		        R4-2504555

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	6.5.3

Source: 	Huawei, HiSilicon

Title:	On HPUE PC2 requirements

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in the last meeting to progress the PC2 requirements for band n26 as well as merge the 1Tx and 2Tx PC2 A-MPR for NS_48 and NS_49.

Issue 1: n26 PC2

There are still A-MPR requirements that need to be defined for n26 PC2, specifically NS-12, NS-13, NS-14, NS-15 and RSD.

- Set the analysis (including 3MHz) in the draft CR R4-2503019 as baseline.

- These NS and RSD values need to be defined based on the analysis results from TR 38.896, and any additional relevant input.

Issue 2: A-MPR clean-up

For NS_48 and NS_49, and to account for PC2 2Tx rev-IMD, further study the approach below:

- Void Note 7 in Table 6.2.3.1-1,

- Replace the [2] dB PC2 2Tx A-MPR relaxation with a maximum of 0.5dB relaxation for cases where the specified PC2 (1Tx) A-MPR < PC3 A-MPR +3dB,

- To avoid tightening the Rel-17 and Rel-18 requirements, these changes are applicable to Rel-19 only.

# 2	Discussion

Issue 1: n26 PC2

We have examined the candidate requirements in the draft CR R4-2503019 and found them agreeable.

Proposal 1: It’s feasible to adopt the PC2 requirements in draft CR R4-2503019 for band n26.

Issue 2: A-MPR clean-up for NS_48 and NS_49

The new PC2 A-MPR requirements have been proposed in [3] and copied below:

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | QPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 16 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 64 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 256 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |

Proposal 2: It’s feasible to adopt the PC2 A-MPR requirements for NS_48 and NS_49 in R4-2501984 for both 1Tx and 2Tx.

On the other hand, Note 7 in Table 6.2.3.1-1 also applies to NS_05 and NS_05U, for which new A-MPR requirements are under discussion in a separate thread. To avoid confusion, it’s better not to void Note 7 before work on NS_05(U) is completed.

Proposal 3: Remove Note 7 for NS_48 and NS_49 from Table 6.2.3.1-1 when updating the corresponding PC2 A-MPR requirements. Do not void Note 7 before the work on NS_05(U) is completed.

# 3	Conclusion

Regarding the PC2 requirements for band n26 and n1, the following proposals are made:

Proposal 1: It’s feasible to adopt the PC2 requirements in draft CR R4-2503019 for band n26.

Proposal 2: It’s feasible to adopt the PC2 A-MPR requirements for NS_48 and NS_49 in R4-2501984 for both 1Tx and 2Tx.

Proposal 3: Remove Note 7 for NS_48 and NS_49 from Table 6.2.3.1-1 when updating the corresponding PC2 A-MPR requirements. Do not void Note 7 before the work on NS_05(U) is completed.

# References

- R4-2502825 WF on HPUE for single NR bands, CMCC, RAN4#114

- R4-2503019 Draft CR to 38.101-1: Introduction of requirements for PC2 operation in band n26, Ericsson, RAN4#114

- R4-2501984 NS_05 NS_48 NS_49 PC2 A-MPR clean-up, Skyworks, RAN4#114