3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504585

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.10.2.2.2

Source:	Ericsson

Title:	On the UE RF Rx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Rx requirements for NR-NTN HPUE from the approved WF in RAN4#114 [2].

# Discussion

In the approved WF [2], some of the open issues which are for further study are the following:

Issue 3-2: RSD requirements for PC1.5

- Proposals

  - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo)

  - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek)

Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [1.8] | [1.8] | [1.8] | [1.8] |
| n255 | [2.4] | [2.4] | [2.5] | [2.5] |
| n254 | [1.8] | [1.8] | [1.8] |  |

Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [2.2] | [2.2] | [2.2] | [2.2] |
| n255 | [3.2] | [3.2] | [3.3] | [3.3] |
| n254 | [2.2] | [2.2] | [2.2] |  |

  - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson)

Way forward:

- Further discuss above options.

Issue 3-3: RSD requirements for PC1

- Proposals

  - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. And FFS PC1 2Tx or 3Tx. (vivo, Samsung)

  - Option 2: Regarding NR NTN HPUE operated in PC1, discuss whether the RX RSD values as shown in tables below could be used for band n256.

Table 6 RSD from PC3 to PC1 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |

Table 7 RSD from PC3 to PC1 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3.7] | [3.7] | [3.7] | [3.7] |

  - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson)

Way forward:

- Further discuss above options.

For TN, the RSD requirements are not specified for PC1.5 and PC1 [3]. FDD bands do not support PC1.5, while PC1 UEs are not targeted for smartphone form factor.

For the NTN, in the previous meeting it was agreed to define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19 (other cases are FFS). Since only non-handheld UEs will be considered for PC1.5, even though 2Tx architecture will be assumed, in our view no RSD should be specified because for TN, no RSD was specified for PC1, for which the maximum output power is 2dB larger than for PC1.5.  no will no RSD for PC1.5 and PC1 can be taken as the starting point.

For PC1, it was previously agreed only to consider 1Tx architecture and non-handheld UEs for NTN (4Tx architecture is still on the table). For such case, no RSD should be specified which is in line with TN, where also 1Tx architecture was assumed and non-handheld UEs only.

Proposal 1: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1.

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2502954, “WF on HPUE requirments for NR-NTN”, Samsung, RAN4#114.

- 3GPP TS 38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”