3GPP TSG-RAN WG4 Meeting #114bis	R4-2504605

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.26.4.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for LP-WUR

Document for:	Discussion

# Introduction

RAN4 has been discussing various scenarios and issues related to RRM requirements for LP WUR/WUS over the past few meetings and achieved many agreements [2]-[7]. Furthermore, RAN4 received an LS from RAN1 [8] on the MR wake-up delay. In this paper, we present our views on RAN1 assumptions on the wake-up delay and other open issues.

# MR wake-up delay

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

The first issue that we want to address is related to MR wake-up delay in IDLE/INACTIVE mode. RAN1’s agreement from the LS [8] is copied below:

It’s clear from RAN1 discussions and agreements that the MR wake-up delay constitutes of two components, MR warm up delay and synchronization. The three candidate values for the wake-up delay assume 10ms, 400ms and 800ms and 3, 5 and 5 SSBs for synchronization delay respectively. RAN1 also mentions that the wake-up delay is the minimum time gap between LP-WUS reception and MR to start monitoring PDCCH. Since the WUS decoding or processing may also take non-negligible time, we think that should also be included in the overall MR wake-up delay.

Observation 1: RAN1 only assumed MR ramp-up time and synchronization time in the wake-up delay.

Proposal 1: RAN4 to discuss whether LP-WUS processing/decoding delay should also be considered as a part of the MR wake-up delay.

Secondly, the UE performs measurements based on SMTC periodicity and not based on SSB periodicity, so that needs to be clarified to RAN1.

Observation 2: UE performs measurements for synchronization based on SMTC periodicity and not based on SSB periodicity.

Proposal 2: RAN4 to discuss whether SMTC periodicity should be considered instead of SSB periodicity for synchronization delay.

Thirdly, we wonder why different number of SSBs (3 and 5) are needed for different warm-up delays. Since we are not expecting any different synchronization behavior on the MR, it’s not clear under what circumstances the UE may use more SSBs than other.

Proposal 3: RAN4 to discuss the need to have different number of SSBs (3 and 5) for different warm-up delays.

We think these are some of the questions that RAN4 should discuss and address before agreeing to the delay values proposed by RAN1.

# LP-WUR operation in Connected mode

The next issue is related to WUR implementation. Although 3GPP may assume that the WUR is completely detached from the MR, it’s highly likely that UE may share some RF components between the two radios. So, for CONNECTED mode, when the UE uses measurement gaps for neighbour cell measurements, it may tune away from the serving cell. This typically causes interruptions on the UE where the UE cannot transmit or receive any signal other than those intended for RRM measurements. We think such interruptions may also be needed for WUS monitoring when the WUS MO collides with the MG. Similarly, there are other interruption requirements defined in Clause 8.2 of 38.133 where the UE is not expected to transmit or receive any thing. These interruption requirements should also apply to WUS monitoring by the WUR in Connected mode.

Observation 3: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 4: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs.

Proposal 5: The interruption requirements specified in Clause 8.2 of TS 38.133 also apply to WUS monitoring by WUR in the Connected mode.

# Conclusion

Observation 1: RAN1 only assumed MR ramp-up time and synchronization time in the wake-up delay.

Proposal 1: RAN4 to discuss whether LP-WUS processing/decoding delay should also be considered as a part of the MR wake-up delay.

Observation 2: UE performs measurements for synchronization based on SMTC periodicity and not based on SSB periodicity.

Proposal 2: RAN4 to discuss whether SMTC periodicity should be considered instead of SSB periodicity for synchronization delay.

Proposal 3: RAN4 to discuss the need to have different number of SSBs (3 and 5) for different warm-up delays.

Observation 3: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 4: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs.

Proposal 5: The interruption requirements specified in Clause 8.2 of TS 38.133 also apply to WUS monitoring by WUR in the Connected mode.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103

[2] R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#110bis

[3] R4-2410296, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#111

[4] R4-2413899, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112

[5] R4-2416861, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112bis

[6] R4-2420100, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#113

[7] R4-2502600, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#113

[8] R1-2501624, “LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode”, Apple, RAN1#120