3GPP TSG-RAN WG4 Meeting #114bis	R4-25xxxxx

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussions on spatial channel models

Source: 	Huawei, HiSilicon

Agenda item:	7.16.2

Document for:	Discussion

Background

In last meeting, WF R4-2419782 [1] related to spatial channel models was approved, this contribution provides our views.

- TDL based approach

There are two candidate options for enhanced TDL channel model. One is proposed in [2], denoted as cluster model 1, the other is proposed in [3], denoted as cluster model 2. This clause provides our views on these two models.

Cluster model 1

Comparison with legacy channel

Cluster model 1 is simple extension of existing dual cluster model with beam steering defined in 38.101-4.  We did a simulation to verify the performance for Rank4 and Rank8 and compare the performance with legacy channel model.

Simulation assumptions are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for comparison of cluster model 1 with legacy channel

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1) for 4Tx, (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Random precoder |
| Rank | 4,8 |
| Antenna configuration | 4T4R for Rank4, 8T8R for Rank8 |
| Channel | Legacy TDLC300-100 XPL medium Legacy TDLC300-100 XPL high Cluster model 1 with 2 clusters for Rank4 Cluster model 1 with 4 clusters for Rank8 |
| MCS | 13 |
| Cluster powers | For 2 clusters: p1, p2=[0dB, 0dB] For 4 clusters: p1, p2 , p3 p4 =[0dB, 0dB, 0dB, 0dB] |
| Cluster TX steering in 1st array dim | For 2 clusters: θtx1(1)=0, θtx1(2)=π For 4 clusters: θtx1(1)=0, θtx1(2)=π/2, θtx1(3)=π, θtx1(4)=3π/2 |
| Cluster RX steering in 1st array dim | For 2 clusters: θrx1(1)=0, θrx1(2)=π For 4 clusters: θrx1(1)=0, θrx1(2)=π/2, θrx1(3)=π, θrx1(4)=3π/2 |

Figure 2.1-1: Simulation results for comparison of cluster model 1 with legacy channel model

It can be observed that legacy TDL channel has rank deficiency and cluster model 2 can improve the spatial freedom degree, from the simulation it can be observed that 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 1: For cluster model 1, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Configurability

The power ratio of four clusters is a variable which can adjust per CW/layer SINR to the desired level. For example, fixed precoder i1,1=i1,2=i2=0 can gather the signal of each rank on special direction, which can be aligned with the direction of pre-configured clusters with pre-configured power, in this case, equal power for all clusters lead to same performance for each layer, unequal power leads to difference performance. We did a simulation to verify this characteristic with the simulation assumptions in Table 2.1-2. In the simulation, two cases are simulated, case1: p1,p2,p3,p4=[0dB, 0dB, 0dB, 0dB] and case2: p1,p2,p3,p4=[0dB, 0dB, -9dB, -9dB] with fixed precoder i1,1=i1,2=i1,3=0. It can be observed that for case1, two codeword has same performance and for case2, two codeword have quite different performance, the reason is that precoder maps the signal of second codeword to last two clusters with low power.

Table 2.1-2: Simulation assumptions for verification on configurability

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Fixed precoder i1,1=i1,2=i1,3=0 |
| Rank | 8 |
| Antenna configuration | 8T8R |
| Channel | Cluster model 1 with 4 clusters |
| MCS | 13 |
| Cluster powers | Case 1: p1,p2,p3,p4=[0dB, 0dB, 0dB, 0dB] Case 2: p1,p2,p3,p4=[0dB, 0dB, -9dB, -9dB] |
| Cluster TX steering in 1st array dim | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

Figure 2.1-2 Simulation results for verification on configurability

Observation 2: The configurability of cluster model 1 is reflected in the fact that power ratio of clusters is a variable to control the SINR imbalance of layers to the desired level.

Cluster model 2

Introduction

The idea of cluster model 2 is to implement multi clusters in one TDL channel, where each cluster corresponds to some taps and each cluster is beam steering of other clusters with steering matrixes in B.2.3.2.3A of 38.101-4 The formula is:

ht,τn=Dθk1,k2rx,n*ht,τn*Dθk1,k2tx,n

Where ht,τn (NRx x NTx) is channel coefficient for tap n in time domain with legacy correlation matrix applied, n is tap index and n=0,1,2,…,N-1, N is number of tap, i.e. 12 for simplified TDL channel in 38.101-4. Pn normalized power of tap n. Dθk1,k2rx,n and Dθk1,k2tx,n are Rx beam steering matrix and Tx beam steering matrix respectively for tap n. ht,τn is final channel coefficient for tap n after applying beam steering.

Dθk1,k2rx,n=1001⨂Dθk1rx,nNrx1⨂Dθk2rx,nNrx2

Dθk1,k2tx,n=1001⨂Dθk1tx,nNtx1⨂Dθk2tx,nNtx2

-	Dθk1rx,nN1, Dθk1tx,nN2is the steering matrix for tap n in first dimension with same polarization for Rx beam and Tx beam respectively

-	Dθk2rx,nN1, Dθk2tx,nN2is the steering matrix for tap n in second dimension with same polarization for Rx beam and Tx beam respectively

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	For antenna array with only one direction, number of antenna element in second direction equals 1，

For 1 antenna element of the same polarization in one direction, .

For 2 antenna elements of the same polarization in one direction,Dθk,i2=100ejθk,i.

For 4 antenna elements of the same polarization in one direction, .

Comparison with legacy channel

To check the performance of cluster model 2 and compare it with legacy channel model, a simulation was done with simulation assumptions captured in Table 2.2-1:

Table 2.2-1: Simulation assumptions for comparison of cluster model 2 with legacy channel

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1) for 4Tx, (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Random precoder |
| Rank | 4,8 |
| Antenna configuration | 4T4R for Rank4, 8T8R for Rank8 |
| Channel | Legacy TDLC300-100 XPL medium Legacy TDLC300-100 XPL high Cluster model 2 in Table 2.2-3 for Rank4,TDLC300-100, XPL high for each cluster Cluster model 2 in Table 2.2-4 for Rank8, TDLC300-100, XPL high for each cluster |
| MCS | 13 |

Table 2.2-2: Cluster model 2 with 2 TX-RX clusters

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θk1=0 θk2=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 |  |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 | Cluster 2 θk1=π θk2=π |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

Table 2.2-3: Cluster model 2 with 4 TX-RX clusters-type1

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θk1=0 θk2=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θk1=π/2 θk2=π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 | Cluster 3 θk1=π θk2=π |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 | Cluster 4 θk1=3π/2 θk2=3π/2 |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

It can be observed that legacy TDL channel has rank deficiency and cluster model 2 can improve the spatial freedom degree, similar observation as cluster model 1, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 3: For cluster model 1, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Figure 2.2-1: Simulation results for comparison of cluster model 1 with legacy channel model

Configurability

The mapping between clusters and taps can be a variable to control the per CW/layer SINR to the desired level. For example, mapping one cluster to a group taps with low power and large delay leads to poor performance for the layer corresponding to this cluster and mapping one cluster to a group of taps with high power and small delay can lead to good performance for the layer corresponding to this cluster. Same time, fixed precoder is used to project the signal of each layer to specific cluster. A simulation was done to verify this characteristic, two types of cluster-tap mapping were assumed. Type 1 was captured in Table 2.2-4, type 2 was captured in Table 2.2-6. From the simulation results it can be observed that for type 1, the power of each cluster varies slightly, leading to not large performance difference between two codewords. For type 2, 2nd cluster and 3rd cluster are mapped to last four taps with low power and large delay, leading to big performance difference between two codewords. Noted that fixed precoder i1,1=i1,2=i1,3=0 project signal corresponding to first codeword to first 2 clusters and second codeword to last 2 clusters.

Table 2.2-4: Simulation assumptions for verification on configurability

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Fixed precoder i1,1=i1,2=i1,3=0 |
| Rank | 8 |
| Antenna configuration | 8T8R |
| Channel | Cluster model 1 with 4 clusters, XPL high for each cluster, TDLC300-100 |
| MCS | 13 |
| Cluster TX steering in 1st array dim | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

Table 2.2-5: Cluster model 2 with 4 TX-RX clusters-type2

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θk1=0 θk2=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θk1=-π/2 θk2=π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 | Cluster 3 θk1=-π θk2=-π |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 | Cluster 4 θk1=-3π/2 θk2=-3π/2 |
| 12 | 2595 | -16.0 |  |

Figure 2.2-2 Simulation results for verification on configurability

Observation 4: The configurability of cluster model 2 is reflected in the fact that mapping between clusters and taps is a variable to control the SINR imbalance of layers to the desired level

Cluster model 3

The pros and cons of two cluster model are summarized as following:

|  | Cluster model 1 | Cluster model 2 |
| --- | --- | --- |
| Pros | More configurable, performance gap between two codewords is predictable can be adjusted the any desired level. | No complexity introduced compared to legacy TDL channel |
| Cons | More complicate, the complexity increases linearly as number of clusters increase | Less configurable, the trend but not exact of gap between two codewords is predictable and configurable. |

Then next a new extended TDL channel model is proposed harmonizing the cluster model 1 and cluster 2. The key idea is that instead of implementing each cluster in one TDLC channel or all clusters in one TDL channel, all clusters are split into fixed two TDL channels and specific PMI projects two CWs to clusters corresponding to two TDL channels respectively, which means signal of two CWs will pass two TDL channels respectively and power ratio of two channels is introduced to adjust the performance of two codewords to the exact desired level. Take following table as example:

Table 2.3-1: Cluster configuration for each TDL channel

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam for TDL channel 1 p1 | Tx-Rx Beam for TDL channel 2 p2 |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx11=0 θtx21=0 θrx11=0 θrx21=0 | Cluster 3 θtx13=-π θtx23=-π θrx13=-π θrx23=-π |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 |  |  |
| 5 | 195 | -2.4 | Cluster 2 θtx12=-π/2 θtx22=-π/2 θrx12=-π/2 θrx22=-π/2 | Cluster 4 θtx14=-3π/2 θtx24=-3π/2 θrx14=-3π/2 θrx24=-3π/2 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 |  |  |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Since at most 8 layers will be tested so two TDL channels with 4 clusters is enough to cover all potential tests. The advantage of this new approach is that it keeps the balance of complexity and configurability since no additional complexity is introduced compared to existing dual cluster model in 38.101-4 and is more configurable compared to cluster model 2.

Please see the simulation verifying the characteristic of cluster model 3:

We propose the following for alignment and comparison for cluster model 2 in study stage:

Proposal 1: Use following for alignment and comparison for cluster model 2 in study stage:

4T4R, 4Layers:

- For alignment: Table 2.2-2,

- For comparison: Compare channel in Table 2.2-2 with TDLC300-100 XPL high and XPL medium.

8T8R, 8Layers:

- For alignment: Table 2.2-3 and Table 2.3-1 Random precoder

- For comparison (Full throughput): Compare channel in Table 2.2-2 with TDLC300-100 XPL medium, random precoder

- For comparison (Per CW): Channel in Table 2.2-3 and Table 2.2-5, Table2.3-1 (Cluster model 3) fixed precoder with i1,1=i1,2=i2=0

- CDL based approach

Generic of CDL

The target frequency for A3 CDL channel is 6GHz, which is only applicable for TDD band. However, CDL channel parameters such as desired angular spread and doppler highly depends on frequency but demodulation requirements are defined as band agnostic.

Observation 5: The key parameters of CDL channel such as doppler and desired angular spread highly depends on frequency but demodulation requirements are defined as band agnostic.

Our proposal is to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

Proposal 2: RAN4 to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

CDL based approach

We have following candidate options for CDL channel model:

Options: CDL from TR 38.753 will include randomness reduction for the purpose of the SI - CDL (TR 38.753) - Option A1 : with Table 7.2.1-8 in TR 38.827 - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation - Option A3: based on Table 7.7.1-3 in TR 38.901 with truncation According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis.

Our understanding is that channel model A1 is limited to UMa scenario with 7GHz frequency, however, A2 can be modified to any specific scenario with angular scaling operation, which means A2 is more flexible and can cover A1. Therefore, A1 can be dropped. Option A3 truncates half of clusters of A2, which is less complicate for TE implementation. However, on our simulations in [4] shows that optionA3 performs worse than A2, especially for rank8, the reason is that truncating clusters will reduce the spatial degree of freedom. Therefore, there is a risk that option A3 may not represent the desired scenario due to the big performance degradation. Hence  further study on how to improve truncated channel to keep similar performance with non-truncated channel is needed.

Observation 6: Option A3(Truncated CDL model) according to the truncation procedure in [1] will reduce the spatial degree of freedom, which causes big performance degradation especially for Rank8. Such degradation causes a risk that option A3 may not represent the desired scenario.

Proposal 3: RAN4 to further study how to improve truncated channel to keep similar performance with non-truncated channel.

Antenna Array Virtualisation

There are two options for antenna array virtualisation, one is option 1Y, i,e, one CSI port corresponds to one subarray with 8 antenna elements. The other is option 3, i,e, one CSI port corresponds one antenna element.

Downselection to be based upon (1. Best performance, 2. Ease of alignment, 3. Practicality) Options: - Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration - [(M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only] - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports - Option 3: with AAV with the following configuration: - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Our preference is option 1Y, which is widely used by most BS vendor. Furthermore, option 1Y has great compatibility, for example, option 1Y is available for beamforming, which can be utilized for topic such FR2 test, AI and beam management.

Proposal 4:  RAN4 to choose option 1Y, i,e, (8,2,2,8,8,1) for 4Tx and (8,4,2,8,1) for 8Tx for AAV.

SNR definition

We have following agreements on SNR definition for CDL channel:

#### SNR Definition with CDL Agreement from RAN4-113: Define normalised receive signal power as the following: The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain]. The normalisation factor will be aligned and can be included in TR 38.753. Agreement from RAN4-114: Normalization factor is constant over the duration of the simulation

From above the agreement, the normalisation factor shall be derived with the consideration of all potential factors impacting on power including antenna radiation pattern, AAV spatial response and interference between polarized antennas. It’s hard to derive it by mathematical derivation, so the best way is to do measurement in the simulations, in the case, the procedure of measurement shall be unified otherwise it is hard to align the result. Our proposed procedure is shown as following:

Assume H(k,t)∈CNR×NTis channel coefficient in at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. The normalization factor should be:

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation. Given the above normalization factor, the average power at each Tx-Rx channel coefficient is 1. I.e. The average power at each Rx is 1, which reaches the same condition as TDL channel.

Proposal 5: RAN4 to do measurement to derive normalization with following formula:

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

Proposal 6: The measurement results shall be aligned to respect the previous agreement.

Approach to corrlated clusters mitigation

Option A2 and A3 have some clusters with exactly same angular parameters, which may potentially cause the correlation between these clusters. There is an email discussion whether to apply the ray split defined for 38.827 model to A2/A3 model to avoid the correlations.  However, the impact on performance is not clear. It’s proposed to study the impact and make the decision later.

Proposal 7:  RAN4 to further study the whether the configuration that same angular parameters for some clusters impacts the performance.

- Test cases

PMI selection

Following options for PMI selection were made:

#### PDSCH PMI Selection Way Forward: Random PMI already agreed. Regarding potential fixed PMI: - Option 1: Random PMI from sub-set - Option 2: Collectively chosen fixed PMI - Option 3: Independently chosen fixed PMI Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

The concern for fixed PMI is that best PMI highly depends on channel realization and test configurations, it will be a huge work for best PMI selection if diverse test cases are introduced in future. Hence, our understanding is that RAN4 to choose random PMI as first priority and consider fixed PMI for the cases that random PMI cannot work. According to our simulation results by using fixed subarray AAV, 70% of maximum throughput is achievable for rank4 but unachievable for rank8, hence fixed PMI can be used for Rank8. Furthermore, the fixed PMI shall be unified for easy TE implementation. Hence, we propose the following:

Proposal 8: RAN4 to only use following PMI configuration:

Rank4: Only random PMI

Rank8: Fixed PMI

PMI for alignment of SCM

There is a discussion on granularity for PMI for single panel I PMI:

FFS Subband or Wideband for Type I

We support wideband PMI since our study shall target for requirements definition but subband PMI is hardly ever used for type 1 PMI requirements definition.

Proposal 9: RAN4 to use wideband PMI for type I codebook.

Comparison with legacy TDL

Following options were made for legacy TDL channel:

#### Comparison with Legacy TDL Agreement: The following Legacy TDL models for comparison: - Option 1: TDLC 300-10/100 with following correlation: - Option 1a: Low - Option 1b: XP Medium - Option 1c: XP High - Option 1d: Medium

We support option 1b since XP Medium is the best spatial channel model among all legacy channel models defined in 38.101-4, the correct logic is that improved spatial channel shall be compared with best legacy channel model and show the benefits under the comparison.

Proposal 10: RAN4 to use XP Medium as baseline channel for comparison.

- Conclusion

This contribution provides our views on spatial channel model, the proposals are:

Observation 1: For cluster model 1, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 2: The configurability of cluster model 1 is reflected in the fact that power ratio of clusters is a variable to control the SINR imbalance of layers to the desired level.

Observation 3: For cluster model 1, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 4: The configurability of cluster model 2 is reflected in the fact that mapping between clusters and taps is a variable to control the SINR imbalance of layers to the desired level

Proposal 1: Use following for alignment and comparison for cluster model 2 in study stage:

4T4R, 4Layers:

- For alignment: Table 2.2-2,

- For comparison: Compare channel in Table 2.2-2 with TDLC300-100 XPL high and XPL medium.

8T8R, 8Layers:

- For alignment: Table 2.2-3, Random precoder

- For comparison (Full throughput): Compare channel in Table 2.2-2 with TDLC300-100 XPL medium, random precoder

- For comparison (Per CW): Channel in Table 2.2-3 and Table 2.2-5, fixed precoder with i1,1=i1,2=i2=0

Observation 5: The key parameters of CDL channel such as doppler and desired angular spread highly depends on frequency but demodulation requirements are defined as band agnostic.

Proposal 2: RAN4 to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

Observation 6: Option A3(Truncated CDL model) according to the truncation procedure in [1] will reduce the spatial degree of freedom, which causes big performance degradation especially for Rank8. Such degradation causes a risk that option A3 may not represent the desired scenario.

Proposal 3: RAN4 to further study how to improve truncated channel to keep similar performance with non-truncated channel.

Proposal 4:  RAN4 to choose option 1Y, i,e, (8,2,2,8,8,1) for 4Tx and (8,4,2,8,1) for 8Tx for AAV.

Proposal 5: RAN4 to do measurement to derive normalization with following formula:

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

Proposal 6: The measurement results shall be aligned to respect the previous agreement.

Proposal 7:  RAN4 to further study the whether the configuration that same angular parameters for some clusters impacts the performance.

Proposal 8: RAN4 to only use following PMI configuration:

Rank4: Only random PMI

Rank8: Fixed PMI

Proposal 9: RAN4 to use wideband PMI for type I codebook.

Proposal 10: RAN4 to use XP Medium as baseline channel for comparison.

- Reference

[1]  R4-2502378 Way Forward for [113][320] NR_SCM. Nokia

[2]  R4-2500337 TP to TR38.753: TDL approaches and related Annex. MediaTek Inc.

[3]  R4-2501738 Discussions on spatial channel models. Huawei, HiSilicon.

[4]  R4-25xxxxx Simulation results for spatial channel models