3GPP TSG-RAN WG4 Meeting #114bis	R4-2504708

Wuhan, China, April 7 – 11, 2025

Title: 	Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF

Agenda Item:	7.29.5

Source: 	Qualcomm Inc.

Document for:	Approval

## Issue 1-1: Output power enhancement for NR NTN PC3

Agreements:

- If defined, additional output power enhancement will be optional for UE to support.

  - FFS on need of new UE capability

- Evaluate potential to further enhance output power for RAN4#115:

  - Amount of output power enhancement [0.5 to 1 dB]

    - Less than 0.5 dB improvement is not specified

  - Initially consider all RB allocations and modulation orders

    - Consider both MPR reduction and MOP increase

      - FFS how to capture MOP increase in the specification

    - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders

  - Evaluate both HD-FDD and FD-FDD

    - Consider at least insertion loss impact

    - FFS whether to apply same enhancement to both HD-FDD and FD-FDD

  - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously

    - FFS whether simultaneous support is specified

    - Consider whether PC3 or PC2 ACLR requirement should apply

  - FFS Applicability to PC3 non-RedCap NR NTN

## Issue 1-3: draft CR

Agreement:

- Consider draftCR in R4-2504551 as baseline for further improvements and possible additions