3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504747

Wuhan, China, 7th – 11st April, 2025

Agenda item:			7.23.4

Source:	Moderator (Huawei)

Title:	Ad-hoc meeting minutes for [114bis][307] NR_duplex_evo_BSRF

Document for:	Information

Introduction

This document is provided for the moderator summary on NR_duplex_evo_BSRF for Rel-19 work item on evolution of NR duplex operation (SBFD), in which the following agenda items are supposed to be covered specifically:

7.23.2.2	Modification of existing Tx requirements for FR1 and FR2-1	[NR_duplex_evo-Core]

7.23.2.3	Modification of existing Rx requirements for FR1 and FR2-1	[NR_duplex_evo-Core]

Note: R4-2504300 in 7.23.2 is included in the thread.

Topic #1: Modification of existing requirements - TX

Companies’ contributions summary

Skipped since all Tdocs are summarized in Topic#1 already.

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2503287 | Tejas Network Limited | Discussion on existing Tx requirements for SBFD operation |
| R4-2503403 | CMCC | Discussion on existing Tx requirements for SBFD |
| R4-2503467 | CATT | Discussion on modification of existing Tx requirements for SBFD operation |
| R4-2504009 | Ericsson | - Impact on SBFD BS RF TX requirements |
| R4-2504195 | Nokia | On existing BS RF TX requirements for SBFD operation |
| R4-2504220 | Samsung | On the modification of existing TX requirements for SBFD-capable BS |
| R4-2504301 | Huawei, HiSilicon | On modification of existing TX RF requirements for SBFD |
| R4-2504340 | Qualcomm Germany | Views on existing BS RF requirements for SBFD |
| R4-2504370 | ZTE Corporation, Sanechips | Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS |
| R4-2504300 | Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom | Deployment scenarios for FR1 SBFD macro BS |

Open issues summary

Sub-topic 1-1: Deployment scenarios

- Issue 1-1-1: Deployment scenarios for FR1 SBFD macro BS

- Proposals:

  - Proposal (Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom):

At least for FR1 macro BS it is confirmed the deployment scenario of SBFD-SBFD co-existence in adjacent channels shall be within the scope of Rel-19.

- Moderator Recommendation:

  - It is a common issue for both TX and RX so it should be discussed firstly.

  - To discuss whether the proposal is agreeable

Ad-hoc discussion:

Ericsson: This proposed scenario is related to the Case 3 we have studied in Rel-18. During the study, 100% grid shift is the baseline of the SI conclusion. Anyway WI time is limited.

Nokia: Same concern as Ericsson. We see the necessity of this maybe in future WI.

Verizon: About 20% grid-shift, we think this is more related to the real deployment. In reality we normally consider co-located deployment. In terms of operator demand, tightened ACLR can help such deployment in a way adjacent BS for different operators can have better performance. We just want to focus on the scenario with real benefit. In USA, there is one in band n77.

CMCC: We think SBFD-SBFD scenario is needed and it also gives vendors opportunity to polish the implementation. We can consider this case in this release or 6G day 1.

Cable Lab: We agree with Verizon, we need to consider some realistic grid-shifts.

Huawei: Let’s respect operators demand on the scenario. Regarding the grid-shift, we just reuse the existing platform and assumptions but look into different ACIR and grid-shift.

Ericsson: To CMCC, compromise on the grid-shift can be made for the SBFD-TDD instead of SBFD-SBFD. For Huawei’s proposal, you need to have fixed number rather than fully depending on the declaration.

Samsung: We support to include SBFD-SBFD in Rel-19. For ACIR analysis, we think it is quite similar as what we have done in Rel-18.

Huawei: There are more approaches to study the requirements.

Nokia: 30dB ACIR improvement would guarantee the co-ex performance. But we think it is something that very difficult to achieve in reality. We should focus on the SBFD-TDD case considering what we have studied in Rel-18.

ZTE: It’s better to clarify more about the applicable scenarios.

Huawei: To Nokia, in our paper we mentioned about per operator filter and with the help from DL subband as guard band we think it is not difficult to achieve such improved ACIR but not in a direct way.

CATT: We want to make sure no signalling for inter-operator co-ex will be specified in this release.

For offline discussion:

Concerns:

- About the Rel-18 SI scope

Rel-18 SI didn’t cover this scenario. The proponent proposed to reuse Case 3 for SBFD-SBFD scenario, but the Case 3 was the most problematic case, the results from all companies were not conclusive quite well, maybe only the result based on 100% grid-shift was conclusive. The proponent is proposing 20% grid-shift, which was not studied in Rel-18, and needs to be fully studied.

- About the Rel-19 WI work plan

But considering the work plan of this WI, there is not time for the study. More importantly, it is out of the scope of WID.

- About the potential restriction to deployment & how to derive such additional requirement

The SBFD-SBFD requires all operators deploy SBFD at all sites. The configurations are fixed and cannot be changed. The SBFD is static, instead of semi-static, i.e., operators cannot switch from SBFD to regular TDD DL freely or another configuration..

20% GS still means the BSs need to be 60 meters away from each other, and this specific configuration only solves inter-operator interference. Issues caused by self-interference and inter-sector interference still cannot be resolved.

Feedback:

- The Case 3 can be directly reused for SBFD-SBFD case from simulation methodology perspective. Just because RAN4 agreed that TDD BS ACLR/ACS cannot be enhanced, so the deployment scenario for SBFD-TDD is limited. The justification of including SBFD-SBFD case is self-evident. About the GS, 20% or other value can be easily used based on co-ex platform in Rel-18 SBFD SI.

- Per feedback above and the analysis from some contributions in this meeting, this study can be in parallel with the remaining work for this WI.

- The requirement is applied to the SBFD-capable BS. This has no limitation for any configuration change.

Proposed Way forward:

- Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope.  If it is in the scope of Rel-19, companies are also encouraged to provide input on how to derive the additional requirements.

Sub-topic 1-2: Unwanted emissions

- [Moderator] The following agreement is achieved in previous meetings:

- ACLR and OBUE Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. (RAN4#112) WF: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

- Issue 1-2-1: Whether to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS?

- Proposals:

  - Option 1 (Tejas Network, Samsung, Huawei, Qualcomm, ZTE): Yes

    - Proposal 1 (Tejas Network):

      - Define scenario-specific ACLR and OBUE requirements. For SBFD co-located deployments, strengthen ACLR1 and ACLR2 to meet the minimum necessary co-location ACLR values.

      - Implement a CLI mitigation framework to manage gNB-to-gNB interference in adjacent channels. This framework should provide clear guidelines for dynamic interference management and establish co-location requirements specifically designed for SBFD-capable gNBs.

    - Proposal 2 (Samsung): The support of additional ACLR and/or OBUE requirements for FR1 macro BS is manufacture declaration based, for certain gNB.

    - Proposal 3 (Qualcomm): RAN4 to adopt option 2 from RAN4#114 WF, where RAN4 to define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

    - Proposal 4 (ZTE): for SBFD vs SBFD coexistence scenario among different operators, propose to treat the band n104 as 1st priority, for other interested bands, it depends on the operator’s demand.

  - Option 2 (Nokia, Ericsson, CATT): No

    - Proposal 5 (Nokia): No need for RAN4 to specify additional emission requirements.

    - Proposal 6 (Ericsson): RAN4 do not introduce the new ACLR and/or OBUE requirements for FR1 WA BS.

    - Proposal 7 (CATT): There is no need to define requirements for special configuration and scenario.

- Moderator Recommendation:

  - It is related to Issue 1-1-1 and can be discussed together

- Issue 1-2-2: How to define the additional co-existence requirements?

  - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level.

    - Proposal 1(CATT): agree with 1dB degradation of victim BS sensitivity.

    - Proposal 2 (CATT): For two SBFD-capable BSs that are co-located and have adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

  - Option 1A: the co-existence limit -49 dBm/MHz for protection of FR1 BS UL can be reused for additional OBUE requirement.

    - Proposal 3 (Samsung): If the method of new OBUE requirement is considered, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

    - Proposal 6 (Huawei): Reuse the co-existence limit for protection of BS UL.

  - Option 2: RAN4 co-ex simulation

    - Proposal 7 (Huawei): RAN4 co-ex simulation

- Moderator Recommendation:

  - The limit is FFS.

    - Option 1: The assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

  - Option 2: Co-existence simulation.

Sub-topic 1-3: Transmitter signal quality

- Issue 1-2-1: Joint measurement for normal DL symbols/slots and SBFD symbols/slots

- [Moderator] The following is agreed in SI which ask for more study on joint measurement for normal DL symbols/slots and SBFD symbols/slots:

- 10.1.2.4 Transmitted signal quality Regarding the transmitter signal quality, RAN4 agreed that all the existing requirement for frequency error, modulation quality (EVM) and time alignment error (TAE) shall also be applied to BS in SBFD symbols/slots. - Further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots during WI phase.

The following agreement is achieved in RAN4#114:

- 1.3. Joint measurement for normal DL symbols/slots and SBFD symbols/slots WF: - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity.

- Proposals on joint measurement for normal DL symbols/slots and SBFD DL symbols/slots:

  - Option 1 (ZTE): Further discuss on the following aspects

    - Whether the following two slots configured with flexible symbols could be used for EVM measurement.

      - Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or

      - Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols.

    - Further consider the available frame for EVM measurement considering the limited available slots for EVM measurement, otherwise the EVM measurement uncertainty will increase (e.g. to get the equal slots for EVM samples as the legacy EVM measurement).

  - Option 2 (Ericsson, Nokia): separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

- ---------------------------------------------------------Start of Text Proposal -------------------------------------------------------

B.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured.

For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

-	Thus   is calculated using in the expressions above and  is calculated using  in the  calculation.

-	Thus we get:

EVM=maxEVMframe,l,EVMframe,h

For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

- ---------------------------------------------------------End of Text Proposal---------------------------------------------------------

  - Option 3 (Samsung): Text proposal from R4-2419221 could be used as baseline, while considering the following aspects:

    - No impact on existing FDD/TDD BS EVM measurement, in which it is still slot-based;

    - Ni should be the number of resource blocks in DL subband with the considered modulation scheme in symbol i if the symbol is SBFD symbol.

    - Editorial errors should be corrected such as “Ndl_symbol slots”,

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl_symbol symbols where Ndl_symbol is the number of dl symbols in a 10 ms measurement interval. For TDD, let Ndl_symbolsTDD be the number of downlink SBFD symbols or non-SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from Ndl_symbolsTDD symbols of different 10 ms measurement intervals and should have a minimum of Ndl_symbol symbols averaging length where Ndl_symbol is the number of symbols in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1Ndl_symbolTDDNii=1Ndl_symbolTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in symbol i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl_symbol slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×Nsymbol Ndl_symbolsTDD and Nsymbol=14 for 15 kHz SCS, Nsymbol=28 for 30 kHz SCS and Nsymbol=56 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2

- Moderator Recommendation:

  - Discuss on the 3 options

- Tentative offline agreement

  - Further discuss on the following aspects in the performance WI

    - Whether the following two slots configured with flexible symbols could be used for EVM measurement.

      - Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or

      - Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols.

  - Separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

  - Use the following text proposal as starting point

- --------------------------------------------------------Start of Text Proposal ------------------------------------------------------- - B.7 Averaged EVM EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured. For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step. - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements: - Thus is calculated using in the expressions above and is calculated using in the calculation. - Thus we get: EVM=maxEVMframe,l,EVMframe,h For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×NslotNdlTDD and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2 - ---------------------------------------------------------End of Text Proposal---------------------------------------------------------

- 1.2.4 Sub-topic 1-4: Text Proposals for TX

- Issue 1-4-1: Text Proposals

Proposal 1: The following text proposal is endorsed for BS output power requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

12.3.2 Base station output power for SBFD

12.3.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

12.3.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

Proposal 2: The following text proposal is endorsed for BS output power dynamics requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.3.3	Total power dynamic range for SBFD

#### 12.3.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

Proposal 3: To implement agreements, the following Note can be adopted to clarify OFF power requirement shall not be applied for SBFD symbols, i.e.,

- “For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.”

Proposal 4: The following text proposal is endorsed for transmit OFF power (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.3.4	Transmit ON/OFF power

### 12.3.4.1	Transmitter OFF power

#### 12.3.4.1.1	General

For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.

=================== End of Text Proposal ===================

Proposal 5: The following text proposal is adopted for transmitter intermodulation requirement (taking the conducted requirement as example)):

=================== Start of Text Proposal ===================

# 12	Conducted transmitter characteristics for SBFD capable BS

## 12.1	General

## 12.2	Base station output power for SBFD

## 12.3	Output power dynamics for SBFD

## 12.4	Transmit ON/OFF power for SBFD

## 12.5	Transmitted signal quality for SBFD

## 12.6	Unwanted emissions for SBFD

## 12.7	Transmitter intermodulation for SBFD

12.3.7	Transmitter intermodulation

The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied.

=================== End of Text Proposal ===================

Proposal 6: the following clauses skeleton are proposed for TX requirements.

=================== End of Text Proposal ===================

# 12	Conducted transmitter characteristics for SBFD capable BS

## 12.1	General

## 12.2	Base station output power for SBFD

## 12.3	Output power dynamics for SBFD

## 12.4	Transmit ON/OFF power for SBFD

## 12.5	Transmitted signal quality for SBFD

## 12.6	Unwanted emissions for SBFD

## 12.7	Transmitter intermodulation for SBFD

# 13	Radiated transmitter characteristics for SBFD capable BS

## 13.1	General

## 13.2	Radiated transmit power for SBFD

## 13.3	OTA base station output power for SBFD

## 13.4	OTA output power dynamics for SBFD

## 13.5	OTA transmit ON/OFF power

## 13.6	OTA transmitted signal quality for SBFD

## 13.7	OTA unwanted emissions for SBFD

## 13.8	OTA transmitter intermodulation

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - Discussion on the text proposals and work split

- For offline discussion on clause structure, e.g. Base station output power

  - Option 1:

12.3.2 Base station output power for SBFD

  - Option 2:

12.2	Base station output power for SBFD

Note: Specification structure related issues will be handled in General thread.

- Topic #2: Modification of existing requirements - RX

Companies’ contributions summary

Skipped since all Tdocs are summarized in Topic#2 already.

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2503299 | Tejas Network Limited | Views on modification of existing Rx requirements for SBFD |
| R4-2503402 | CMCC | Discussion on existing Rx requirements for SBFD |
| R4-2503468 | CATT | Discussion on modification of existing Rx requirements for SBFD operation |
| R4-2504010 | Ericsson | Impact on SBFD BS RF RX requirements |
| R4-2504196 | Nokia | On existing BS RF RX requirements for SBFD operation |
| R4-2504221 | Samsung | On the modification of existing RX requirements for SBFD-capable BS |
| R4-2504302 | Huawei, HiSilicon | On modification of existing RX RF requirements for SBFD |
| R4-2504371 | ZTE Corporation, Sanechips | Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS |
| R4-2504340 | Qualcomm Germany | Views on existing BS RF requirements for SBFD |

Open issues summary

Sub-topic 2-1: ACS

- Issue 2-1-1: ACS requirement

- [Moderator] The following WF is achieved in RAN4#113:

- ACS requirement Agreement: - FFS on the followings: - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

- Proposals:

  - Proposal 1 (CMCC): Prioritize discussing the scenarios under non-colocation case first.

  - Proposal 2 (CMCC): The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

  - Proposal 3 (Huawei/Nokia/Qualcomm): The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

  - Proposal 4 (CATT): It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

  - Proposal 5 (Ericsson): The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD.

  - Proposal 6 (Ericsson): Derive ACS requirement level based on the simulation results using 5% GS in SI.

  - Proposal 7 (Ericsson): The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

- Moderator Recommendation:

  - Discussion on the proposals

Sub-topic 2-2: In-band blocking

- [Moderator] The following WF is achieved in RAN4#114:

- Issue 2: In-band blocking and additional emission WF: - For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

- Observations/Proposals:

  - Tejas Networks (R4-2503299)

    - For the in-band blocking requirement, adopt a unified blocking threshold for both scenarios. For the grid shift, consider 10% grid shift.

    - For in-band blocking simulations, investigate both the reduced blocker model and the reduced blocker model with CLI handling mechanisms. Specifically, for UMa with a modified noise figure, use P1 = -37 dBm, P2 = -25 dBm, F1 = 8 dB, and F2 = 14 dB.

    - Consider a 3 dB degradation in noise figure when using the reduced blocker model with additional filtering.

    - For the FR1 Urban Macro scenario, increase the in-band blocking requirement (currently -43 dBm) for SBFD wide-area BSs to accommodate elevated levels of in-band blocking power.

    - For the 10% grid shift requirement, the interference power level and in-band blocking requirement should be evaluated after RAN1 studies the applicability of the CLI handling mechanism for adjacent channels or considers any relevant deployment restrictions.

    - For the Urban Macro scenario with 10% grid shifts, the co-channel gNB CLI handling mechanism should be considered for SBFD networks.

    - Establish a unified blocking requirement that accounts for both coexistence scenarios: legacy TDD systems in the adjacent channel and new SBFD systems in the adjacent channel.

    - For the FR1 Urban Micro scenario with 10% grid shifts, the existing in-band blocking requirement of -38 dBm should be increased for SBFD medium-area BSs to account for the elevated levels of in-band blocking power.

  - CMCC (R4-2503402)

    - The results obtained with the parameters of 20% and 50% grid shift can serve as references.

  - CATT (R4-2503468)

    - For FR1 WA BS, there is no need to define requirements for special scenario.

    - We propose to specify in-band blocking requirements based on data at 100% grid shift.

    - The IBB requirements of FR1 micro scenario can reuse existing requirements.

    - The IBB requirements of FR2-1 urban macro scenario can reuse existing requirements.

  - Nokia（R4-2504196）

    - Add the following option to the list of options for in-band blocking derivation for FR1 WA BS:

      - Option 5: Define the requirement by selecting 10% grid-shift value. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

    - Support the derivation of FR1 WA SBFD BS in-band blocking requirement according to Option 5.

    - Based on companies’ simulation results, the existing FR1 WA BS in-band blocking requirement defined in TS 38.104 can’t be re-used for FR1 WA SBFD BS

    - Introduce a new in-band blocking requirement for FR1 WA SBFD BS of -2 dBm.

    - Support the derivation of FR1 MR SBFD BS in-band blocking requirement according to Option 5.

    - Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm.

    - Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

    - The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

    - RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

  - Huawei（R4-2504302）

    - Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

    - As a general requirement, it is proposed to adopt 100% grid shift to derive the in-band blocking level for SBFD-TDD adjacent operator scenario.

    - Using grid shift less than 20% as simulation assumption will result over stringent requirement.

    - For some scenarios which could lead smaller site distance than 20% grid shift, we think it can be addressed by co-location limits.

    - It is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for SBFD-SBFD adjacent operator scenario.

    - It is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

  - ZTE （R4-2504371）

    - For FR1 WA SBFD BS, propose the in-band blocking level as -15dBm.

    - For FR1 MR SBFD, propose to define the in-band blocking requirement as -38dBm under the assumption of without any analog filtering design after the LNA of receiver chain which is close to 100% grid shift in the practical deployment.

    - For Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

  - Samsung (R4-2501970)

    - For FR2 WA BS capable of SBFD operation, the existing in-band blocking requirement shall be reused.

    - RAN4 consider the following two compromise solution to resolve the deadlock for in-band blocking requirement:

      - In-band blocking level is manufacture-declared based: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting).

      - Two sets of requirements defined, by considering 100% and 20% grid shifts to derive in-band blocking level.

  - Ericsson（R4-2504010）

    - The worst-case scenario should be considered when defining in-band blocking requirements, i.e., grid-shift should be as small as possible, and 5% grid-shift is used.

    - Find the 99% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios.

    - Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

    - Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

    - Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

    - The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

    - To further discuss on the methodology to derive the in-band blocking requirement and decide on if it should be based on an absolute value of the in-band blocking power, or on a relative value of the in-band blocking power compared to a declared EISrefsense + 6 dB, which should be further discussed and agreed.

    - Regarding in-band blocking requirement and new requirement on additional emission, another option is to define the in-band blocking requirement with grid-shift 20%, while additional emission requirement is not considered in Rel-19. If regulators decide to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

  - Qualcomm（R4-2504340）

    - RAN4 to reuse legacy FR2-1 WA BS in-band blocking requirement given in Section 10.5.2.3 in TS 38.104.

    - RAN4 to adopt option 2 from RAN4#114 WF, where RAN4 to define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

    - RAN4 to consider grid shift values between 20% and 50% when defining the in-band blocking requirements for SBFD-TDD coexistence case.

Issue 2-2-1: In-band blocking for FR1 WA BS

- Proposals:

  - Option 1 (Ericsson): define the in-band blocking requirement with grid-shift 20%, while additional emission requirement is not considered in Rel-19. If regulators decide to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

  - Option 2 (Huawei, Samsung): Define general requirements for SBFD-TDD adjacent operator scenario with grid-shift 100%. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario with grid-shift 20%.·

  - Option 3(CMCC): define the in-band blocking requirement with 20% and 50% grid shift. Additional emission issue is (not) included in this release. If additional emission issue is not included in this release, operators could make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

  - Option 4 (Samsung): In-band blocking level is manufacture-declared based: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting).

  - Option 5 (Nokia): Define the requirement by selecting 10% grid-shift value. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

  - Option 6 (Ericsson): Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%.

  - Option 7 (CATT): Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%.

  - Option 8 (Tejas Networks): Define in-band blocking requirements based on co-existence simulation results with grid-shift 10%.

  - Option 9 (Qualcomm): RAN4 to define general requirements for SBFD-TDD adjacent operator scenario with grid shift values between 20% and 50%. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

- Moderator Recommendation:

  - It is related to Issue 1-1-1 and can be discussed together

Ad-hoc discussion:

Huawei: if we only consider SBFD-TDD, it is confused why the outcome of SI, which is 100%, cannot be used for the requirement. We don’t see the reason why company is proposing to use a tightened value here but refused to talk about another scenario. For minimum requirement, the principle is normally to consider the worst case. For SBFD-TDD, small grid-shift w/o improved ACIR, the SI outcome is less feasible.

For offline discussion:

Proposed Way forward:

- Option 1

Define the in-band blocking requirement with the same grid-shift [TBD] for both SBFD-TDD adjacent operator scenario and SBFD-SBFD adjacent operator scenario. Additional emission requirement is included in this release.

- Option 2

Define the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19. If a decision is made to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

Issue 2-2-2: In-band blocking for FR2-1 WA BS

- Proposals:

  - Option 1 (Nokia, Samsung, ZTE,CATT, Qualcomm): The current FR2-1 WA BS in-band blocking requirements can be re-used

  - Option 2 (Ericsson): Define in-band blocking interferer signal power to -23 dBm

- Moderator Recommendation:

  - Discuss whether Option 1 is agreeable

Issue 2-2-3: In-band blocking for FR1 Micro BS

- Proposals:

  - Option 1 (CATT): The IBB requirements of FR1 micro scenario can reuse existing requirements

  - Option 2 (Nokia):  Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm.

  - Option 3 (ZTE): propose to define the in-band blocking requirement as -38dBm under the assumption of without any analog filtering design after the LNA of receiver chain which is close to 100% grid shift in the practical deployment

  - Option 4(Ericsson): Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs

- Moderator Recommendation:

  - To discuss the grid shift assumption firstly

Sub-topic 2-3: Receiver intermodulation

- Issue 2-3-1: Necessity of New RX intermodulation requirement with 1 interfering signal

- [Moderator]: the following WF is achieved in RAN4#113:

- 1.9. Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the followings: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

- Proposals:

  - Option 1 (CATT/Huawei/CMCC): IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2 (Nokia): RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal, and to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

- Moderator Recommendation:

  - Discussion on options.

- Issue 2-3-2: Impact on existing RX intermodulation requirement with 2 interfering signals

- [Moderator]: the following WF is achieved in RAN4#112:

- Necessity of New RX intermodulation requirement with 2 interfering signals WF: - FFS on the following: - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

- Proposals:

  - Option 1 (ZTE):

    - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

    - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver.

  - Option 2 (CATT/Huawei): new IMD requirement with 2 interfering signal scenario is not needed.

  - Option 3 (Nokia): RAN4 to consider new RX intermodulation requirement with 2 interfering signals. RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

  - Option 4 (Ericsson): The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

- Moderator Recommendation:

  - Discussion on options.

Sub-topic 2-4: Text Proposals for RX

Issue 2-4-1: Text Proposals

OTA sensitivity

Proposal 1: The following text proposal is adopted for OTA sensitivity requirement:

=================== Start of Text Proposal ===================

## 12.7.2	OTA sensitivity

### 12.7.2.1	BS type 1-H and BS type 1-O

#### 12.7.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

#### 12.7.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and BS channel bandwidth in clause 7.2 shall be interpreted as UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

=================== End of Text Proposal ===================

Dynamic range

Proposal 2: The following text proposal is adopted for dynamic range requirement (by taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.4.3	Dynamic range

#### 12.4.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

#### 12.4.3.2	Minimum requirement for BS type 1-H

The same requirement as clause 7.3.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be interpreted as SBFD UL subband bandwidth.

=================== End of Text Proposal ===================

In-channel selectivity

Proposal 3: The following text proposal is adopted for in-channel selectivity requirement (by taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.4.8	In-channel selectivity

#### 12.4.8.1	General

The same requirement as clause 7.8.1 is also applicable for SBFD operation.

#### 12.4.8.2	Minimum requirement for BS type 1-H

The same requirement as clause 7.8.2 is applicable to in-channel selectivity requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be interpreted as SBFD UL subband bandwidth.

=================== End of Text Proposal ===================

Proposal 4: the following clauses skeleton are proposed for RX requirements.

=================== Start of Text Proposal ===================

# 14	Conducted transmitter characteristics for SBFD capable BS

## 14.1	General

## 14.2	Reference sensitivity level for SBFD

## 14.3	Dynamic range for SBFD

## 14.4	In-band selectivity and blocking for SBFD

## 14.5	Out-of-band blocking for SBFD

## 14.6	Receiver spurious emissions for SBFD

## 14.7	Receiver intermodulation for SBFD

# 15	Radiated receiver characteristics for SBFD capable BS

## 15.1	General

## 15.2	OTA sensitivity for SBFD

## 15.3	OTA reference sensitivity level for SBFD

## 15.4	OTA dynamic range for SBFD

## 15.5	OTA in-band selectivity and blocking for SBFD

## 15.6	OTA out-of-band blocking for SBFD

## 15.7	OTA receiver spurious emissions for SBFD

## 15.8	OTA receiver intermodulation for SBFD

## 15.9	OTA in-channel selectivity for SBFD

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - Discussion on the text proposals and work split.

Note: Specification structure issues will be handled in General thread.