# 3GPP TSG-RAN WG4 Meeting #114bis	                 		                                R4-2504909

# Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.27.3

Source:	Apple

Title:	WF on RRM requirements for NR_Mob_Ph4_Part1

Document for:	Information

# Topic #1: Event triggered L1 measurement reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: (vivo)

  - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions:

    - No indicate TCI change during the evaluation period.

    - No configuration change w.r.t measurement RS during the evaluation period

    - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

- Option 2 (Apple)

  - Seems no need to capture assumptions listed under option-1 into core requirements.

#### Issue 2-2: Event triggered reporting delay for L1 reporting

Agreement:

- Reuse the framework from L3 event triggered reporting. Discuss the details further.

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo)

- Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

#### Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options:

- Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC)

- Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

- Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

#### Issue 2-5: whether and how to coordinate with R19 MIMO

Agreement:

- RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

#### Issue 2-6: L1 filtering assumption in event evaluation

Agreement:

- No L1 filter is assumed for requirements in event triggered L1 measurement reporting in mobility.

#### Issue 2-7: Impact on other LTM requirements

Agreement:

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

#### Issue 2-8: which result to report if UE has chance to update measurement after report is triggered and before report is actually sent

Candidate options:

- Option 1: UE ignores new L1 result after L1 report is triggered

- Option 2: UE consider new L1 result after L1 report is triggered

- Option 3: up to UE implementation

Agreement:

- Up to UE implementation.

  - This can be revisited pending progress in other working groups.

#### Issue 2-9: CR split

Event triggered L1 reporting

| Requirements | Title | Leading company |
| --- | --- | --- |
| (FFS) 9.1.4 | Capabilities for Support of Event Triggering and Reporting Criteria |  |
| 9.14.3.4 | Event-triggered Periodic Reporting | Ericsson |
| 9.14.3.5 | Event-triggered Reporting |  |
| 9.15.3.4 | Event-triggered Periodic Reporting | Nokia |
| 9.15.3.5 | Event-triggered Reporting |  |
| 9.14a.3.x1 | Event-triggered Periodic Reporting Note: for CSI-RS based L1 measurement | CMCC |
| 9.14a.3.x2 | Event-triggered Reporting Note: for CSI-RS based L1 measurement |  |

# Topic #2: CSI-RS based L1 measurement

#### Issue 4-1: Known cell definition in CSI-RS based L1 measurement

Issue 4-1-1: Whether to use detectable or measurable?

Agreement:

- Confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable”

Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell

Agreement:

- Use legacy side condition of SINR=-3dB.

#### Issue 4-2: Measurement procedure

#### Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

Agreement:

- Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

- Further discuss whether step 2 can be skipped for serving cell or not.

#### Issue 4-2-2: How to interpretate the UE capability (for FR2-1)

Agreement:

- For a UE not supporting the capability, Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

- For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

  - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

  - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

  - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

#### Issue 4-3: Definition of “inside/outside active BWP”

Agreement:

- RRM requirements (delay and accuracy) apply under the following condition:

  - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP.

#### Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition

Candidate options:

- Option 1: Apple, CMCC

A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured).

- Option 2:  MediaTek, ZTE, Ericsson, Nokia, Qualcomm

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- Option 3: Huawei

If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference.

If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred.

- Option 4: vivo

  - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

  - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

  - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

#### Issue 4-5: Measurement on CSI-RS with repetition ON/OFF

Agreement for FR2:

- Only consider CSI-RS with repetition OFF in RAN4 requirements.

#### Issue 4-6: measurement GAP and RTD assumption

Agreement:

- From RAN4 perspective,

  - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell.

  - Not to define RAN4 requirements for RTD > CP.

#### Issue 4-7: CSI-RS resource type

Agreement:

- Define RRM requirement for CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS.

#### Issue 4-8: Restriction on CSI-RS resource configuration

Candidate options:

- Option 1: Ericsson

  - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

#### Issue 4-9: requirement applicability

Candidate options:

  - Option 1: Qualcomm

RAN4 requirements apply only when all CSI-RS resources from candidate cells on the same layer are configured with the same SCS and CP type.

  - Option 2: MediaTek

For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, legacy measurement period and scheduling restriction requirements are both applicable.

  - Option 2: vivo

RRM requirements for CSI-RS based LTM are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

#### Issue 4-10: Definition of frequency layer

Agreements:

- No need to introduce definition of frequency layer.

#### Issue 4-11: Whether to consider CSI-RS based L1-SINR measurement

Agreement:

- RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

#### Issue 4-12: Early CSI acquisition on a candidate cell

Candidate options:

- Option 1: Qualcomm

  - For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements.

  - For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded:

    - Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc.

- Option 2: Nokia

  - Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

#### Issue 4-13: CSI-RS accuracy requirements

Candidate options:

- Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (ZTE, Apple, Nokia)

#### Issue 4-14: others

Candidate options:

- Option 1: Nokia

  - After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

  - If the number of resources/cells, including the number of resources/cells configured for CSI-RS based L1-RSRP measurement exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

  - Discuss how many measurements CSI-RS measurements UE is capable of performing during each measurement period.

#### Issue 4-14: CR split

CSI-RS based L1 measurement

| Requirements | Title | Leading company |
| --- | --- | --- |
| 9.14a | CSI-RS based L1-RSRP measurements for neighbour cell | MTK |
| 9.14a.1 | Introduction |  |
| 9.14a.2 | Requirements applicability | Nokia |
| 9.14a.3 | Measurement reporting requirement Note: not include event triggered | Apple |
| 9.14a.4 | Number of CSI-RS, number of cells and number of CSI-RS resources | CATT |
| 9.14a.5 | CSI-RS based L1-RSRP measurement requirements | Apple |
| 9.14a.6 | Measurement restriction for CSI-RS based L1-RSRP measurement | ZTE |
| 9.14a.7 | Scheduling availability of UE during CSI-RS based L1-RSRP measurement | Huawei |