3GPP TSG-RAN WG4 Meeting #114bis													R4-2504924

# Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.12.7

Source:	Moderator (CMCC)

Title:	 Coffee break discussion minutes on NR_ATG_enh

Document for:	Information

Issue 2-1-1: Measurement accuracy

- Proposals

  - Proposal 1: (CMCC, CATT, ZTE)

    - For ATG L1/L3 measurement of SCell/SCC, reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network.

- Recommended WF

  - Check whether Proposal 1 can be agreed.

Ericsson: Support

Agreement: 

For ATG L1/L3 measurement of SCell/SCC, reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network.

Issue 2-2-1: Test configuration

- Proposals

  - Proposal 1: (CMCC, ZTE, Ericsson)

    - Following three test configurations are supported for both PCell and SCell:

      - Configuration 1:15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode

      - Configuration 2: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode

      - Configuration 3: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode

        - If legacy TN test cases don’t have this test configuration, then no need to introduce it into the corresponding ATG test cases.

  - Proposal 2: (CATT)

    - RAN4 need to discuss whether to add new test configuration to include more UE types defined in R19 ATG core part

- Recommended WF

  - Check whether Proposal 1 can be agreed.

  - Further discuss Proposal 2.

CATT： No need to add more configuration about UE types. But for test setups, need check regarding UE types.

ZTE: What type of test setups CATT mentioned? Antenna

CATT: Handover, etc have antenna assumption. We need more time to check the new test cases.

Ericsson: Keep the antenna configuration part open.

Agreement: 

- Following three test configurations are supported for both PCell and SCell:

  - Configuration 1:15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode

  - Configuration 2: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode

  - Configuration 3: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode

    - If legacy TN test cases don’t have this test configuration, then no need to introduce it into the corresponding ATG test cases.

Issue 2-2-2: Test applicability rule

- Proposals

  - Proposal 1: (CMCC, Ericsson)

    - UE is only required to be tested in one of the supported test configurations

    - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

- Recommended WF

  - Check whether Proposal 1 can be agreed.

HW, ZTE: further discuss

Issue 2-2-3: Test setups

- Proposals

  - Proposal 1: (CMCC, CATT, ZTE)

    - Following test setups agreed in R18 can be reused:

      - TDD pattern: The legacy TDD pattern are used

      - Channel model: AWGN with residual doppler channel model

  - Proposal 2: (CMCC, ZTE)

    - Following test setups agreed in R18 can be reused:

      - Number of neighbour cells: one neighbour cell in the test if needed

  - Proposal 3: (CMCC, CATT)

    - Following test setups agreed in R18 can be reused:

      - GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

      - BS location: The specific gNB reference location is emulated by test system

  - Proposal 4: (CMCC, Ericsson, CATT)

    - Following test setups agreed in R18 can be reused:

      - RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases.

      - OCNG:

        - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133.

        - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133.

        - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

- Recommended WF

  - Check whether Proposal 1, 2, 3 and 4 can be agreed.

- LGE: what about UL of SCell

- ZTE: all symbol/slots is DL

- Intel: why we need BS location?

- ZTE: From SIB22

- Agreement:

- Following test setups agreed in R18 can be reused:

- TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell

- Channel model: AWGN with residual doppler channel model

- Number of neighbour cells: one neighbour cell in the test if needed

- GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

- BS location: The specific gNB reference location is emulated by test system

- RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases.

- OCNG:

  - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133.

  - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133.

  - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

Issue 2-2-4: Test method and scaling factor for UE with antenna array

- Proposals

  - Proposal 1: (CATT)

    - RAN4 need to discuss whether to reuse the legacy test method and scaling factor for all types of UE.

  - Proposal 2: (ZTE)

    - Reuse the test method defined in R18 ATG for all antenna array cases including intra-band CA and inter-band CA.

- Recommended WF

  - To be discussed.

ZTE: R18 have already ignore the scaling factor due to no OTA test will be applied in RRM test. The methodology of test method can be used.

Ericsson: No agreement here.

Tentative agreement:

The scaling factor related with antenna array will not be applied in test requirements

FFS of the test parameters, test requirements for UE with antenna array on both PCC and SCC

Issue 2-2-5: Test scope

- Proposals

  - Proposal 1: (CMCC)

    - Define following test cases for ATG, related legacy TCs can be referred:

      - Interruptions during measurements on deactivated NR SCC in FR1

      - Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

      - Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

      - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle

      - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

      - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

      - Direct SCell activation at SCell addition of known SCell in FR1

      - Direct SCell activation at handover with known SCell in FR1

      - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle

      - Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

      - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle

      - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

      - Inter-band SSB-less SCell Activation based on A-TRS

      - Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode

      - Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

      - Simultaneous RRC-based Active BWP Switch on multiple CCs

      - SCell dormancy switch

      - Event triggered reporting on SCC with deactivated SCell test under non-DRX

  - Proposal 2: Define test case for deactivated SCell measurement requirement (CMCC)

  - Proposal 3: Open to discuss whether define test cases for the following core requirement (CMCC)

    - Direct SCell Activation at RRC Resume

    - non-simultaneous Timer based BWP switch delay on multiple CCs

    - Pre-configured measurement gap requirement

  - Proposal 4: (CATT)

    - TC for SCell Activation and Deactivation Delay for ATG

    - FFS: TC for Interruption for ATG

    - TC for Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

    - TC for Simultaneous RRC-based Active BWP Switch on multiple CCs

  - Proposal 5: (HW)

    - Performance requirement of interruption at NR SRS antenna port switching needs to be defined.

    - FFS The performance requirement of interruption at NR SRS antenna port switching based on core requirement.

    - Further discuss whether or not need to define the performance requirement of SCell Activation of SSB-less SCell in FR1 inter-band CA for ATG

    - The SCell activation and deactivation delay need the performance requirements. The following performance requirements table is a reference for RRM to discuss

      - SCell Activation and deactivation performance requirements

      - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle

      - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

      - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

      - Direct SCell activation at SCell addition of known SCell in FR1

      - Direct SCell activation at handover with known SCell in FR1

      - PUCCH SCell Activation and deactivation of known SCell in FR1

      - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

      - SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command

      - SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX

      - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle

      - SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

      - SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX

      - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle

      - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

  - Proposal 6: (ZTE)

    - Focus on the new functionalities introduced for R19 ATG that were not defined in R18 single carrier ATG, i.e. mainly the DL CA relevant features and antenna array relevant features.

      - TC1: Intra-frequency measurement(with scheduling restriction verified as well)

      - TC2: Inter-frequency measurement(with scheduling restriction verified as well)

      - TC3: L1-RSRP measurement

      - TC4: The interruption of SCell addition/release

      - TC5: The interruption of SCell activation/deactivation

    - RAN4 discusses whether all these test cases need to verify two categories of UE antenna assumption:

      - Category 1: common Rx beam in PCC and SCC band(s), including intra-band antenna array and Case 4 of inter-band

      - Category 2: separate Rx beam in PCC and SCC bands, including other cases

  - Proposal 7: (Ericsson)

    - New test cases developed for NR ATG CA will only be relevant to DL Co-existence FR1 CA.

    - discuss and agree on the test cases based on Table 1

| No. | Item of Core Requirements | Type of Test Cases | Subclause | Note |
| --- | --- | --- | --- | --- |
| 1-1 | Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | A.19.4.X | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.1 |
| 1-2 |  | Interruptions at NR SRS antenna port switching |  | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.4 |
| 2-1 | SCell Activation Delay | SCell activation and deactivation of Scell for ATG | A.19.4.X | Modify Test case for SCell known, unknown based on A.6.5.3.1 / A.6.5.3.2 / A.6.5.3.3 |
| 3-1 | Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | A.19.4.2.X | Modify test case based on A.6.5.5 |
| 4-1 | Active BWP Switch Delay | NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX | A.19.4.3.X | New test case required based on A.6.5.6.1.1 |
| 5-1 | L3 Measurement | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | A.19.X.X | New test case required based on A.6.6.10 and A.6.6.11 |

- Recommended WF

  - Try to summary based on the input from companies

| Test category | Test cases | Company comments |
| --- | --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | CMCC: Support Ericsson: Support |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | interruption at NR SRS antenna port switching | HW: FFS |
|  | Interruptions at NR SRS carrier based switching | CATT: FFS |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | CMCC: Support CATT: Support HW: Support Ericsson: Support |
|  | Direct SCell activation at SCell addition of known SCell in FR1 | CMCC: Support CATT: Support HW: Support |
|  | Direct SCell activation at handover with known SCell in FR1 | CMCC: Support CATT: Support HW: Support |
|  | Direct SCell Activation at RRC Resume | CMCC: FFS CATT: Support |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | CMCC: Support CATT: Support HW: Support |
|  | Inter-band SSB-less SCell Activation based on A-TRS | CMCC: Support CATT: Support HW: FFS |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | Ericsson: Support |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support Ericsson: Support |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support |
|  | non-simultaneous Timer based BWP switch delay on multiple CCs | CMCC: FFS |
| SCell dormancy switch | SCell dormancy switch | CMCC: Support |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | CMCC: Support |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement | CMCC: FFS |
| Others | Intra-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | Inter-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | L1-RSRP measurement | ZTE: Support |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | Ericsson: Support |
|  | The interruption of SCell addition/release | ZTE: Support |
|  | The interruption of SCell activation/deactivation | ZTE: Support |

  - To be discussed based on the table above.

Issue 1-2-4: SCell Addition/Activation mechanism

- Proposals

  - Proposal 1: Distance triggered SCell addition can be considered (CATT)

  - Proposal 2: define distance triggered SCell addition and/or activation requirements with some distance related conditions only in RAN4 spec, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted. (CATT)

  - Proposal 3: The distance triggered SCell addition and/or activation is applicable to UE antenna Case 4 Exclusively (HW)

  - Proposal 4: RAN4 should not support any UE initiated DL SCell activation mechanism (Ericsson)

- Recommended WF

  - To be discussed

ZTE: We have concern on SCell activation and deactivation. The procedure shouldn’t out of network control. Prefer to follow legacy.

CATT: This only for Case#4, we can deprioritize this one and focus on the measurement enhancement.

CMCC: also ok to deprioritize

QC: don’t support location-based mechanism

Issue 1-2-5: SCell  Deactivation mechanism

- Proposals

  - Proposal 1: introduce a distance-based DL SCell deactivation mechanism (Ericsson)

- Recommended WF

  - To be discussed

CMCC: similar as last issue.

Ericsson: UE out of SCell coverage, but the timer still active, may cause performance issue.

QC: don’t know how it will work. Network could control the deactivation by measurement report.

HW: deprioritize this one

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement

- Background

  - Agreement in RAN4#114:

Agree on the following Proposal 1 for the intra-frequency measurement and L1-RSRP measurement, and apply the similar principle to other related requirements.

- Proposal 1:

  - The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example):

For UE with the antenna array,

    - Klayer1_measurement=1,

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

    - Klayer1_measurement=1.5, otherwise.

- Proposals

  - Proposal 1: (LGE)

    - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

  - Proposal 2: (ZTE)

    - No need to distinguish whether the L1 measurement is targeted at PCell or SCell since UE applies a same Rx beam to cover both of them.

    - The L1/L3 measurement sharing factor could be summarized as two cases:

      - For the intra-band CA with antenna array assumed and inter-band CA with Case 4 assumed,

        - Klayer1_measurement = 1 for the not fully overlapping case and fully overlapping with SMTC occasion but not overlapped with RRM RS symbols and 1 symbol before and after;

        - Klayer1_measurement = 1.5 for otherwise.

      - For other cases,

        - Klayer1_measurement = 1 since UE could perform RRM and L1 measurement simultaneously.

- Recommended WF

  - To be discussed

Issue 1-1-1: Side condition for Case#4 UE

- Proposals

  - Proposal 1: (Ericsson)

    - If, the frequency ranges of inter-band co-located DL CA bands are less than or equivalent to X MHz, then the ATG UE may support CBM with an antenna array on the PCC and SCC in FR1.

    - Otherwise, ATG UE with an antenna array on the bands of PCC and SCC will adhere to IBM in FR1.

    - DL frequency separation (i.e., X MHz) between the two bands for FR1 CBM should be

      - Option 1: ≤50 MHz.

      - Option 2: ≤100 MHz.

      - Option 3: Between 50-100MHz, FFS.

- Recommended WF

  - This should be discussed in RF session.

Issue 1-1-2-6: Others

- Proposals

  - Proposal 1: This new capability only affects the following RRM requirements: (HW)

    - 1.	SCell deactivated measurement with L1/L3 measurement factor.

    - 2.	Scheduling Availability/Restrictions.

    - 3.	SCell measurement enhancement.

    - 4.	FFS

  - Proposal 2: RAN4 should send an LS to RAN2

- Recommended WF

  - Proposal 1 will be discussed separately in each requirements and directly reflected in the spec. No further discussion on Proposal 1

  - If all contents of UE capability are agreed in this meeting, then discuss whether to send a LS to RAN2 in this meeting, or next meeting together with other RAN2 impact if exists.

Issue 1-1-4: UE antenna type signalling

- Proposals

  - Proposal 1: UE capability of [antennaArrayType-r19] to distinguish whether UE capable of antenna array in intra/inter-band CA ( ZTE)

- Recommended WF

  - Based on RF discussion, no separate UE capability will be introduced for UE antenna type in CA case, instead, UE will report its antenna type per band, and network can know the UE antenna type per BC.

  - For example, UE report antenna array for n3 and omni-directional antenna for n39, then for n3 and n39 inter-band CA, UE antenna type will be antenna array n3 + omni-directional antenna n39.

Issue 1-3-4: UE capability about simultaneous Reception of Data and SSB with Different Numerology

- Proposals

  - Proposal 1: (Ericsson)

      - Discuss and support new UE capability to simultaneously perform the following tasks without requiring measurement gaps:

        - Receive scheduled DL data/control signals (PDSCH/PDCCH) on a serving cell.

        - Concurrently perform measurements on SSBs transmitted on inter-frequency or intra-frequency cells that have a different numerology (subcarrier spacing) from the scheduled serving cell.

- Recommended WF

  - Based on moderator’s  knowledge, there has already existed one UE capability ‘simultaneousRxDataSSB-DiffNumerology’ as described above.

  - To be discussed