3GPP TSG-RAN WG4 Meeting #114-bis	     R4-2505094

Wuhan, China, 7th – 11th April 2025

Title: 	WF on n48 50MHz CBW

Agenda Item:	6.12

Source: 	CableLabs, Apple, Qualcomm, Skyworks

Document for:	Approval

# Introduction of new channel bandwidth

Adding new channel bandwidth in band n48 was approved in RAN #104 [1].

| Band | Channel bandwidth and SCS | Contact name, company | Contact email | Other supporting companies | Justification | Additional information | status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n48 | 50 MHz (15, 30 and 60kHz SCS) 60, 70, 80, 90, 100 MHz (30 and 60 kHz SCS) | Ruoyu Sun, Cable Labs | R.Sun@cablelabs.com | Charter, Comcast, Samsung | This channel BWs are already supported in DL, they should be supported in UL as well, see RP-241541. |  | new |

A-MPR simulation results have been provided with the contributions [2], [3], [4], [5], [6], and [8]. Two proposals, [7] and [8] were submitted for RB regions and values for A-MPR were provided, as listed in Table 1. This WF captures RB regions for channel sizes of 50MHz to 100MHz.

Way forward: Use Table 1 as starting point for further discussion and alignment on RB regions.

- Companies are requested to provide feedback on the RB regions and propose changes if needed and to provide A-MPR proposals for the individual regions.

Table 1: RB regions for channel bandwidth

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 50 MHz | 3575 ≤ FC < 3605 | <12.6 MHz |  |  | A7 |
|  |  | ≥12.6 MHz ≤31.0 MHz |  | ≥22.32 MHz | A2 |
|  |  |  |  | <22.32 MHz | A1 |
|  |  | ≥38.52 MHz |  | ≤3.6 MHz | A7 |
|  | 3645 < FC ≤ 3675 |  | Option 1: >36.0 MHz (Apple) Option 2: >43.2MHz (Qualcomm) | ≤3.6 MHz | A1 |
|  |  |  | ≤36.0 MHz | ≥22.32 MHz | ? |
|  |  |  | ≥17.6 MHz | <22.32 MHz | ? |
|  |  |  | ≤10.08 MHz | ≤3.6 MHz | ? |
|  | 3605 ≤ FC ≤ 3645 | ≤6.12 MHz |  | ≤3.6 MHz | A8 |
|  |  |  | ≥42.48 MHz | ≤3.6 MHz | A8 |
| 60 MHz | 3580 ≤ FC < 3620 | <15.84 MHz |  |  | A7 |
|  |  | ≥15.84 MHz ≤31.0 MHz |  | ≥27.36 MHz | A2 |
|  |  |  |  | <27.36 MHz | A1 |
|  |  | ≥44.64 MHz |  | ≤3.6 MHz | A7 |
|  | 3630 < FC ≤ 3670 |  | >42.48MHz |  | A7 |
|  |  |  | ≤42.48 MHz | ≥27.36 MHz | A2 |
|  |  |  | ≥27.32 MHz | <27.36 MHz | A1 |
|  |  |  | ≤13.68 MHz | ≤3.6 MHz | A7 |
|  | 3620 ≤ FC ≤ 3630 | ≤6.12 MHz |  | ≤3.6 MHz | A8 |
|  |  |  | ≥52.2 MHz | ≤3.6 MHz | A8 |
| 70 MHz | 3585 ≤ FC ≤ 3665 | ≤22.32 MHz |  | <25.56 MHz | A7 |
|  |  |  | Option 1: ≥ 55.8MHz (Apple) Option 2: ≥ 50.4 MHz (Qualcomm) | <25.56 MHz | A7 |
|  |  |  |  | ≥25.56 MHz | A7 |
| 80 MHz | 3590 ≤ FC ≤ 3660 | ≤22.32 MHz |  | <25.56 MHz | A7 |
|  |  |  | ≥55.8 MHz | <25.56 MHz | A7 |
|  |  |  |  | ≥25.56 MHz | A7 |
| 90 MHz | 3595 ≤ FC ≤ 3655 | ≤25.92 MHz |  | <28.8 MHz | A7 |
|  |  |  | ≥62.28 MHz | <28.8 MHz | A7 |
|  |  |  |  | ≥28.8 MHz | A7 |
| 100 MHz | 3600 ≤ FC ≤ 3650 | ≤29.16 MHz |  | <32.4 MHz | A7 |
|  |  |  | ≥69.12 MHz | <32.4 MHz | A7 |
|  |  |  |  | ≥32.4 MHz | A7 |

# References

- RP-241684, “Basket WID on adding channel bandwidth support to existing NR bands”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #104

- R4-2415525, “Simulation results for NS_27”, Apple, 3GPP RAN WG4 Meeting #112bis

- R4-2415531, “n48 wider BWs and NS_27 values”, Qualcomm, 3GPP TSG-RAN4 Meeting #112bis

- R4-2416214, “n48 NS_27 A-MPR simulations beyond 40 MHz CBW”, Nokia, 3GPP TSG-RAN4 Meeting #112bis

- R4-2417746, “n48: Adding channels of 50MHz and beyond”, Apple, 3GPP RAN WG4 Meeting #113

- R4-2500306, “n48: Adding channels of 50MHz and beyond”, Apple, 3GPP RAN WG4 Meeting #114

- R4-2503193, “n48: Adding channels of 50MHz and beyond”, Apple, 3GPP RAN WG4 Meeting #114-bis

- R4-2503563, “On adding n48 NS_27 A-MPR for CBW 50MHz and higher”, Qualcomm Incorporated, 3GPP RAN WG4 Meeting #114-bis