# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2505098

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	WF on requirements for 7MHz channel bandwidth

Agenda item:	7.8.4

Source:	Ericsson

Document for:	Approval

# General

## 1-2-1: Initial BWP

Agreement:

- There is no restriction on the initial BWP size.

# UE RF

## 2-1-1, 2-1-3, 2-1-5, 2-1-6: NS_12, NS_13, NS_14 and NS_15 – A-MPR and frequency carrier Fc

Agreement:

- Evaluate A-MPR for:

  - NS_12 considering 7 MHz deployed within 814-824 MHz

  - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz).

  - NS_14 considering 7 MHz deployed within 824-849 MHz

  - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

## 2-2-1: REFSENS for n5

Agreement:

- 7 MHz channel BW REFSENS for n5: -96.5 dBm value (25 RBs uplink configuration).

2-2-2: REFSENS for n26

Agreement:

- 7 MHz channel BW REFSENS for n26: -96.0 dBm value (25 RBs uplink configuration).

2-	Remaining open issue:

A-MPR requirement

# BS RF

## 3-1-1: OTA requirements

Agreement:

- Specify OTA requirements for 7 MHz channel BW.

## 3-1-2: FRCs

Agreement:

- Reuse existing FRC(s) for 7MHz BS testing.

3-2-1: Output power dynamics

Agreement:

- Output power dynamics

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 7 | 15.4 | N/A | N/A |

3-2-2: Output power dynamics – NB-IoT in-band

Agreement:

- Output power dynamics – NB-IoT in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 7, 10 | Any | +6 |

3-2-3: ACLR

Agreement:

- For 7 MHz channel BW, the ACLR limit in non-contiguous spectrum or multiple bands and the CACLR limit shall be the same ones as for 5 MHz channel BW.

3-3-1: REFSENS

Agreement:

- Specify REFSENS for 7 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

3-3-2: Dynamic range

Agreement:

- Specify Dynamic Range for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 7 | 15 | G-FR1-A2-1 (Note 1) | -70.7 | -65.7 | -62.7 | -81.0 | -76.0 | -73.0 |

3-3-3: Dynamic range for NB-IoT operation in NR in-band

Agreement:

- Specify Dynamic Range for NB-IoT operation in NR in-band and for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 7 | FRC A15-1 in Annex A.15 in TS 36.104 [13] | -99.7 | -81 | AWGN |
| 7 | FRC A15-2 in Annex A.15 in TS 36.104 [13] | -105.6 | -81 | AWGN |

3-3-4: ACS

Agreement:

- Specify ACS for 7MHz channel BW according to the following tables:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3 | PREFSENS + 8 dB | Wide Area BS: -52 Medium Range BS: -47 Local Area BS: -44 |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

3-3-5: In-band general blocking

Agreement:

- Specify In-band general blocking for 7MHz channel BW according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

3-3-6: Narrowband blocking - limit

Agreement:

- Specify narrowband blocking limit for 7MHz channel BW according to the following tables:

Table 7.4.2.2-2: Base Station narrowband blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: -49 Medium Range BS: -44 Local Area BS: -41 |

Table 7.4.2.2-2a: Base Station narrowband blocking requirement for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50 | PREFSENS + x dB (Note 2) | Wide Area: -49 Medium Range: -44 Local Area: -41 |

3-3-7: Narrowband blocking – interferer

Agreement:

Specify narrowband blocking interferer for 7MHz channel BW according to the following table:

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |

3-3-8: Intermodulation interferer

Agreement:

- Specify intermodulation interferer for 7MHz channel BW according to the following table:

Table 7.7.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

3-3-9: Narrowband intermodulation interferer

Agreement:

- Specify narrowband intermodulation interferer for 7MHz channel BW according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 7 | ±355 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

3-3-12: EVM

Agreement:

- Specify EVM window length for 7MHz channel BW according to the following table:

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 7, 10 | 1024 | 72 | 28 | 40 |

3-	Remaining open issue:

ICS requirement