3GPP TSG-RAN4 Meeting # 114bis	R4-2505130

Wuhan, China, Apr.7th – 11th, 2025

# Source:	ZTE Corporation,China Unicom, Huawei, Hisilicon, China Southern Power Grid, Ericsson

# Title:	TP for TR38.749 UE RF requirements (Clause 8)

# Agenda Item:	6.17.2

# Document for:	Approval

- Introduction

In this contribution, a TP to TR38.749[1] is provided for UE RF requirements(Clause 8).

This TP is revised from R4-2503974.

- Reference

[1] TR 38.749 v0.1.0 Protection of Earth Exploration Satellite Service (EESS) Huawei, HiSilicon.

- Text Proposal

----- Start of TP -----

- 8	UE RF requirements

- 8.1		Background

The Earth Exploration Satellite Service (EESS) protection requirements in the frequency range 23.6-24.0 GHz (as per WRC-19 Radio Regulations Resolution 750 [2] and EU Decision 2020/590 [4]) were specified using NS_202/CA_NS_202 and NS_203/CA_NS_203 signaling.

NS_202 should be signaled for operation in band 257 or n258 in CEPT countries, while NS_203 would be signaled for operation in those bands in other countries.

Nevertheless, only the +1dBm/200MHz limit was captured, which is applicable until 1 September 2027 (ITU-R) and until 1 January 2024 (CEPT countries).

In terms of the EU Decision 2020/590 [4], the additional UE spurious emission requirement for EESS passive services protection in the 23.6GHz-24GHz for band n257 and n258 are defined in table 8.1.

Table 8.1: Additional emission requirements for only EESS requirement in Rel-19

| Frequency band (GHz) | Spurious emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | -5dBm | - MHz |

The above limit of -5dBm/200MHz was not considered, not anticipating 1 September 2027 when this limit should apply. Also, this is causing a major issue for CEPT countries where this limit is applicable for any device brought to use from 1 January 2024.

- 8.2		UE RF impacts

Within this WI, to address the Regulation issues mentioned above, the modifications in legacy specification TS38.101-2 are:

- The legacy NS_202 and CA_NS_202 are updated by replacing the limit of +1 dBm/200 MHz with -5dBm/200MHz, and the corresponding A-MPR values were adjusted based on this new more stringent limit.

- The newly introduced NS_205/CA_NS_205 with -5dBm/200MHz limit for band n257 and n258, and the corresponding A-MPR values were also specified.

The above NS values specified in Rel-19 for UE EESS are mandatory. For bands n257 and n258, a corresponding modifiedMPRbehavior bit indicated in the IE RF-Parameters are specified:

- A modifiedMPRbehaviour bit to differentiate between legacy NS_202/CA_NS_202 and new NS_202/CA_NS_202 requirements. It is mandatory to report modifiedMPRbehaviour bit from Rel-15 for new UEs.

- A modifiedMPRbehaviour bit to indicate the UE supports NS_205 and CA_NS_205. It is mandatory to report modifiedMPRbehavior bit from Rel-15 for new UEs..

The detailed descriptions of modifiedMPRbehavior indication in Rel-19 TS38.101-2 are captured in Table 8-2.

Table 8-2. modifiedMPRbehavior indication for NS_202/CA_NS_202 and NS_205/CA_NS_205 in Rel-19

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | ... | ... |
|  | 1 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
|  | 2 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
| n258 | 0 (leftmost bit) | ... | ... |
|  | 1 | ... | ... |
|  | 2 | .... | ... |
|  | 3 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

As those updates have been considered as critical to comply with the Regulations, they have been reported back to Rel-15. i.e.  TS38.101-2 v15.26.0.

In CEPT countries where the -5dBm/200MHz limit already applies, only NS_202 and CA_NS_202 shall be broadcasted by the network.

While in other ITU-R countries, from 1 September 2027, the network will have to broadcast NS_205 (and CA_NS_205) followed by NS_203 (and CA_NS_203) to allow any UE brought to use after or before 1 September 2027 to connect to the network.

The A-MPR requirements for NS_202/CA_NS_202 to comply with the -5dBm/200MHz limit have been updated according to Table 8.3.

Table 8.3. Updated A-MPR for NS_202 and CA_NS_202 with the modifiedMPR-Behavior indication

| NS values | Single band/intra-band CA | Updated A-MPR for PC1 (dB) | Updated A-MPR for PC2~PC7 (dB) |
| --- | --- | --- | --- |
| NS_202 | Single band | 11.0 | 2.0 |
| CA_NS_202 | Contiguous UL CA with contiguous allocation | 11.0 | [5.0] if offset frequency < BWintraCA, 2.0 otherwise. |
|  | Contiguous UL CA with non-contiguous allocation or NC UL CA | [13.0] if offset frequency < BWintraCA, 11.0 otherwise | [8.0] if offset frequency < BWintraCA, 2.0 otherwise. |

The A-MPR requirements for NS_205/CA_NS_205 to comply with the -5dBm/200MHz limit have been updated according to in Table 8.4.

Table 8.4. A-MPR requirements for NS_205 and CA_NS_205

| NS values | Single band/intra-band CA | A-MPR for PC1 (dB) | A-MPR for PC2~PC7 (dB) |
| --- | --- | --- | --- |
| NS_205 | Single band | 7.0 if Offset frequency < BWchannel, 6.0 otherwise | 2.0 if offset frequency < BWchannel, 0.0 otherwise |
| CA_NS_205 | Contiguous UL CA with single contiguous RB allocation | [9.0] if offset frequency < BWintraCA, 6.0 otherwise. | [5.0] if offset frequency < BWintraCA, 0.0 otherwise. |
|  | Contiguous UL CA with non-contiguous allocation or NC UL CA | [13.0] if offset frequency < BWintraCA, 6.0 otherwise. | [8.0] if offset frequency < BWintraCA, 0.0 otherwise. |

NOTE 1: For NS_202 and NS_205, the Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the channel bandwidth.

NOTE 2: For CA_NS_202 and CA_NS_205, the Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the lowest CC among the configured UL CA.

----- End of TP -----