3GPP TSG RAN WG4 Meeting #114bis	R4-2505156

Wuhan China, 7th – 11th April, 2025

Agenda item:			7.20.3

Source:	Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Title:	Simulation assumptions on AI/ML CSI compression

Document for:	Approval

- Introduction

RAN4#114bis Ad hoc minutes on AI/ML air interface study [1] Issue 2-1-3: Preliminary observations for option 3 track 2 … - Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered - A couple of initial results suggest similar SGCS can be obtained with lower complexity CNN, but SGCS would drop with a lower complexity transformer encoder - Further results are encouraged for the next meeting bring further simulation results with different encoder structures

Issue 2-1-4: Preliminary conclusions for option 4 … - For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used - For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b) … For option 4, consideration should also be given to the compatibility of “own” encoder that has a different structure to the agreed structure (lower complexity).

According to Moderator’s summary and ad-hoc tentative observations, companies are encouraged to bring further simulation results with lower-complexity encoder structures. Since the agreed encoder is impractical due to its extremely high complexity, considering low complexity encoders can make the feasibility verification more substantial. The simulation results will also provide guidance for the design of the encoder/decoder in the WI stage (if AI-CSI turns to WI).

- Verification procedure for lower-complexity encoder training

With the assumption of a common encoder structure both for the frozen decoder training and the encoder retraining, the observations have been concluded as below.

- When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far o Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details o Preliminary observation; further results may be added

With a common encoder structure, the feasibility of achieving a common decoder has been verified. The intention here is to check the feasibility of achieving a common decoder that is compatible to encoders with different complexities.

## 2.1 Verification procedure in Option 3

- Step 0: Train agreed encoder with the agreed decoder from the beginning (i.e., joint-training, simulation results have been submitted and summarized in RAN4#114bis [2]) to obtain SGCS-0

- Step 1: Train agreed encoder with the frozen decoder and test (i.e., separate-training, simulation results have been submitted and summarized in RAN4#114bis [2]) to obtain SGCS-1

- Step 2: Train a low-complexity encoder with the frozen decoder and test (i.e., separate-training) to obtain SGCS-2

- Step 3: Train a low-complexity encoder with the agreed decoder from the beginning (i.e., joint-training) and test to obtain SGCS-3

By comparing SGCS-1 and SGCS-2,

- if there is no performance loss, we can conclude the feasibility of achieving a common decoder that is compatible to encoders with different complexities.

- if performance loss is observed, but SGCS-3 is similar to SGCS-0, we can conclude that low-complexity encoder should be considered at the beginning of deriving the test decoder.

- if performance loss is observed, and SGCS-3 is much lower than SGCS-0, it may because the complexity of the encoder is too low. The encoder within a reasonable range of complexity should be considered.

Note: Observations observed from Option 3 also apply to Option 4.

- Model structure assumptions for encoder with lower complexity (for information)

Although the model structures are just for feasibility study, it would be helpful for simulation results alignment to use common model structures for the lower-complexity encoder. In practice, UE vendors may use CNN, Transformer or MLP as the backbone. According to the simulation results from RAN4 #114bis, SGCS may drop with a transformer encoder retraining even if the complexity level is comparable to the CNN encoder. Companies are encouraged to provide simulation results not only for low-complexity encoder with CNN, but also with low-complexity transformer and MLP.

## 3.1 Transformer based lower-complexity encoder

Table 1. Description of Transformer-based low-complexity encoder

| Model type | Transformer |
| --- | --- |
| FLOPs | 3.63M |
| Model size | 1.6Mbyte |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 |
| N_TF | 4 |

## 3.2 CNN based lower-complexity encoder

Table 1. Description of CNN-based low-complexity encoder

| Model type | CNN (Resnet like design) |
| --- | --- |
| FLOPs | 4.14M |
| Model size | 92Kbyte |
| Number of Resnet blocks | 2 |
| Layer after Resnet blocks | 2 layers (one FC layer, one Conv layer) |
| Layer before Resnet blocks | 1 layer (one Conv layer) |
| Layer within a resnet block | 2 Conv layer and direct combination |
| CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
| Kernel/Channel size within a resnet block | (3,3,16) |
| Size of message (feature map) passed across layers | (13,32) |

- Format of simulation results to submit in next meeting

| Option 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Joint training |  | New encoder training with (Samsung’s) frozen decoder |  |  |  |
|  |  | Testing on mixed |  | Testing on mixed |  |  | Testing on own |
|  |  | training on mixed | (2nd priority) training on own | training on mixed | Additional results 1 training on own | (2nd priority) training on mixed & own [Note: Not captured in WF] | Additional results 2 training on own |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Company’s name |  |  |  |  |  |  |
| Encoder-2 (Transformer with 3.63M FLOPs) | Company’s name |  |  |  |  |  |  |
| Encoder-3 (CNN with 4.14M FLOPs) | Company’s name |  |  |  |  |  |  |

| Option 4 Provide results for - ‘own encoder’ with ‘own decoder’ - ‘own encoder’ with other companies’ decoder |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st priority Dataset1=Dataset2=Dataset3=mixed |  |  |  |  |  |  |  |  |  |  |
|  |  | Source of 'own' decoder (retrained with the frozen decoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Company’s name |  |  |  |  |  |  |  |  |  |  |  |
| Encoder-2 (Transformer with 3.63M FLOPs) | Company’s name |  |  |  |  |  |  |  |  |  |  |  |
| Encoder-3 (CNN with 4.14M FLOPs) | Company’s name |  |  |  |  |  |  |  |  |  |  |  |

| Option 4 Provide results for - ‘own encoder’ with ‘own decoder’ - ‘own encoder’ with other companies’ decoder |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2nd priority Dataset1=Dataset2=mixed, Dataset=own |  |  |  |  |  |  |  |  |  |  |
|  |  | Source of 'own decoder’ (retrained with the frozen encoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Company’s name |  |  |  |  |  |  |  |  |  |  |  |
| Encoder-2 (Transformer with 3.63M FLOPs) | Company’s name |  |  |  |  |  |  |  |  |  |  |  |
| Encoder-3 (CNN with 4.14M FLOPs) | Company’s name |  |  |  |  |  |  |  |  |  |  |  |

- References

- R4-2505108, Ad hoc minutes on AI/ML air interface study, RAN4#114bis

- R4-2505155, Collection of simulation results for AI/ML CSI compression, RAN4#114bis